STATE OF DELAWARE

QUALITY ASSURANCE PROJECT AND VERIFICATION PLAN:

NONPOINT SOURCE

BEST MANAGEMENT PRACTICE

IMPLEMENTATION DATA

Effective Date: December 1, 2022

EPA Document Control 
Number (DCN): 220076

285 Beiser Blvd, Ste 102

Dover, DE 19904

# A	PROGRAM MANAGEMENT

## A1.	Approval Sheet

Concurrence

Preparer/Author

| Name: Jennifer Walls Title: Principal Planner Organization: DNREC Division of Watershed Stewardship | Signature: Date: |
| --- | --- |

Management/Supervisor

| Name: Marcia Fox Title: Conservation Programs Administrator Organization: DNREC Division of Watershed Stewardship | Signature: Date: |
| --- | --- |

Quality Assurance Officer

| Name: Eric Dawson Title: Chief Operations Officer Organization: DNREC | Signature: Date: |
| --- | --- |

EPA Region 3

| Name: Autumn Rose Title: R3 Designated Project Manager Organization: EPA R3 CBPO | Signature: Date: |
| --- | --- |

Approval

EPA Region 3

| Name: Durga Ghosh Title: R3 Delegated Approving Official Organization: CBP / USGS | Signature: Date: |
| --- | --- |

Note: This approval action represents EPA’s determination that the document(s) under review comply with applicable requirements of the EPA Region 3 Quality Management Plan [https://www.epa.gov/sites/production/files/2020-06/documents/r3qmp-final-r3-signatures-2020.pdf] and other applicable requirements in EPA quality regulations and policies [https://www.epa.gov/quality].  This approval action does not represent EPA’s verification of the accuracy or completeness of document(s) under review and is not intended to constitute EPA direction of work by contractors, grantees or subgrantees, or other non-EPA parties.

## A2.	Revision History

This table shows changes to this controlled document over time.  The most recent version is presented in the top row of the table.  Previous versions of the document are maintained by Quality Manager.

| Document Control Number | History/ Changes | Effective Date |
| --- | --- | --- |
| 220076 | Updated Contacts, Stormwater Process, Added Appendix L | 12/1/2022 |

## A3. Table of Contents

Table of Contents

A	PROGRAM MANAGEMENT	2

A1.	Approval Sheet	2

A2.	Revision History	3

A3. Table of Contents	4

A4.  Data Providers	6

A5.  Project / Task Organization	7

A6.  Problem Definition and Background	7

A7.  Project / Task Description	8

A8.  Quality Objectives and Criteria	8

A9.  Special Training / Certification	8

A10.  Documents and Records	8

B DATA Generation and Acquisition	9

B1. Sampling Process Design (Experimental Design)	9

B2. Sampling Methods	9

B3. Sample Handling and Custody	9

B4. Analytical Methods	9

B5. Quality Control	9

B6. Instrument / Equipment Testing, Inspection, and Maintenance	9

B7. Instrument / Equipment Calibration and Frequency	9

B8. Inspection / Acceptance of Supplies and Consumables	9

B9. Non-direct Measurements	9

B10. Data Management	10

B10.1 Data Management: BMPs for Agricultural Source Sector	12

B10.1.1. List of Agricultural BMPs	13

B10.2. Data Management: BMPs for Forestry Source Sector	13

B10.2.1 – List of Forestry BMPs	14

B10.3. Data Management: BMPs for Wetland and Stream Restoration Source Sector (Restoration)	15

B10.3.1 – List of Restoration (Wetland and Stream) BMPs	15

B10.4 – Data Management: BMPs for Urban Stormwater Source Sector	16

B10.4.1 – List of Urban/Suburban Stormwater BMPs	19

B 10.5. Data Management: BMPs for Wastewater Source Sector	20

B 10.5.1. List of Wastewater BMPs	21

C. Assessment and Oversight	23

C1.  Assessments and Response Actions	23

C2. Reports to Management	23

D.  Data Validation, Verification and Usability	23

D1. Data Review, Verification, and Validation	23

D 1.1 - Selection of Priority BMPs for Verification	24

D2. Verification and Validation Methods	25

D2.1.  Agriculture Sector Verification Protocol	27

D2.1.4.3 Manure Relocation (Transport) Data Validation	46

D2.2 Forestry Sector Verification Protocol	60

D2.3 Restoration Sector Verification Protocol	70

D2.4 Stormwater	79

D2.5 Wastewater Sector Verification Protocol	88

E References	95

Appendix A:  BMP Assessment for Delaware	97

Appendix B Irrigated Land Methodology	149

Appendix C Water Control Structure Methodology	153

Appendix D Forest Harvesting Methodology	157

Appendix E Septic Connection Methodology	163

Appendix F Mapping of Jurisdiction BMP Verification Protocol Components to the Relevant QAPP Sections	166

Appendix G BMP Targeting and Prioritization (Watermelon Charts)	185

Appendix H NEIEN Methodology for Historical Data Clean Up	188

Appendix I Condensed Version of Urban & Agriculture Spreadsheet Used for Upload into NPS BMP Database	194

Appendix J Procedures for Using the Revised & Simplified Cropland Roadside Transect Survey for Obtaining Annual Tillage/Crop Residue/Cover Crop Data	196

Appendix K Standard Operating Procedures for Delaware Nutrient Management Plan Verification for Land and/or Animal Operations	211

Definitions	211

Appendix L Delaware Agriculture BMP Guidebook (2022)	223

## A4.  Data Providers

| Last | First | Title | Organization |
| --- | --- | --- | --- |
| Absher | Debbie | Agriculture BMP Data Provider | Sussex Conservation District |
| Arvay | Bonnie | Stormwater BMP Data Provider | DNREC-DWS-SSP |
| Barnett | Ashley | NonPoint Source Data Provider | DNREC-DWS-NPS |
| Braunskill | Kesha | Community and Urban Forestry | DDA-Forestry |
| Brosch | Chris | Agriculture BMP Data Provider | DDA-NMP |
| Coverdale | Michael (Ben) | Program Manager | DNREC-DWS-NPS |
| Deputy | Terry | Director | DNREC – DWS |
| Devereux | Olivia | Contractor | Devereux Consulting |
| Donnelly | Kevin | Agriculture BMP Data Provider | New Castle Conservation District |
| Eicholz | Scott | Wastewater BMP Data | DNREC-DW-RSS |
| Esposito | Sarah | BMP Data Provider | DELDOT |
| Fox | Marcia | Program Administrator | DNREC-DWS-Conservation Programs |
| Gill | Clint | Nutrient Management and Manure Relocation Data Provider | DDA-NMP |
| Hart | Eugenia | Contractor | Tetra Tech |
| Howard | Andrew | CBP Regulatory and Accountability Grant Manager | DNREC-DWS-WAMS |
| Hubert | Melissa | Restoration Data Provider | DNREC-DWS-Drainage |
| Mwangi | George | Wastewater Treatment Plant Data | DNREC-DW-CGSS |
| Rebar | John | NPDES Permit Data | DNREC-DW-CGSS |
| Rhoads | Craig | Restoration BMP Data Provider | DNREC-DFW |
| Riley | Timothy | Agriculture BMP Data Provider | Kent Conservation District |
| Rose | Autumn | Project Officer | EPA-Chesapeake Bay Program Office |
| Sevcik | Clare | BMP Data Manager | DNREC-DWS-NPS |
| Smith | Nicole | MS4 and Industrial Stormwater Data | DNREC-DW – Surface Water Discharges Section |
| Soriano | Catie | Trees for Every Delawarean Data | DNREC-CCE |
| Stewart | Elena | Restoration BMP Data Provider | DNREC-Parks |
| Sweeney | Jeffrey | University of Maryland / EPA-CBPO | NPS Data Manager |
| Walker | Holly | CBP Implementation Grant Manager | DNREC-DWS-NPS |
| Walls | Jennifer | Quality Assurance Manager | DNREC-DWS |
| Webb | Elaine | Stormwater BMP Data Provider | DNREC-DWS-SSP |
| Williams | Steve | Administrator | DNREC - DWS – WAS |
| Woodrow | Gordon | MS4 Permit Data Provider | DNREC-DW – Surface Water Discharges |
| Wool | Lisa | BMP Data Provider | NWA |
| Wozniak | Sara | CBP Regulatory and Accountability Grant Manager | DNREC-DWS-WAMS |

## A5.  Project / Task Organization

Best management practices (BMPs) to reduce nonpoint source (NPS) pollution are funded and installed by numerous federal, state, local, and private agencies within Delaware including the Department of Natural Resources and Environmental Control (DNREC), the Department of Agriculture (DDA), the Natural Resource Conservation Service (NRCS), three county Conservation Districts, and counties and towns. The BMP data that are generated are maintained and undergo quality assurance procedures by the implementing organization, which includes periodic spot checks of installed BMPs.

Data are aggregated from these multiple groups and reported to funding agencies for tracking purposes. Historically, Delaware provided the Environmental Protection Agency – Chesapeake Bay Program Office (EPA-CBPO) with BMP implementation data in a spreadsheet or tabular format. In an attempt to standardize, streamline, and document data manipulations, CBPO and the jurisdictions in the bay watershed signed an agreement specifying that data associated with BMPs will be transferred exclusively through the National Environmental Information Exchange Network (NEIEN) as of December 31, 2010. Grant guidance specifies that the exchange should contain data for projects that were implemented between July 1 and June 30 each year.

An independent Quality Assurance Manager develops and maintains the official, approved Quality Assurance Project Plan (QAPP) covering all programs receiving funds from the CBP Implementation Grant and the CBP Regulatory and Accountability Grant. The Grant Managers prepare and submit annual reports to the EPA-Chesapeake Bay Program Office (CBPO) providing a qualitative description of ongoing activities being done to achieve restoration goals. An organization chart showing reporting and quality assurance responsibilities is provided in Figure 1.

## A6.  Problem Definition and Background

The tracking, reporting, and quality assurance of NPS BMPs are requirements of the Delaware CBP Implementation Grant from the EPA-CBPO. Data are provided to EPA-CBPO via NEIEN exchange for inclusion in watershed model progress evaluations on or before December 1st of each year or as otherwise stipulated in the grant documents. Since this work involves the acquisition of environmental data generated from direct measurement activities, data collected from other sources, and data compiled from computerized information databases and systems, an approved QAPP must be in place. This technical document of quality assurance and control procedures and specifications serves as the QAPP in accordance with 40 CFR 30.54 and 31.45. This QAPP will support the quality of the data behind the CBP’s annual Restoration Assessment for Reducing Pollution, will allow the EPA-CBPO to understand the various sources of NPS BMP data and any analyses done by jurisdictions prior to submission to the EPA-CBPO, and will assist the EPA-CBPO in preparing for possible future scrutiny of all watershed model inputs under the Chesapeake Bay Total Maximum Daily Load (TMDL).

BMPs reported in a particular year include only the implementation of a new BMP. As of 2015, previously reported BMPs have been given a lifespan or credit duration based on the CreditDurations05222015.xlxs spreadsheet provided by CBP. This spreadsheet includes credit durations for each BMP type approved by the Ag and Urban Stormwater Workgroups in 2015. The lifespan is now added to the implementation date of a particular BMP to calculate the Lifespan End Date. Once the Lifespan End Date has been passed, that BMP will be tagged as “retired” and removed from the database. See Section D2-1 through D2.5 for more detail.

## A7.  Project / Task Description

Data regarding the implementation of NPS BMPs are compiled in order to assess progress toward reaching water quality goals, which includes both State of Delaware prescribed TMDL reductions for nutrients and bacteria as well as EPA TMDL reductions for nutrients and sediment. Implementation is ongoing and data are reported to the EPA-CBPO annually (on or before December 1st each year) to reflect recent implementation activities. A full description of the quality assurance activities performed on these data sets is included in Section B10 Data Management.

The following sections of this QAPP will be updated annually (on or before September 1st) to reflect any changes to field, sample handling and storage, laboratory, quality control, or data management activities.

Details regarding BMP names and crosswalk with Scenario Builder names are listed in Section 10.1.  Each BMP is listed by name with BMP short name, a description, the unit in which it is reported and the agency providing the data.

## A8.  Quality Objectives and Criteria

Details regarding the quality of the NPS BMP data reported by the DNREC-DWS-NPS to the EPA-CBPO for use in watershed modeling to estimate restoration progress are contained in the following sections. All efforts have been made to produce data that are comparable to data collected previously and currently by other Chesapeake Bay Program grant recipients and partners. Details on the quality of data provided by DNREC are included in the following sections. All BMPs completed must be certified as complete and meeting appropriate standards as deemed by the authorized cost share program.

## A9.  Special Training / Certification

Any special training or certification required to implement or inspect NPS BMPs is determined and overseen by the implementing organization. Additionally, individuals involved with NPS BMP data management and data quality assurance and control procedures are not required to have any special training or certification, however in order to perform these functions effectively, training in spreadsheets, databases, and geographic information systems (GIS), as well as computer programming and code writing may be necessary. Delaware’s previous Quality Assurance Manager received training from the EPA on Quality Assurance Strategies for the use of Existing Data in February 2013. It is anticipated that the new Quality Assurance Managers will undergo the same training when available. Due to privacy concerns, BMP implementing organizations determine who may have clearance to complete data sets and in some situations restrict the transfer of personal and locational information.

See Sections D2.1 through D2.5 for specific training and certification requirements for BMP Verification and Validation.

## A10.  Documents and Records

Implementing organizations will maintain NPS BMP data sets. These data sets are needed for the NEIEN schemas and are transmitted via established NEIEN protocols for inclusion in the annual progress run input deck. Data included in EPA-CBPO annual reports will be retained electronically in Extensible Markup Language (XML) format by the DNREC-DWS-NPS in perpetuity. The DNREC-DWS-NPS will send the QAPP electronically to all individuals on the distribution list (A3) each year for annual review and comment. Any edits to reflect changes in status or procedure will be incorporated into the final document submitted to the EPA-CBPO on or before December 1st each year. The final, EPA-CBPO approved QAPP will be electronically distributed to the same individuals and will be retained in both electronic and paper format in perpetuity by the DNREC-DWS-NPS. Any inspection forms and/or methodology for documenting information are discussed in sections D2.1 through D2.5 for each specific source sector (agriculture, forestry, stream and wetland restoration, stormwater, and wastewater).

# B DATA Generation and Acquisition

Sections B1 through B8 of this QAPP are not directly applicable to NPS BMP data tracking and reporting. Situations where implementing organizations generate data through sampling to answer research questions do occur. For example, soil samples are taken during the development of a nutrient management plan to determine appropriate fertilizer and manure application rates. Likewise, manure is sampled to determine nutrient content. In addition, samples may be taken to determine the performance level of a BMP, such as taking effluent samples from alternative and innovative onsite wastewater treatment and disposal systems. Details regarding any sampling protocols related to NPS BMPs will be incorporated in future versions of this QAPP. Details regarding surface water quality monitoring protocols can be found in both the DNREC (DNREC, 2007) and Nanticoke Creekwatcher QAPP documents (NWA, 2015). Additionally, the Delaware Natural Resources and Conservation Service (NRCS) completed a QAPP and Corrective Action Plan in FY2012 (NRCS, 2012).

## B1. Sampling Process Design (Experimental Design)

## B2. Sampling Methods

## B3. Sample Handling and Custody

## B4. Analytical Methods

## B5. Quality Control

## B6. Instrument / Equipment Testing, Inspection, and Maintenance

## B7. Instrument / Equipment Calibration and Frequency

## B8. Inspection / Acceptance of Supplies and Consumables

## B9. Non-direct Measurements

DNREC’s Division of Watershed Stewardship, Nonpoint Source Program (DNREC-DWS-NPS) obtains NPS BMP tracking data from both internal and external sources (See Figure 1), which are then reported to the EPA-CBPO for inclusion in model scenario runs via NEIEN.

- Developed Sector -Stormwater BMP implementation and Stormwater BMP maintenance verification data is collected by the DNREC Sediment and Stormwater Program (DNREC-DWS-SSP) from nine Sediment and Stormwater Delegated Agencies on a biannual basis.

- BMPs associated with wastewater treatment are implemented, tracked, and reported by DNREC’s Division of Water - Groundwater Discharges Section (DNREC-DW-GWDS).

- BMP data associated with agriculture are implemented, tracked, and/or maintained by multiple agencies including the NRCS, DNREC’s Non-Point Source 319 Program, Delaware Department of Agriculture, and the three county Conservation Districts.

In the spring of 2007, DNREC’s Divisions of Water Resources and Soil and Water Conservation (now known as the Divisions of Water and Watershed Stewardship) contracted with URS Corporation to conduct an assessment of BMP data collection activities across the state. The resulting report, which summarizes the points of contact, type of BMP data maintained by each agency, data storage structures, data sharing limitations, and supporting software, can be found in Appendix A.

The implementing agencies described in Appendix A are responsible for ensuring delivery of quality data. The data providers collect, manage and report data to the DNREC BMP Data Manager The independent Quality Assurance Manager reviews all data to ensure BMP reported levels reasonably reflect on-the-ground conditions.  DNREC-DWS-NPS addresses the quality assurance process related to data as received from data providers.

## B10. Data Management

BMP data are requested on an annual or more frequent basis from numerous agencies that implement, track, and/or maintain this type of data in the stormwater, wastewater, and agriculture-related sectors. Figure 1 depicts BMP data reporting and quality assurance responsibilities.

Previously, the majority of data submitted to DNREC-DWS-NPS were done electronically in Excel spreadsheets; however, paper copies were occasionally submitted from some reporting agencies as well. This varied data had to be compiled into a single document with a consistent format and as such, was inconvenient and time consuming for all involved. In an attempt to standardize, streamline, and document data manipulations, CBPO and the jurisdictions in the bay watershed signed an agreement specifying that data associated with BMPs will be transferred exclusively through the National Environmental Information Exchange Network (NEIEN) as of December 31, 2010. Grant guidance specifies that the exchange should contain data for projects that were implemented between July 1 and June 30 each year.

The NEIEN is a partnership between the Bay jurisdictions and the CBPO for the secure, real-time exchange of environmental information. The Network uses extensible markup language (XML), web services, and common data standards to transmit data from the jurisdictions to the CBPO. Existing data management systems are able to remain in place and, through the Network; data are delivered based on pre-described methods, or a schema. The CBP NPS BMP schema was developed by PA, VA, and MD with a $390,000 grant, which included the building of a node at the CBPO. Delaware began mapping data from state sources into the schema. The schema in use contains fields such as jurisdiction, data source, contact information, name of practice, practice components, location, unit of measure, quantity, status, and funding source.

In Delaware data from each implementing organization was supplied to DNREC’s OIT for conversion into an XML document until 2019. Once all data sources were received, data was transmitted through DNREC’s network node. Since the 2010 data submission was the first through NEIEN, Delaware required the assistance of Tetra Tech to complete several of these XML documents. DNREC’s OIT prepared the XML data for stormwater and onsite wastewater practices. Data from the DDA Forest Service and Nutrient Management Program (manure relocation and nutrient management planning) were provided to NPS in GIS, database, or Excel format for this work. Additionally, in 2010, an agreement (Basinwide 1619 Agreement) was reached to have federal agencies, such as the USDA’s NRCS and FSA, report practices directly to the USGS for CBP modeling rather than have jurisdictions report on their behalf.

Beginning in with the 2019 data submission, Delaware partners began submitting data to the DNREC through a BMP Tracking Tool. DNREC staff were able to download the data as xml files, conduct QA/QC review, and submit to NEIEN. Delaware’s BMP database experienced fatal errors in 2018 that rendered it inoperative. In 2019 Delaware contracted with Devereux Consulting to develop a new BMP Tracking and Reporting Tool that is capable of housing and formatting Delaware’s BMP Implementation and Verification information for submission to NEIEN.

Errors with the BMP Reporting tool were identified with the 2020 progress data submission which resulted in the potential expiration of best management practices. As of early 2021, Delaware’s BMP Tracking and Reporting Tool only contained BMP data from 2019 and 2020. Delaware’s partners have completed verification reports since the inception of its BMP Verification Program; however, Delaware has encountered hurdles incorporating BMP verification information into the BMP database framework, causing loss of model credit for BMPs in annual progress. Because Delaware did not have 1985-2018 records in the new tool, the state was not able to enter verification information for BMPs that were falling out of lifespan in years prior to 2019.

In addition to the database failure, Delaware was asked to submit historical BMP records from 1985-2015 for “historical BMP clean-up” in the new Phase 6 Chesapeake Bay Model in 2015. The BMP clean-up was an opportunity to identify BMPs that were previously underreported, or in some way deficient, to receive proper nutrient reduction credits for those practices. Through the cleanup process, it was identified that the amount of BMP implementation in Delaware fluctuated annually based on various factors, including but not limited to, enactment of regulations, landowner participation interest, available funding, etc. As per the Historic BMP Cleanup Rules provided to Delaware from EPA/Chesapeake Bay Program in 2014, Delaware’s BMP implementation levels fluctuated too significantly and/or lacked sufficient detailed records. As a result of these findings, data were adjusted to represent a linear increase in BMP implementation over time per EPA historical cleanup guidance (Historic BMP Cleanup Rules of the Road [WTWG 2014]). Delaware was able to achieve this using existing data to extrapolate back to 1985 or use interpolated BMP data between gaps in years. Delaware completed this exercise for over 30 BMPs and submitted the data for incorporation into the Phase 6 version of the Chesapeake Bay Model, with approval from EPA.

Because of the BMP historic cleanup process, Delaware had an implementation history that did not always reflect true implementation timing. This impacted the state’s ability to accurately match reported verification with existing BMP practices. For example, practices that may have been implemented post-1985 were extrapolated back to 1985 and, therefore, did not have the correct implementation date in the database, making it difficult to match the BMP in the database with the associated BMP on the ground. Without the ability to associate verification information with specific practices, the state was losing BMP credit in the Chesapeake Bay model in subsequent years. Delaware’s partners continue to collect and submit inspection and verification records; however, the database and data extrapolation limitations that Delaware experienced prevented the state from submitting verification records.

In 2020-2021, DNREC contracted with Tetra Tech’s Eugenia Hart to make the necessary adjustments to the database and update historic data sets. Tetra Tech has assisted Delaware with gathering and reporting BMP records since 2010 and had access to the original records that were reported prior to the historic BMP cleanup process. In 2020 through early 2021, Tetra Tech assisted with identifying the BMPs that no longer received model credit due to falling out of lifespan. The BMP’s original records were identified and accurately incorporated into the new BMP Tracking and Reporting Tool (with correct implementation dates) with any available inspection/verification records from partner organizations.

In addition to the 1985-2015 historical data, Tetra Tech also formatted the 2016 through 2018 BMP data and all available verification data for inclusion in Delaware’s new BMP Tracking and Reporting Tool in 2021.The revised data sets were resubmitted with 2021 data submission.

### B10.1 Data Management: BMPs for Agricultural Source Sector

NRCS/FSA Data - Data are provided by Devereux Environmental Consulting (third party contractor of USGS) in excel format at the state and county level. The NRCS cover crop data as well as detailed cover crop information submitted by the Conservation Districts are subtracted from FSA cover crop data. The NRCS data, the Conservation District Data, and the remaining FSA acreage are reported to avoid double counting.

DDA Manure Transport – Manure Transport is provided by DDA as tons of poultry manure. The data include the sending watershed, receiving watershed, receiving town, receiving state, claim tons, claim date, application number, and whether the relocation was “farm to farm in DE”, “farm to farm outside DE”, “farm to alternative use”, and “farm to alternative use (off peninsula)”. Delaware does not transport any manure besides poultry. The poultry in Delaware are all broilers except for one-layer facility; therefore, the Animal Group is labeled as “Poultry”. Majority of the Nanticoke watershed is in Sussex County (86%) and a small portion is in Kent County (14%); therefore, the assumption was made that all manure (within the Nanticoke watershed) comes from Sussex County. The Marshyhope Watershed is within two counties, so the claim tons are split evenly between the 2 counties. Only manure exported from the Chesapeake Bay watershed is included and all other watersheds (Indian River, Indian River Bay and Murderkill watershed entries) are deleted. COUNTY_TO in the Excel sheet is left blank if the manure leaves the Chesapeake Bay watershed or is identified as “farm to alternative use” and “farm to alternative use off peninsula”.

Irrigation Management – The acreage of irrigated land was calculated in July 2010 based on Google Earth Imagery by NRCS. The 2013 Irrigated Land Project is an update to this dataset based on 2012 imagery in ArcGIS. A complete methodology is listed in Appendix B. Data are reported as acreage by HUC using 2013 as the implementation year. This GIS analysis will be conducted periodically or until the cropland irrigation management BMP is approved by the Partnership.

Conservation District Cover Crop Data – Detailed cover crop information is received from each County Conservation District – New Castle, Kent, and Sussex. Data are received in excel format. Since 2019 the data has been submitted using the BMP Database Template (Excel). Cover crop data are reviewed and determined to be commodity (harvested) or traditional (destroyed). Only those crops identified in the Chesapeake Bay Watersheds are included.

Sussex County – In 2012, some cover crops were provided as multiple crops (e.g., barley/wheat) which means part of the field was planted in one and one planted in the other. Sometimes crops are planted as a seed mix. Records with seed mixes are split 50/50 for acreage in each crop. In 2013, the Cover Crop Expert Panel Report was approved and many of these seed mixes are acceptable in Phase 5.3.2 for 2013 progress. Planting dates are provided and were used to determine whether the crops are early/late/standard.

Kent County – Data are compiled using the criteria set above for Sussex County. Additionally, a few records had two planting dates listed. For these entries, the latter date was assumed as implementation date.

New Castle County – The same methodology was followed as Sussex and Kent counties.

DNREC Restoration Database – DNREC –DWS-WAS maintains a restoration database that captures restoration practices like grass buffers and water control structures. These practices are compiled from various projects throughout DNREC. The restoration database links DNREC BMPs to NRCS practice codes. The database is not set to match the BMPs reporting to EPA-CBPO. Therefore, DNREC-DWS-NPS must make judgment calls when assigning acres (or other units) to specific EPA-CBPO BMPs.

Historical Water Control Structures – DNREC-DWS-NPS and Sussex Conservation District worked collaboratively in the summer of 2013 to update water control structure data by ground truthing and verifying structures with GPS. A complete methodology is listed in Appendix B. Data are reported as acreage by HUC using 2013 as the implementation year. Water control structures implemented by DNREC are also captured in the DNREC Restoration Database.

Double counting of these agricultural practices is avoided by submitting data by the primary funding source or the primary implementing agency. For example, BMP implementation data that are cost-shared with NRCS are submitted by NRCS. Non-cost shared data are submitted by the state or conservation districts.

### B10.1.1. List of Agricultural BMPs

Refer to Appendix L Chesapeake Bay Agriculture BMP Guidebook for definitions, and reporting characteristics.

### B10.2. Data Management: BMPs for Forestry Source Sector

DDA Forestry Harvesting - The DDA Delaware Forest Service (DFS) provides acreage of harvested forestland. DDA-DFS provides GIS coverage of permitted timber harvest practices in the Chesapeake Bay watershed. HUCs are identified using GIS by intersecting the Timber Harvest coverage with the USGS HUC12 coverage to determine the HUC 12 for each harvest area.

Historical Harvested Forest Data – DNREC-DWS-NPS and DDA-DFS worked collaboratively in the summer of 2013 to update forest harvest area data by digitizing harvested forest areas with ArcGIS. The digitization of these harvest areas are linked to an Access database containing all permit information, creating a spatial reference. Capturing these data will allow Delaware to report these historical harvested forest data for inclusion in the CBWSM. A complete methodology is listed in Appendix D.

DDA Forestry Tree Planting – The Department of Agriculture’s Delaware Forest Service (DFS) provides acreage of afforestation tree plantings. DDA provides GIS coverage of tree planting in the Chesapeake Bay watershed. HUC12s are identified by using GIS for each planting area. The GIS coverage includes an attribute table that includes the “type” of project (either afforestation or reforestation). Only “afforestation” records are included in the progress run. Most of Forestry’s reforestation projects are cost-shared through NRCS funds; and therefore, are already counted by NRCS data. When the project is paid by DDA Forestry or the private landowner that information will not be reported by NRCS and only DFS acreage will be used in the progress run.

DDA Urban Tree Planting – The DDA-DFS provides number of trees planted by the Urban and Community Forestry Program. The data are entered into Excel with unique identifier, implementation date, number of trees, and HUC using the BMP Database Template.

DNREC Restoration Database – DNREC –DWS-WAS maintains a restoration database that captures restoration practices like wetland restoration, tree plantings, forest buffers, and grass buffers. These practices are compiled from various projects throughout DNREC. The restoration database links DNREC BMPs to NRCS practice codes. The database is not set to match the BMPs reporting to EPA-CBPO. Therefore, DNREC-DWS-NPS must make judgment calls when assigning acres (or other units) to specific EPA-CBPO BMPs.

Double counting is unlikely to occur for forestry harvesting practices because they are being provided by one agency (DDA).  The same is true for urban tree planting data.  This data is only submitted by DDA-DFS.  Forest buffers are submitted by multiple agencies and funding sources are distinctively tracked by the QA Manager. As a result, double counting is avoided.

### B10.2.1 – List of Forestry BMPs

| BMP | BMP Short Name | BMP Description | Unit | Data Source |
| --- | --- | --- | --- | --- |
| Streamside Forest Buffers | ForestBuffersTrp | Converts streamside areas to forest. In the model, converts degraded riparian pasture to hay without nutrients. Should be used with Stream Access Control with Fencing to convert from hay without nutrients to forest. | acres in buffers | NRCS, DNREC, DFS |
| Vegetative Environmental Buffers | Delaware definition only | Tree planting includes any tree planting, except those used to establish riparian forest buffers, targeting lands that are highly erodible or identified as critical resource areas. | acres | DDA, DNREC |
| Forest Buffers | ForestBuffers | Agricultural riparian forest buffers are linear wooded areas along rivers, stream and shorelines. Forest buffers help filter nutrients, sediments and other pollutants from runoff as well as remove nutrients from groundwater. The recommended buffer width for riparian forest buffers (agriculture) is 100 feet, with a 35 feet minimum width required. | acres in buffers | DDA, DNREC, USFWS |
| Tree Planting | TreePlant | Tree planting includes any tree planting, except those used to establish riparian forest buffers, targeting lands that are highly erodible or identified as critical resource areas. | acres | NRCS, USFWS, DFS, DelDOT, DNREC |
| Forest Harvesting Practices | ForHarvestBMP | Forest harvesting practices are a suite of BMPs that minimize the environmental impacts of road building, log removal, site preparation and forest management. These practices help reduce suspended sediments and associated nutrients that can result from forest operations. | acres | DDA |
| Urban Tree Planting; Urban Tree Canopy | UrbanTreePlant | Urban tree planting is planting trees on urban pervious areas at a rate that would produce a forest-like condition over time. The intent of the planting is to eventually convert the urban area to forest. If the trees are planted as part of the urban landscape, with no intention to covert the area to forest, then this would not count as urban tree planting | acres | DDA |

### B10.3. Data Management: BMPs for Wetland and Stream Restoration Source Sector (Restoration)

NRCS/FSA Data - Data are provided by Devereux Environmental Consulting (third party contractor of USGS) in excel format at the state and county level for wetland restoration practices.

DNREC Restoration Database – DNREC –DWS-WAS maintains a restoration database that captures restoration practices like wetland restoration and creation, and stream restoration.  These practices are compiled from various projects throughout DNREC. The restoration database links DNREC BMPs to NRCS practice codes. The database is not set to match the BMPs reporting to EPA-CBPO. Therefore, DNREC-DWS-NPS must make judgment calls when assigning acres (or other units) to specific EPA-CBPO BMPs.

Double counting of these agricultural practices is avoided by submitting data by the primary funding source or the primary implementing agency. For example, BMP implementation data that are cost-shared with NRCS are submitted by NRCS. Non-cost shared data are submitted by the state or conservation districts.

### B10.3.1 – List of Restoration (Wetland and Stream) BMPs

| BMP | BMP Short Name | BMP Description | Unit | Data Source |
| --- | --- | --- | --- | --- |
| Streamside/Tax Ditch Restoration | Delaware definition only | A suite of innovative alternative practices designed to enhance the removal of nutrients once they leave the field. These include increasing vegetative buffers that protect ditches from sediment and nutrient runoff. This may include reengineering of drainage channels to reestablish floodplains or redirect storm flows to wetland areas. | linear feet | DNREC, DFS, USFWS, Conservation Districts |
| Non Urban Stream Restoration | NonUrbStrmRest | Stream restoration in urban areas is used to restore the urban stream ecosystem by restoring the natural hydrology and landscape of a stream, help improve habitat and water quality conditions in degraded streams. The reduction is 0.2 lb nitrogen per foot, 0.068 phosphorus per foot, and 54.25 lbs sediment per foot . | feet | DDA, DNREC, NRCS, USFWS |
| Streamside Wetland Restoration | WetlandRestoreTrp | Converts degraded riparian pasture to forest. | acres | NRCS, DNREC, DFS |
| Wetland Restoration | WetlandRestore | Agricultural wetland restoration activities re-establish the natural hydraulic condition in a field that existed prior to the installation of subsurface or surface drainage. Projects may include restoration, creation and enhancement acreage. Restored wetlands may be any wetland classification including forested, scrub-shrub or emergent marsh. | acres | NRCS, DDA, DNREC |

### B10.4 – Data Management: BMPs for Urban Stormwater Source Sector

DelDOT Street Sweeping – DelDOT compiles street sweeping data from roadways in New Castle and Kent Counties. Pollutant loads are calculated using the mass loading approach outlined in the Chesapeake Urban Stormwater Workgroup’s recommendations memo (http://chesapeakestormwater.net/wp-content/uploads/downloads/2012/06/CBP-Expert-Panel-Memo-on-Street-Sweeping.pdf).

DNREC Stormwater Practices - Data were previously pulled from the MudTracker Database. DNREC-DWS-NPS worked with OIT to extract data inputted into MudTracker by the DNREC-DWS-SSW.   BMP Maintenance Reviews for DNREC reviewed projects are being tracked in SWPPPTrack. DNREC-DWS-SSP has begun collecting data including construction inspection numbers, new BMP data and BMP maintenance inspection data from all Delegated Agencies, including DelDOT on a bi-annual basis and will submit on behalf of all SSP Delegated Agencies.

DNREC’s Sediment and Stormwater Program (SSP) solicits BMP implementation and inspection data from delegated agencies and then reports the data to the NPS Program using the BMP Database Template.

Delegated Agencies

The following entities are included in the delegated agencies list and are solicited for data by the SSP.

- DNREC Sediment and Stormwater Program

- DelDOT

- Sussex Conservation District

- Kent Conservation District

- New Castle Conservation District

- New Castle County

- City of Wilmington

- City of Newark

- Town of Middletown

Reporting Period

Delegated Agencies of the DNREC Sediment and Stormwater Program are required to submit program information and metrics of Sediment and Stormwater program implementation two times per year as follows:

- Biannual Report due April 15 for the preceding time period of October 1 – March 31

- Biannual Report due October 15 for the preceding time period of April 1 – September 30

After these six-month periods, the SSP will report BMP data in the BMP Data Template to the NPS program within one month of the delegated agency due dates.

Metrics

The metrics submitted to the SSP are as follows:

- Compliance Monitoring Strategy (CMS) Metrics.  Report the number of construction site regulatory inspections conducted during the reporting period:

  - Project Name – this is a required field

  - Project Number – include an agency-specific tracking number if it exists; this is not a required field.

  - NOI number

  - Number of regulatory inspections conducted by the Delegated Agency, not including CCR inspections

- Post Construction BMP Construction and Maintenance Verification

  - Report post construction stormwater BMPs verified during the reporting period (BMP Implementation)

    - Project Name – this is a required field.

    - Project Number – include an agency-specific tracking number if it exists; this is not a required field.

      - *Note that a single project site (name and number) having more than one BMP will have more than one report as each BMP is reported separately.

      - **BMPs reported in the Chesapeake Bay watershed should all have a unique identifier associated with them for reporting purposes.

    - Land Use Type based on the following categories:

      - Commercial

      - Single family residential

      - Multi-family residential

      - Highway

      - Other

    - BMP Type based upon the 17 major categories of BMPs in BMP Standards and Specifications.

    - Date of BMP construction based upon the acceptance date of the PCVD.

    - Latitude and Longitude of BMP outfall expressed in decimal degrees to 6 decimal places.  The BMP outfall location for infiltrating BMPs having no surface discharge should be a location within the footprint of the BMP.

    - Total acres treated by BMP.

    - Impervious acres treated by BMP.

  - BMP maintenance reviews (BMP inspections) conducted during the reporting period

    - Project Name – this is a required field.

    - Project Number – include an agency-specific tracking number if it exists; this is not a required field.

      - *Note that a single project site (name and number) having more than one BMP will have more than one report as each BMP is reported separately.

      - **BMPs reported in the Chesapeake Bay watershed should all have a unique identifier associated with them for reporting purposes.

    - BMP Type based upon the 17 major categories of BMPs in BMP Standards and Specifications.

    - Latitude and Longitude of BMP outfall expressed in decimal degrees to 6 decimal places.  The BMP outfall location for infiltrating BMPs having no surface discharge should be a location within the footprint of the BMP.

    - Date of the maintenance review.

    - BMP status: functional or non-functional resulting from the maintenance review.

Data Submission to the BMP Tracking and Reporting Tool

Upon receipt of the data from the SSP, NPS staff ensures the data is complete, able to be submitted to the BMP Tracking and Reporting Tool, and work with the SSP to make any corrections that are needed.

Delaware submits numerous urban BMPs under two categories: New Stormwater Retrofits and New Runoff Reduction. Each BMP submitted under these two categories needs three measurements: acres treated, impervious area, and volume of runoff. The SSP reports the area treated and the impervious area for each implemented BMP (see Metrics). The NPS program calculates the volume using the following formula:

Volume=Area Treated*0.083 feet

Double counting is unlikely to occur for these stormwater practices because they are being provided by one agency (DNREC) and there are no cost-share practices.

### B10.4.1 – List of Urban/Suburban Stormwater BMPs

| Sediment and Stormwater BMP (Reference to Delaware Sediment and Stormwater Regulations | NEIEN BMP | Runoff Reduction or Stormwater Treatment? |
| --- | --- | --- |
| 1-A Infiltration Trench | Infiltration Trench | Runoff Reduction |
| 1-B Infiltration Basin | Infiltration Basin | Runoff Reduction |
| 1-C Underground Infiltration | Underground Infiltration System | Runoff Reduction |
| 2-A Traditional Bioretention | Bioretention | Runoff Reduction |
| 2-B In-Situ Bioretention including Rain Gardens | Rain Garden | Stormwater Treatment |
| 2-C Streetscape Bioretention | Bioretention | Runoff Reduction |
| 2-D Engineered Tree Boxes | Bioretention | Runoff Reduction |
| 2-E Stormwater Planters | Bioretention | Runoff Reduction |
| 2-F Advanced Bioretention Systems | Bioretention | Runoff Reduction |
| 3-A Porous Asphalt (PA) | Permeable Pavement w/o Sand, Veg. - A/B soils, underdrain | Runoff Reduction |
| 3-B Pervious Concrete (PC) | Permeable Pavement w/o Sand, Veg. - A/B soils, underdrain | Runoff Reduction |
| 3-C Permeable interlocking concrete Pavers (PP) or Concrete grid Pavers (CP) | Permeable Pavement w/o Sand, Veg. - A/B soils, underdrain | Runoff Reduction |
| 3-D Plastic Grid Pavers (GP) | Permeable Pavement w/o Sand, Veg. - A/B soils, underdrain | Runoff Reduction |
| 4-A Extensive Vegetated Roofs | Green Roofs | Runoff Reduction |
| 4-B Intensive Vegetated Roofs | Green Roofs | Runoff Reduction |
| 5-A Seasonal Rainwater Harvesting Systems | Cisterns & Rain Barrels | Runoff Reduction |
| 5-B Continuous Rainwater Harvesting Systems | Cisterns & Rain Barrels | Runoff Reduction |
| 6-A Step Pool RSCS (Regenerative stormwater conveyance system) | Regenerative Stormwater Conveyance | Stormwater Treatment |
| 6-B Seepage Wetland RSCS | Regenerative Stormwater Conveyance | Stormwater Treatment |
| 6-C Streambank Stabilization | Streambank Stabilization | Separate BMP |
| 7.0 Rooftop Disconnection | Disconnection of Rooftop Runoff | Runoff Reduction |
| 8-A Bioswale | Bioswale | Runoff Reduction |
| 8-B Grassed Channel | Vegetated Open Channels | Runoff Reduction |
| 9-A Sheet Flow to Grassed Filter Strip | Filter Strip | Runoff Reduction |
| 9-B Sheet Flow to Afforested Filter Strip | Filter Strip | Runoff Reduction |
| 9-C Sheet Flow to Forested Filter Strip | Filter Strip | Runoff Reduction |
| 9-D Sheet Flow to Grassed Open Space | Filter Strip | Runoff Reduction |
| 9-E Sheet Flow to Afforested Open Space | Filter Strip | Runoff Reduction |
| 9-F Sheet Flow to Forested Open Space | Filter Strip | Runoff Reduction |
| 10-A Dry Detention Pond | Dry Detention Ponds | Stormwater Treatment |
| 10-B Dry Extended Detention (ED) Basin | Dry Extended Detention Ponds | Stormwater Treatment |
| 10-C Underground Detention Facilities | Dry Extended Detention Ponds | Stormwater Treatment |
| 11-A Non-Structural Sand Filter | Surface Sand Filter | Stormwater Treatment |
| 11-B Surface Sand Filter | Surface Sand Filter | Stormwater Treatment |
| 11-C Three-Chamber Underground Sand Filter | Proprietary Stormwater Treatment Device | Stormwater Treatment |
| 11-D Perimeter Sand Filter (including “Delaware” Modular Sand Filter) | Proprietary Stormwater Treatment Device | Stormwater Treatment |
| 12-A Traditional Constructed Wetlands | Constructed Wetland | Stormwater Treatment |
| 12-B Wetland Swales | Vegetated Open Channels | Runoff Reduction |
| 12-C Ephemeral Constructed Wetlands | Constructed Wetland | Stormwater Treatment |
| 12-D Submerged Gravel Wetlands | Constructed Wetland (until better option available) | Stormwater Treatment |
| 12-E Floating Wetlands (to be added at a later date) | Floating Treatment Wetland | Stormwater Treatment |
| 13-A Wet Quantity Management Pond | Wet Pond | Stormwater Treatment |
| 13-B Wet Extended Detention (ED) Pond | Wet Extended Detention | Stormwater Treatment |
| 14.0 Soil Amendments | NONE | N/A |
| 15.0 Proprietary Practices | Proprietary Stormwater Treatment Device | Stormwater Treatment |
| 16-A Nutrient Management | Urban Nutrient Management Plan | Separate BMP |
| 16-B Street Sweeping | Street Sweeping | Separate BMP |
| 17-A. Afforestation | Urban Forest Planting | Separate BMP |
| 17-B. Urban Tree Planting | Tree Planting | Separate BMP |

### B 10.5. Data Management: BMPs for Wastewater Source Sector

DNREC Onsite Wastewater Practices – Data are pulled from the Delaware Environmental Network (DEN). The Chesapeake Bay Program Data Manager works with OIT/DTI, and Groundwater Discharges Program staff to extract data inputted into DEN by the GWDS. Information is compiled for septic connections, septic pumping, and septic inspections and OIT creates XML for CBPO reporting.

DNREC Septic System and Abandonment – DNREC-DWS staff and GWDS worked collaboratively in November 2013 with to update septic system connection data with ArcGIS using the methodology described in Appendix E. The digitization of these septic connections is linked to the Delaware Environmental Navigator database containing all permit information, creating a spatial reference.

Double counting is unlikely to occur for these wastewater practices because they are being provided by one agency (DNREC) and there are no cost-share practices.

### B 10.5.1. List of Wastewater BMPs

| BMP | BMP Short Name | BMP Description | Unit | Data Source |
| --- | --- | --- | --- | --- |
| Septic Connection | SepticConnect | This is when septic systems get converted to public sewer. This reduces the number of systems because the waste is sent into the sewer and treated at a wastewater treatment plant. | systems | DNREC |
| Septic Denitrification Septic Tank Advanced Treatment | SepticDenitrify | Septic denitrification represents the replacement of traditional septic systems with more advanced systems that have additional nitrogen removal capabilities. Traditional septic systems usually consist of a large tank designed to hold the wastewater allowing grits and solids time for settling and decomposition. Wastewater then flows to the second component, the drainfield. An enhanced septic system like that shown can provide further treatment of nitrogen through processes that encourage denitrification of the wastewater. | systems | DNREC |
| Septic Effluent – Advanced |  |  | systems | DNREC |
| Septic Effluent Elevated Mound |  |  | systems | DNREC |
| Septic Effluent Shallow Pressure |  |  | systems | DNREC |
| Septic Tank System Repair |  |  | systems | DNREC |
| Septic Tank Pumpout | SepticPump | Septic systems achieve nutrient reductions through several types of management practices, including frequent maintenance and pumping. On average, septic tanks need to be pumped once every three to five years to maintain effectiveness. The pumping of septic tanks is one of several measures that can be implemented to protect soil absorption systems from failure. When septic tanks are pumped and sewage removed, the septic system’s capacity to remove settable and floatable solids from wastewater is increased. | systems | DNREC |

# C. Assessment and Oversight

## C1.  Assessments and Response Actions

A variety of assessments are performed on the NPS BMP data that are reported to the EPA-CBPO for inclusion in model scenario runs. Depending on the type of BMP, field assessments may be performed and implementing organizations are responsible for ensuring that reported BMPs have indeed been installed. Procedures are in place for verifying implementation when cost share or permits are involved. Funding from the Regulatory and Accountability grant helps to ensure that adequate staff and resources are available to inspect the upkeep and maintenance of long-term BMPs, such as stormwater ponds, on a regular basis rather than only if a problem is reported. Inspection frequencies can be found in Appendix A. If a BMP is found to be unsatisfactorily installed or maintained, cost share funds may be recouped if the BMP is not brought into compliance. In addition to field inspections, BMP data are regularly assessed by the BMP Data Manager to determine status and trends. This analysis will review any anomalies, errors, or questionable levels of implementation. Verification and validation procedures for each sector are provided in sections D2.1 through D2.5 for agriculture, forestry, stream and wetland restoration, stormwater, and wastewater practices.

## C2. Reports to Management

Status and trends assessments of BMP implementation levels by the BMP Data Manager are done annually as data are submitted, prepared, and reported to the EPA-CBPO. If anomalies, errors, or questionable levels of implementation are suspected, the BMP Data Manager will work directly with implementing organizations to verify and validate reported data.

# D.  Data Validation, Verification and Usability

## D1. Data Review, Verification, and Validation

Executive Order 13508, the Chesapeake Executive Council, the Scientific and Technical Advisory Council, the National Academy of Sciences, and others have called for increased transparency and scientific rigor in the verification of the best management practices (BMPs) that are implemented as part of the states’ Watershed Implementation Plans and the Chesapeake Bay Total Maximum Daily Load (TMDL). To respond to this request, Strengthening Verification of Best Management Practices Implemented in the Chesapeake Bay Watershed: A Basinwide Framework, Report and Documentation from the Chesapeake Bay Program Water Quality Goal Implementation Team’s BMP Verification Committee (Verification Framework) (Chesapeake Bay Program 2014), was developed. The Verification Framework is intended to serve as a guide for the states to document the methodology for verification of BMP installation, function, and continued effectiveness of practices over time. This Verification Framework provides the requirements for reporting and documentation of practice verification for the states to follow. Specific guidance is provided for each of the source sectors (agriculture, forestry, restoration [streams and wetlands], urban stormwater, and wastewater).

Verification is formally defined by the Chesapeake Bay Program partners as “the process through which agency partners ensure practices, treatments, and technologies resulting in reductions of nitrogen, phosphorus, and/or sediment pollutant loads are implemented and operating correctly.” The Chesapeake Bay Program partnership’s Principals’ Staff Committee formally adopted five verification principles in December 2012; these are described in Table D1-1.

Table D1-1. Verification Principles adopted by the Principles’ Staff Committee.

| Principle | Description |
| --- | --- |
| Practice Reporting | Affirms that verification is required for practices, treatments and technologies reported for nitrogen, phosphorus and/or sediment pollutant load reduction credit through the Bay Program. This principle also outlines general expectations for BMP verification protocols. |
| Scientific Rigor | Asserts that BMP verification should assure effective implementation through scientifically rigorous and defensible, professionally established and accepted sampling, inspection and certification protocols. Recognizes that BMP verification shall allow for varying methods of data collection that balance scientific rigor with cost effectiveness and the significance of or priority placed upon the practice in achieving pollution reduction. |
| Public Confidence | Calls for BMP verification protocols to incorporate transparency in both the processes of verification and tracking and reporting of the underlying data. Recognizes that levels of transparency will vary depending upon source sector, acknowledging existing legal limitations and the need to respect individual confidentiality to ensure access to non-cost shared practice data. |
| Adaptive Management | Recognizes that advancements in practice reporting and scientific rigor, as described above, are integral to assuring desired long-term outcomes while reducing the uncertainty found in natural systems and human behaviors. Calls for BMP verification protocols to recognize existing funding and allow for reasonable levels of flexibility in the allocation or targeting of funds. |
| Sector Equity | Calls for each jurisdiction’s BMP verification program to strive to achieve equity in the measurement of functionality and effectiveness of implemented BMPs among and across the source sectors. |

### D 1.1 - Selection of Priority BMPs for Verification

While it is the goal to verify implementation of all BMPs implemented within the Chesapeake Bay Watershed, resource constraints dictate that priorities be set to focus on those BMPs of greatest importance to achieving Delaware’s pollutant load reduction goals. BMPs considered to be of the highest priority for developing verification protocols were those that are projected to contribute at least 5 percent of the load reduction to the state by 2025. This determination was based on the “watermelon charts” provided by the Chesapeake Bay Program in Appendix P of the Verification Framework document. These watermelon charts provided the percent contribution from each BMP based on the state WIP. The resulting priority BMPs were grouped appropriately and are listed in Table D1-2. Verification protocols for other BMPs with lower anticipated contributions to the overall load reductions will be developed but at a slower pace, given the reduced reliance on these practices to Delaware’s reduction strategy.

Table D1-2. Highest Priority BMPs for verification protocol development.

| Sector | BMP Groupings |
| --- | --- |
| Agriculture | Cover Crops; Conservation Tillage; Grass Buffers; Manure Transport; Animal Waste Management Systems |
| Forestry | Forest Buffers |
| Restoration | Stream Restoration |
| Restoration | Wetland Restoration |
| Stormwater | Wet ponds and wetlands; Infiltration practices; Filtering practices; Bioretention; Bioswales |
| Stormwater | Erosion and sediment control |
| Wastewater | Septic Connections |

It is the responsibility of the implementing organization to verify that all data reported to the DNREC-DWS-NPS are complete, correct, and complies with all rules and policies of that organization. The independent Quality Assurance Manager conducts an additional review of compiled NPS BMP data for completeness, anomalies, errors, or questionable levels of implementation through a status and trends evaluation as a validation procedure. Section D2 provides a more detailed description of the data review, verification, and validation process for each BMP group listed in Table D1-2.

## D2. Verification and Validation Methods

DNREC hosted a BMP verification kickoff meeting in March 2015 with the 5 source sector groups (agriculture, forestry, stream and wetland restoration, stormwater, and wastewater) to review the CBP’s Verification Framework. Workgroups were formed for each of the five sectors listed above at the kickoff meeting (Table D2-1). The kickoff meeting was followed by two additional meetings for each individual workgroup where they developed Verification and Validation Protocols for each BMP group within each source sector. The resulting Verification and Validation Protocols are presented in sections D2.1 through D2.5.

Table D2-1. Source sector workgroups (Original)

** Denotes Workgroup Chairs

| Workgroup | Group member | Agency/Organization |
| --- | --- | --- |
| Agriculture | Ben Coverdale | DDA |
|  | Bob Coleman | DDA |
|  | Bob Palmer** | DNREC |
|  | Dale Churchey | DNREC |
|  | Dan Severson | UD |
|  | Debbie Absher | SCD |
|  | Gary Chambers | Perdue |
|  | Gene Vanderwende | DNREC |
|  | Jacob Urian | DNREC |
|  | Jayme Arthurs | USDA |
|  | Jen Nelson | Resource Smart Consulting |
|  | Jennifer Volk | UD |
|  | John Bushey | USDA |
|  | Kerin Hume | DNREC |
|  | Kip Foskey | SCD |
|  | Larry Towle | DDA |
|  | Lauren Torres | DDA |
|  | Marcia Fox | DNREC |
|  | Marianne Hardesty | USDA |
|  | Michael Biggs | DNREC |
|  | Rick Mickowski | DNREC |
|  | Robert Baldwin | DACD |
|  | Robin Talley | USDA |
|  | Sally Kepfer | USDA/NRCS |
|  | Sharon Webb** | DNREC |
|  | Susan Truehart | UD |
|  | Tim Riley | KCD |
|  | Tom Barthelmeh | DNREC |
|  | Tyler Monteith | DNREC |
|  | Ziggy Savage | USDA |
| Forestry | Bill Jones | DNREC |
|  | Bill Seybold | DDA |
|  | Bob Palmer | DNREC |
|  | Chris Miller | DDA |
|  | Craig Rhoads | DNREC |
|  | Jason Davis | DNREC |
|  | Jim Dobson | DDA |
|  | Kesha Braunskill | DDA |
|  | Kyle Hoyd | DDA |
|  | Lynn Manges | USDA |
|  | Marcia Fox** | DNREC |
|  | Mike Valenti | DDA |
|  | Sally Claggett | USFS |
|  | Sally Kepfer | NRCS |
|  | Sam Topper | DDA |
|  | Tom Barthelmeh | DNREC |
| Restoration (streams & wetlands) | Amy Jacobs | TNC |
|  | Bill Jones | DNREC |
|  | Brian Jennings | USFWS |
|  | Brittany Sturgis** | DNREC |
|  | Brooks Cahall | DNREC |
|  | Craig Rhoads | DNREC |
|  | Dale Churchey | DNREC |
|  | Elena Stewart | DNREC |
|  | Jake McPherson | Ducks Unlimited |
|  | Mark Biddle | DNREC |
|  | Robert Gano | DNREC |
|  | Sara Esposito | DNREC |
|  | Steve Williams | DNREC |
|  | Tim Garrahan | USDA |
|  | Tom Barthelmeh | DNREC |
|  | Tyler Monteith** | DNREC |
| Stormwater | Beau Croll | DNREC |
|  | Elaine Webb | DNREC |
|  | Eugenia Hart** | TetraTech |
|  | Jamie Rutherford | DNREC |
|  | Jared Adkins | KCD |
|  | Jennifer Roushey | DNREC |
|  | Jessica Watson | SCD |
|  | LaTonya Gilliam | DelDOT |
|  | Randy Cole | DelDOT |
|  | Randy Greer | DNREC |
| Wastewater | Andy Whitman | DNREC |
|  | Dave Schepens | DNREC |
|  | Jason Baumgartner | DNREC |
|  | Jennifer Walls** | DNREC |
|  | Jim Cassidy | DNREC |
|  | John DeFriece | DNREC |
|  | Ron Graeber | DNREC |
|  | Scott Eichholz | DNREC |

Delaware Source Sector Workgroups meet at minimum annually to review and discuss verification program needs.

### D2.1.  Agriculture Sector Verification Protocol

This section of the verification protocol represents the BMP groupings for the Agriculture Source Sector. Table F-1 in Appendix F provides a sector-specific checklist of Delaware BMP verification protocol components and maps them to the relevant QAPP sections where they are documented. Delaware’s verification program focuses on cost shared, regulatory, and permitted practices.  In the future, as Delaware implements the verification program, resource improvements will be incorporated.

D2.1.1. Cover Crops

D2.1.1.1 Cover Crops - Visual Assessment – Single Year:  Traditional and Commodity)

Through Delaware’s Conservation District’s Cover Crop Cost Share Programs, staff from each Conservation District – New Castle, Kent, and Sussex - inspect and report cover crop best management practices.  Each conservation district operates its own Cover Crop Program to ensure best management practices comply with respective cover crop policies.  Additional cover crop information is obtained from NRCS and FSA. Cover crop practices are annual practices.  Details regarding verification and validation procedures for cover crop practices are contained in Table D2.1.1.1 and summarized in the following sections.

Table D2.1.1-1. Visual Inspection – Cover Crops

| Jurisdictional Agriculture Verification Protocol Design Table: Visual Assessment BMPs - Single Year |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| High | Visual Assessment: Annual | Cover Crops | Visual Assessment | Annual | Trained District or NRCS Conservation Planners | District BMP Compliance Inspection Forms or NRCS Conservation Desktop | Single Year | 100% inspected annually | Not Eligible for Cost Share Funds | Single Year | 100% Tracked during inspections within District Cover Crop databases or NRCS Conservation Desktop. QA’d data entered into NPS BMP Database. |

D2.1.1.2 Cover Crops – Data Verification

Method

Cover Crop practices are non-regulatory and 100% of all practices are inspected after planting and destruction of crop.  The information is gathered annually to assure Cover Crop Program compliance.

Conservation District practices are reported at the site-level, with planting date, crop type, destruction date (if applicable).  Additionally, NRCS and FSA submit cover crop data; however, the data reported by these agencies are not as detailed as District Cover Crop Programs.  All BMPs meet NRCS standards, state standards, and Chesapeake Bay Program definitions for inclusion in the Chesapeake Bay Watershed Model.  Currently, resource improvement practices are not reported.

Verifiers

Cover Crops are visually inspected by District or NRCS Conservation Planners.  All District planners are trained in the interpretation of Cover Crop Program standards and specifications necessary to perform the inspections. Once cover crops are planted, the farmer will self-certify in writing by completing a certification form for the acres of cover crops planted.  These forms are mailed to the farmer along with a cost-share approval letter specifying how much cost-share the farmer may get.  Information requested on the certification form include farm and tract number, farm name, number of acres, type of cover crop species, seeding date, previous crop, and planting method.  After certification, District planners map the acres and physically inspect each field for program compliance.  In the spring, a destruction form is mailed to the farmer and they must certify in writing each field as destroyed.  The destruction certification form captures acres destroyed, acres harvested, destruction method, and destruction date.  The planners go out again to inspect each field to ensure the cover crop has been destroyed.  Application of commercial fertilizers or animal manures (N and P) are not allowed on crop fields intended to receive or is receiving cover crop incentive payments. During the course of inspections, a planner will document all cover crop information on District BMP inspection forms and NRCS information is entered into Conservation Desktop. When inspections are complete, and conditions are met, payment is made to the landowner.

Documentation of Verification

BMPs are inspected by Conservation District Planners and documented on District specific inspection forms.  Each District has separate databases for their cover crop programs - New Castle Conservation District, Kent Conservation District, and Sussex Conservation District.  Cover crop data is entered into spreadsheets by the Conservation Planners. The data are maintained on private servers within each of the three Districts. Information from both the planting and destruction certification forms is recorded into an Excel spreadsheet.  Data recorded includes the tract number, watershed name, crop species, total acres, harvested acres, destroyed acres, planting date, cost-share amount, planting method, destruction method, and destruction date.  Future spreadsheets will also record prior crop.

Additional cover crop information is provided by NRCS and FSA; however, the data reported by these agencies are not as detailed as District cover crop data.  NRCS data are provided by USGS, per the Basinwide 1619 Agreement for CBP modeling, at the state and county level to be evenly distributed.  FSA data are aggregated and reported by the state office as part of the federal crop insurance program.  Farmers are required to annually file a crop report, certifying the location and acres of crops.  Crop reports are printed for producer signatures and maintained in files at the county office.  The data is also stored by FSA’s computer system.  FSA data are submitted at the lowest nutrient use efficiency.

End of contract/project lifespan

Cover crop BMPs are annual practices and thus have a lifespan of one year. New data are reported annually by all cover crop implementing agencies.

D2.1.1.3 Cover Crops – Data Validation

Quality Assurance

All (100%) cover crop practices are inspected annually.  Inspections are made after implementation and after destruction/harvest. All records are provided to DNREC’s Nonpoint Source Program (NPS) for inclusion in Delaware’s existing NPS BMP Database and submission to the CBP through NEIEN.

The DE BMP Tracking and Reporting Tool is an online database that serves as a means of reporting and tracking BMPs in the state of Delaware. The use of this tool allows for a more streamlined approach to submitting data for bay program progress and for generating reports needed for water quality assessment and monitoring purposes. This database is used to submit data for inclusion in the Chesapeake Bay Watershed Model. Individual organizations are responsible for entering their practices into the DE BMP Tracking and Reporting Tool with their provided login information and are only permitted to review their own data.

The data are entered into the agriculture Excel template (Appendix I) for upload into the NPS BMP Database. To confirm that the correct information has been copied into the templates from the raw data, the total area (e.g., acres) or numbers for each BMP type are summed and compared to the original dataset to be sure there were no errors in translating the data. Data are also compared to previous years’ submissions to see if numbers are consistent (i.e., no extreme increases or decreases in acreage or count). The methodology for entering data for each BMP type is recorded every year so that the same methodology is consistently used year to year. See QAPP Section D2.1 for specific agricultural BMP submittal methodology.

The BMP progress data are submitted to CBP every year by DNREC.

Data Entry

Data are collected and entered into the respective cost sharing entity’s database by trained staff.   Cover crop data are provided to NPS to input into the DE BMP Tracking and Reporting Tool using the NEIEN input template with the correct NEIEN BMP names (see QAPP Section B10 for additional details on the parties involved in data submission to NEIEN). DE BMP Tracking and Reporting Tool is mapped to provide the data required to NEIEN and the CBP.

The lifespan or credit duration of each BMP is also entered in the template based on the CreditDurations05222015.xlxs spreadsheet provided by CBP. The code in the DE NPS BMP Database has been modified so that the lifespan/credit duration is added to the implementation date of a particular BMP to calculate the Lifespan End Date. Once the Lifespan End Date has been passed, that BMP will be tagged as “retired”.

Double counting is avoided by submitting data by the primary funding source or the primary implementing agency. For example, BMP implementation data that is cost-shared with NRCS is submitted by NRCS. Cover crop data are also submitted by the conservation districts.

The DNREC BMP Tracking and Reporting Application (https://bmptracker.dnrecapps.net/)  was revised in 2019 as the means of collecting and managing the nonpoint source sector best management practices (BMPs) implemented in the State of Delaware. It replaced the DE NPS BMP Database on December 1, 2019 as the central repository of BMP data from multiple agencies and departments allows a streamlined approach to generate reports needed for BMP tracking, BMP reporting, and water quality assessments. DNREC personnel use the BMP Tool to submit data to the Chesapeake Bay Program as required for the Bay TMDL. The BMP tool also facilitates analyses like evaluating BMP trends and targeting.

This Excel spreadsheet contains a template for reporting and tracking best management practices (BMPs). The BMP template helps agencies in the identification and regular reporting of best management practices to the Chesapeake Bay Program. Reported practices receive credit towards meeting the goals of the Chesapeake Bay nutrient and sediment total maximum daily loads (TMDLs).

External Data

Data are provided from external agencies as mentioned above.  The data are reviewed for accuracy – correct reporting period and that all necessary fields for NEIEN have been included.  District data are the most specific data reported and are specific to the Chesapeake Bay watershed.  NRCS and FSA acres are provided for the entire state/county (not just the CB watershed) so they need to be spread evenly. Any NRCS cover crop acres are subtracted from the FSA cover crop acres and any remaining acres were included as “Commodity Cover Crop Late Other Wheat” for minimum credit.

Historic Data Verification

Historic data were captured in June 2015 – see NEIEN Methodology for Historical Data Clean Up (Appendix H).  Data quality assurance and data entry were conducted as discussed in the above sections.

BMP Performance

During the visual field assessments, BMPs are inspected for compliance or failure by implementing agency. If a BMP is not performing up to its standards and specifications, a maintenance inspection report or letter is provided to the landowner. Agency staff work with landowners to bring the BMP back into compliance or the landowner must pay back the funds used to implement the BMP.

BMP inspection records are entered into the NPS BMP Database.  Each BMP is assigned a lifespan or credit duration. A BMP will be considered “retired” once the Lifespan End Date has passed. If the BMP is re-inspected and deemed functioning (pass), it will be entered back into the database for inclusion in progress reporting. Any BMP deemed failing (fail), will be retired from the system until the BMP is brought back into compliance.

D2.1.2 - Soil and Water Conservation Plans

D2.1.2.1 - Soil and Water Conservation Plans- Non-Visual Assessment – Annual

A Conservation Plan is a written record of management decisions and conservation practices and systems used to develop and maintain a farm. NRCS and Conservation District Planners write the plans for farmers as required for Farm Bill Program eligibility. The plan contains a listing of the conservation practices and a schedule for implementation. Included with these practices are a description of the impacts of the selected practices on their natural resources. The plans are used by the farmer to achieve goals and maintain the resources of the land.  This BMP is a non-visual assessment BMP as it is written once and is considered permanent until land management changes.

| Delaware Agriculture Protocol Design Table: Non-Visual Assessment BMPs – Annual |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| High | NRCS Policy – required for Farm Bill Program eligibility | Conservation Plans – Full Farm Plan | Resource Inventory & Plan Development | 1/year | NRCS and Districts | Plan Development and CPA06 | Upon Plan Amendment | N/A | Ineligibility for cost share funds | Permanent until management changes | Conservation Desktop |

D2.1.2.1 Table - Soil and Water Conservation Plans

D2.1.2.2 - Soil and Water Conservation Plans- Data Verification

Method

Soil and Water Conservation Plans meet NRCS standards, state standards, and Chesapeake Bay Program definitions. Each of Delaware’s Conservation Districts’ partners with USDA's Natural Resources Conservation Service (NRCS) in developing conservation plans.  One hundred percent (100%) of all Conservation Plans are inspected during the lifespan of the practice.  This BMP is reported only by NRCS.  All plans are non-visually assessed each year as required by NRCS contracts.  Funding for this BMP is provided by USDA programs or state cost share funding.

Conservation plans are written by District or NRCS Conservation Planners.  The plans are required by NRCS policy for Farm Bill Program eligibility.  Any landowner seeking BMP cost share funding must have an active Soil and Water Conservation Plan.   District and NRCS enter data into Conservation Desktop for inclusion in the Chesapeake Bay Watershed Model progress runs.

Verifiers

All practices are visually inspected on-site during the lifespan of the BMP as required by USDA. NRCS verification timing will be at the organization’s discretion.  BMP inspectors are trained NRCS or District Planners. Training is ongoing as all new personnel are trained in the collection of BMP data; however, there is no “certification requirement” for staff collecting BMP data. If any of the data collectors have questions regarding functionality, contact is usually made with USDA NRCS.

Documentation of Verification

This BMP is inspected and entered into NRCS Conservation Desktop by trained NRCS or District planners.  An outline of practice data submissions can be found in section B10 of Delaware’s QAPP. Additionally, in 2010, an agreement (Basinwide 1619 Agreement) was reached to have federal agencies, such as the USDA’s NRCS, report practices directly to the USGS for CBP modeling rather than have jurisdictions report on their behalf.  All NRCS data are aggregated at the County level.  All BMPs currently reported are approved by CBP for inclusion in model application.

End of contract/project lifespan

Once the practice contract expires with NRCS, the BMP is retired from the NPS BMP database.  It is the implementing agency’s discretion to submit updated data to the DNREC Quality Assurance Officer for inclusion in Delaware’s reporting and BMP tracking database.

D2.1.2.3 - Soil and Water Conservation Plans- Data Validation

Quality Assurance

All plans are inspected within the practice lifespan by NRCS or Conservation District staff. Additionally, checks are made upon implementation and before contract end dates by the funding agency; hence, BMPs are verified for functionality.  Inspection records (pass/fail) will be provided to DNREC’s Nonpoint Source Program (NPS) for inclusion in Delaware’s existing NPS BMP Database and submission to the CBP through NEIEN.

The DE BMP Tracking and Reporting Tool is an online database that serves as a means of reporting and tracking BMPs in the state of Delaware. The use of this tool allows for a more streamlined approach for Chesapeake Bay progress submission and generating reports needed for water quality assessment and monitoring purposes. This database is used to submit data for inclusion in the Chesapeake Bay Watershed Model. Individual organizations are responsible for entering their practices with their provided login information and are only permitted to review their own data.

The BMP progress data are submitted to CBP every year by DNREC.

Data Entry

Data are collected and entered into NRCS Conservation Desktop by trained staff.   Data are provided to NPS to input into the DE BMP Tracking and Reporting Tool using the NEIEN input template with the correct NEIEN BMP names (see QAPP Section B10 for additional details on the parties involved in data submission to NEIEN). DE BMP Tracking and Reporting Tool is mapped to provide the data required to NEIEN and the CBP.

External Data

Data are provided from external agencies as mentioned above.  Double counting is avoided since this BMP is reported by one agency.  The data are checked to be sure that they have been provided for the correct time period and that all necessary fields for NEIEN have been included.

Historic Data Verification

Historic data were provided by DNREC and NRCS. Data quality assurance and data entry were conducted the same way as in the past (as discussed in the above sections).

BMP Performance

Landowners must have a conservation plan to be eligible for federal cost share funding.  All on-farm BMPs are inspected during this time; however, the conservation plan does not expire until in farming operation changes occur.  When developing a conservation plan, visual field assessments are made and BMPs are inspected for compliance or failure. If a BMP is not performing up to its standards and specifications, a maintenance inspection report or letter is provided to the landowner. Agency staff work with landowners to bring the BMP back into compliance.

D2.1.3 Nutrient Management Planning

D2.1.3.1 Nutrient Management Planning -Non-Visual Assessment – Annual

DNREC, DDA, and the Delaware Nutrient Management Commission (DNMC) have statutory and regulatory authority to manage animal agricultural programs in Delaware, including the Delaware Nutrient Management Law and regulations.  Under this law, DDA inspects Nutrient Management Plans (NMPs) for compliance with Nutrient Management Law & Regulations.  NMPs are cost shared through several agencies in the state including DDA and NRCS.  Farmers that meet the criteria for Nutrient Management are required to become certified through a partnership with University of Delaware.  Once certified, plans are written and captured through a cost share program with NRCS or DDA.  Plans are housed on the farm and the Delaware Nutrient Management Commission is notified of plan development.  Certified Individuals - generators, handlers (private or commercial), and consultants - are required to submit annual reports for the implementation of nutrient management planning activities. Annual reports are recorded with DDA.  Visual inspections are done randomly at the discretion of DDA or by complaints and occur, on average, on approximately 17.7% of farms annually.  DDA staff meets with individuals to determine if compliant with the law and discuss any additional requirements needed to become compliant.  At this time, additional BMPs like animal waste facilities or composters are reviewed.  Landowners have 14 days to make corrective actions; failure to do so will result in penalties documented in the Nutrient Management Law and regulations.

2.1.3.1 Table – Nutrient Management Planning

| Delaware Agriculture Protocol Design Table: Non-Visual Assessment BMPs – Annual |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| High | Non-Visual Assessment: Annual | Nutrient Management Core N & P (Supplemental rate, timing, placement) | Visual Assessment: Plan documented at DDA | Annual | Trained DDA Staff | NM Evaluation Report Forms | Single Year | 100% | Not Eligible for Cost Share Funds | Single Year | Self NM Annual Report; Inspection report; public formal complaint process; all recorded in database |
| High | NMP Creation, Cost Share – Conservation District | Nutrient Management Core N & P cost share | Visual Inspection; Plan Creation | Every 3 years | District Staff – Certified Nutrient Consultants | The NM Plan written to NRCS 590 standard | None – authority transferred to DDA until another NMP update is needed |  |  | 3 years | Plans are housed at District Office. Information is entered into District Database. |
| High | Cost Share | Nutrient Management Core N & P | Farm Record Review | Annually | NRCS | 590 EQIP Standard | Annual | All Contracts | Violation of contract; consultant will need to correct | 3 years | Conservation Desktop |
| High | Cost Share | Nutrient Management Core N & P | Farm Record Review | Annually | NRCS | CSP – Conservation Security Program for Advanced NM Plan | Annual | All Contracts | Violation of contract; consultant need to correct | 5 years | Conservation Desktop |
| High | On-farm record review: Permit and Regulation. | Nutrient Management Core N & P : Nutrient Management Annual Report Review and Compliance Checks | Farm Record Review | Annually | DDA | Invoice for Cost Shared Plan and Inspection Forms for Compliance checks | Compliance Check | 17.7% inspections/ year. Farmers are required to submit an annual report (Appendix) | Work with landowner or consultant to correct | Annual | Self NM Annual Report; Inspection report; public formal complaint process; all recorded in database |

D2.1.3.2 Nutrient Management Planning -Data Verification

Method

Nutrient management practices are regulatory and all practices are non-visually inspected annually through agencies involved in the nutrient management process – rigorously certified private planners or Conservation Districts and NRCS develop and continuously update the plans; DDA receives annual reports documenting implementation of plans; and based on recent previous compliance rates between 75 and 85 percent the Program estimates 17.7% of farms require verification evaluations annually (See Appendix A, Table 1).. The information is gathered annually to assure landowners comply with Delaware’s Nutrient Management Law.  The Nutrient Management Law requires Delaware to make nutrient consultants available through the conservation districts to provide free Nutrient Management Plans (NMPs) to anyone requesting assistance, or to reimburse at a determined rate anyone who chooses to hire a private nutrient consultant.

Plans for nutrient management comply with all applicable federal, state, and local laws and regulations. Nutrient management plans are developed in accordance with policy requirements of the NRCS General Manual Title 450, Part 401.03 (Technical Guides, Policy and Responsibilities) and Title 190, Part 402 (Ecological Sciences, Nutrient Management, Policy); technical requirements of the NRCS Field Office Technical Guide (FOTG); procedures contained in the National Planning Procedures Handbook (NPPH), and the NRCS National Agronomy Manual (NAM) Section 503.

One hundred percent of all nutrient management plans from newly certified planners are inspected at initial implementation or plan conception by the Nutrient Management Program Administrator.  Several agencies conduct follow-up inspections; however, DDA follows compliance requirements per nutrient management regulations.  DDA reviews approximately 17.7% of plans annually.  The Program staff can select an operator and/or operation for audit via any of the following manners:

- Random selection- operators may be picked at random from a database of contacts by Program staff scientist at any given time- not due to non-compliance or complaint- to reach target verification evaluation numbers

    - Selection interval follows the recommendation in Appendix A below:  Sampling Recommendations for Delaware Nutrient Management Verification

- Consultant selection- if willing, consultants can help arrange verification evaluations for their clients to mitigate the travel and schedule burdens of random audits.

- Targeted by Non-Compliance- an operator may be selected for a verification evaluation due to annual report inaccuracies or incompleteness or non-compliance from previous audit.

- Targeted by Program staff- an operator may be selected for a verification evaluation due to complaint investigation initiated by a public citizen or program official. If the Program staff investigates a complaint and has suspicion of non-compliance, officer may ask Program staff scientist to conduct verification evaluation.

Participation in other state and federal programs – higher inspection rates for select groups of farmers participating in state land leasing, federal cost-share programs or other programs for which nutrient management activities are involved may be preselected by a fellow agency for Nutrient Management Program verification evaluation based on available funding and program verification goals.  These evaluations may be performed with Program partner staff in an advisory role and may result in regulatory enforcement beyond the scope of the Program itself.

Each operation is required to have a minimum of one (1) certified nutrient management operator. The certified operator or a certified designee should be present for the nutrient management verification evaluation. An hour-long verification evaluation should receive one nutrient management credit. No more than one credit should be issued per verification evaluation within each renewal cycle.

  - Animal only (and CAFO no-land – GP1): Operator(s) should be certified as Nutrient Generator or higher.

  - Land plans (and CAFO with land – GP2 & 3): Operator(s) should be certified as Private Nutrient Handler or higher.

The Nutrient Management Plan or Animal Waste Management Plan is to be written by a certified nutrient management consultant without conflict of interest in the operation. An Animal Waste Management Plan may be written by the certified (Nutrient Generator or higher) operator as outlined below. During the verification evaluation the Program staff scientist will check for completeness of each plan according to the plan requirements.

Each operator is required to keep certain records of implementation outlining their operation practices for five years. We advise a minimum of previous 3 crops or 24 months of history, whichever is less, be available upon inspection. Plans that are cost-shared or updated annually would need to present two consecutive plans.  Two- and three-year plans cover the requested time period for routine inspection.  The Program Staff scientist may inspect as little as 12 months of record keeping for the following areas.

  - Actual Yield: Specific field or management unit yield information for the last 7 years or less if the yield goal is based on less historical information.

  - Nutrient Type(s): Type of nutrients applied such as inorganic fertilizer, organic fertilizer (manure), or other (ex. DAF). i.e. fertilizer receipts or manure weigh tickets; note: if all inorganic fertilizer was purchased and applied by a custom application fertilizer company, then the invoice detailing the total blend weight would be sufficient.

  - Analysis/Nutrient Content of fertilizers or manures: N-P-K analysis of fertilizers applied i.e. fertilizer labels, blend work orders, manure analyses.

  - Application Rates & Quantity:  Pounds, gallons, or tons applied per acre and total amount applied per total crop acres per application period. Applications such as pre-plant, side-dress or fertigation should be itemized by quantity and traceable to analysis as indicated above. i.e. application records sent by a co-op, log book of applications kept by operator or bulk order receipts with supplemental records of annualized rates.

It is expected that all records are kept on a field scale. Two to three fields will be evaluated per operation. Program staff, at their discretion, may accept farm gate scale records in the few cases where the operation has homogenous management across multiple fields and records to match.

  - Application Timing & Placement: Date(s) applied and indicated method such as banded or starter.  Date of last calibration of spreading equipment should be verified as appropriate. i.e. log book of maintenance and application records kept by operator or plan notations in the margins

  - Manure Management Information:  Manure type, date of removal from production area or manure shed, receiver information, approximate tonnage removed (if applicable). i.e. manure export records kept by operator, receipts from manure export company, weigh tickets from certified truck scales.

The inspection of Component BMPs is outlined below, by BMP type:

  - Phosphorus Site Index (PSI) (if applicable): spot check records of actual nutrient application on fields where a PSI has been calculated to make sure the actual application did not result in over fertilization of phosphorus (P).  If no PSI has been performed on fields with a FIV >150, then the P application can be no more than a 3-year crop removal rate. (Staff would need a printout detailing the University of Delaware's crop removal rates by bushel and crop)  See https://cdn.extension.udel.edu/wp-content/uploads/2016/03/23084930/Part-A-phosphorus-loss-potential-due-to-site-and-transport-characteristics.pdf and https://dda.delaware.gov/nutrients/downloads/PSI_DE_All_Counties_110910.pdf

  - Carry over nitrogen credits: Legumes and cover crops provide plant available nitrogen in the next season with sufficient yield of biomass. Residual Nitrogen values can be estimated using the Mid-Atlantic Nutrient Management Handbook (https://s3.amazonaws.com/udextension/ag/files/2013/06/The-Mid-Atlantic-Nutrient-Management-Handbook-2006.pdf). Additionally, available nitrogen from legume cover crops, non-legume cover crop, or any other green manure source, can be assessed by determining plant available nitrogen (Staff would need to see available nitrogen test results and the provider of the results).

  - Pre-side-dress Nitrate Tests (PSNT): Provide all relevant test results on all fields that received manure applications to check that side-dress N application rates were within the allowable range recommended by a certified consultant or lab.  (Example: UD calculation table & knowledge of how to perform the calculations to determine the side-dress N recommendation - See Attached) http://extension.udel.edu/factsheets/nitrogen-removal-by-delaware-crops/ http://extension.udel.edu/factsheets/phosphorus-removal-by-delaware-crops/

Nutrient Management practices – both Core N and Core P (as well as supplemental rate, timing, placement – are reported at the watershed-level, with implementation date, and acreage. The BMPs featured in this section meet NRCS standards and state standards. Resource improvement practices are not reported.

Verifiers

Once a NMP is completed and implemented, the NMPs must be maintained on-site and must be made available for inspection by the DDA.  Each person who is required to implement a NMP must submit an annual report to the DDA by March 1 of every calendar year, on a form developed and supplied by the DDA. The report details nutrient handling activities that occurred during the previous calendar year, including at a minimum:

- The amount of animal waste applied to the land and the area of land to which it was applied.

- The amount of animal waste transferred for alternative uses (if applicable); and

- The amount of inorganic fertilizer applied to the land.

Information obtained from these reports is used to verify the existence and utilization of the NMP by the DDA. Any revisions to the NMP must be justified, documented, and included in the records. Any significant alterations in operations or upon a 25% or greater increase in operation caused by unforeseen circumstances (ex. weather) that occur prior to a NMP's expiration date will require an addendum to the NMP from the certified nutrient consultant. In the event that 1-2 years of records does not exhibit compliance with the plan specifications, a Program staff scientist, in their sole discretion can evaluate antecedent conditions and records to better evaluate overall farmer performance.

- Additional BMPs may be inspected for crediting practices that exist as separately tracked items.  Some of these may be required for CAFO compliance and others may be collected in an effort to better capture BMP extent across the state for non-cost shared practices.

  - Animal Operation BMPs:

    - Composter

    - Mortality Freezers

    - Manure Shed

    - Heavy use area pads (HUAPs) or concrete end pads

    - Stockpiling

    - Storm Water Retention Pond

    - Pasture Rotation

    - Pasture Stream Fencing

    - Grassed Waterway

    - Windbreaks

    - Regular Manure Sample

  - Land Operation BMPs:

    - Temporary Field Staging

    - Application Setbacks

      - Can be verified with inquiry and application rate and quantity information

    - Application Rates less than Recommendations

    - Cover Crops

    - Grid Soil Sampling

    - Annual or semi-annual Soil Test

    - PSNT; following a recommendation from a certified consultant or approved lab.

    - CSNT; following a recommendation from a certified consultant or approved lab.

    - Precision Application

    - Yield Mapping

    - Strip Trials

    - Split N or P Applications

    - Variable Rate N or P Applications

    - Manure Incorporation

    - Subsurface Injections

Documentation of Verification

In addition to reviewing the completed NMP, DDA staff inspects approximately 17.7% of farms annually to verify the contents and implementation of the Nutrient Management Plan. Agency staff are trained in NM program standards and specifications necessary to perform the inspections.  Each DDA inspection reviews the records maintained by the landowner. During a verification evaluation two different parties are subject to evaluation- the operator and the certified Nutrient Management Consultant (Consultant). The consultant is held accountable for elements of the plan that are missing or incomplete. The operator is held accountable for the certification, implementation (e.g. records) and farm management aspects of the audit. Compliance and non-compliance will be aggregated on a semi-annual basis for reporting purposes under various agreements, but protecting anonymity where legally required. The levels of compliance are listed below:

- Compliance: an operation is in compliance if all aspects of the verification evaluation pass with no reason to follow up. Certification status is enabled and up to date, the operation plan is complete and valid, records of implementation are kept and complete, and farm management is up to specification.

- Substantive Compliance: an operation is in substantive compliance if the findings of the verification evaluation have a good explanation and are within nutrient management recommendations. (i.e. the operator applied less N or P than the plan recommended and this rate-maintained productivity.)

- Procedural Non-Compliance: an operation could be in procedural non-compliance if the findings of the verification evaluation have a good explanation and within nutrient management recommendations. (i.e. According to plan a field should have been planted in corn, instead soybeans were planted due to weather the nutrients however were applied at soybean rate.)

- Non-Compliance: an operation will be in non-compliance if there is an environmental issue that needs to be addressed. (i.e. not following nutrient management plan recommendations, records of implementation not being kept, waste being handled improperly.)

Records include but are not limited to:

- The following are required by §2247 (c) of the DE Nutrient Management Law:

  - Soil test results,

  - Recommendations for nutrient application,

  - Rate and sources of nutrients applied,

  - Manure analysis provided free by State Compliance Lab,

  - Crops planted with estimated yields,

  - Crop residues removed,

  - Dates of nutrient applications,

  - Method of application including type of incorporation, if applicable.

- In addition to what the Law requires to be presented by the producer at time of inspection:

  - Documentation of the actual rate at which nutrients were applied. When the actual rates used differ from or exceed the recommended and planned rates, records will indicate the reasons for the differences.

  - Actual yields with documentation of reasons for missed expectations.

  - Results of applicable water, plant, and/or organic by-product analyses.

  - Amount and type of manure exported from the farm and the name, address, and organization responsible for exporting manure.

Follow up practices will be communicated to the operator and/or the consultant at the conclusion of a verification evaluation. If compliant, no verification evaluation should be repeated for up to 5 years, unless otherwise determined by selection process. Follow-ups can be made as soon as two weeks or as long as 1 year determined by the cause of follow up. These guidelines are outlined below.

- Compliant: normal selection criteria employed

- Substantive Compliance: normal selection criteria employed; next random selection will seek to cull further deviations.

- Procedural Non-compliant: discretion of program staff

- Non- Compliant: follow up verification evaluation as soon as 2 weeks and as late as 1 year depending on cause, examples provided:

  - Waste storage facility being improperly used, or not tidy- 2 week follow up

  - Insufficient records- 1-year follow-up implementation verification

  - No plan or out of date plan- 3-month follow-up to validate current plan and subsequent 1-year implementation verification

On-farm records are maintained for a minimum of six years or longer as required by regulations.

The written guidance and documentation on verification systems for Nutrient Management Plan development and implementation is found on DDA’s Nutrient Management Evaluation Report.  Information obtained from evaluation reports and annual reports are entered into a web-based database by DDA. The required annual reporting information is entered when received, and aggregated data reports are generated to provide information on nutrient management planning implementation and nutrient handling activities throughout the state to establish trends and indicate nutrient use efficiency gains at an aggregated scale.

End of contract/project lifespan

Nutrient management planning BMPs are typically annual practices but can also be written on a 3-year cycle and thus have a lifespan of one to three years. New data are reported annually for each progress reporting period through the Nutrient Management Programs Annual Report.

D2.1.3.3 Nutrient Management Planning -Data Validation

Quality Assurance

All nutrient management plans are inspected upon implementation and results recorded with DDA (via annual reports). Other agencies involved in the nutrient management process include Conservation Districts and NRCS in plan development; and DDA inspects 17.7% of farms annually for compliance. All plan inspections are made before cost share funding is received by the cost sharing agency. DDA conducts 17.7% QA checks annually for compliance of the Nutrient Management Law.  Nutrient management plan data are reported annually to DNREC’s Nonpoint Source Program (NPS) for inclusion in Delaware’s existing DE BMP Tracking and Reporting Tool and submission to the CBP through NEIEN.  In 2016, methods for collection of soil test phosphorus categories will support decadal trends in high fertility index value P soils downwards, as well as an upward trend in low FIV P soils, indicating drawdown of hotspots, utilization of manure resources where agronomic need can be met and maintenance of optimum testing soils.

The DNREC BMP Tracking and Reporting Application (https://bmptracker.dnrecapps.net/) is the means of collecting and managing the nonpoint source sector best management practices (BMPs) implemented in the State of Delaware. A central repository of BMP data from multiple agencies and departments allows a streamlined approach to generate reports needed for BMP tracking, BMP reporting, and water quality assessments. DNREC personnel use the BMP Tool to submit data to the Chesapeake Bay Program through NEIEN as required for the Bay TMDL. The BMP tool also facilitates analyses like evaluating BMP trends and targeting.

The Excel spreadsheet contains a template for reporting and tracking best management practices (BMPs). The BMP template helps agencies in the identification and regular reporting of best management practices to the Chesapeake Bay Program. Reported practices receive credit towards meeting the goals of the Chesapeake Bay nutrient and sediment total maximum daily loads (TMDLs).

The BMP progress data are submitted to CBP every year by DNREC.

Data Entry

Data are collected and entered into the respective cost sharing entity’s database by trained staff.   Nutrient Management data are provided to DNREC to input into the DE BMP Tracking and Reporting Tool using the NEIEN input template with the correct NEIEN BMP names (see QAPP Section B10 for additional details on the parties involved in data submission to NEIEN). DE BMP Tracking and Reporting Tool is mapped to provide the data required to NEIEN and the CBP.

The lifespan or credit duration of each BMP is also entered in the template based on the CreditDurations05222015.xlxs spreadsheet provided by CBP. The code in the DE BMP Tracking and Reporting Tool has been modified so that the lifespan/credit duration is added to the implementation date of a particular BMP to calculate the Lifespan End Date. Once the Lifespan End Date has been passed, that BMP will be tagged as “retired” unless it has been recertified in that lifespan as continuing to operate consistent with NRCS or Chesapeake Bay Program Office approved BMP definitions, whichever is more easily visually certifiable.

Double counting is avoided by submitting data by the primary funding source or the primary implementing agency. For example, BMP implementation data that is cost-shared with NRCS is submitted by NRCS. Non-cost shared data are submitted by the state or conservation districts.

External Data

Data are provided from the DDA. The data are reviewed for accuracy – correct reporting period and that all necessary fields required for NEIEN have been included.  Nutrient management planning acres are calculated by multiplying the measured annual compliance rate by the total number of acres of nutrient management in the State. The annual compliance rate is the percentage of operations passing inspection each year as compliant or substantive compliance. DDA inspects operations to a statistical representation of all operations in the state, as approved by the CBPO Agriculture Workgroup’s Verification Protocol. The total number of acres of nutrient management is based on the annual reported acres under nutrient management plans.  The plans reported are validated with cost-share payments and reporting from the Conservation Districts.

Historic Data Verification

Historic data were submitted by using the NEIEN Methodology for Historical Data Clean Up (Appendix H).  Data quality assurance and data entry were conducted as discussed in the above sections.  Compliance through history is not obtainable based on the adaptive management established through improving verification protocols.

BMP Performance

Nutrient management planning acres are annual practices. Performance is only noted during inspections conducted by DDA. The inspection results of a statistically defensible sample size of 17.7% will be used to determine compliance.  All data are entered annually and will supersede any previous records.

For Core NM:

- Both N and P core NM credit requires nutrient application rates according to LGU recommendations as outlined in the Delaware Nutrient Management Program State Technical Standard ‘Nutrient Management Code 590.’ This standard applies to producers who apply nutrients to ten acres or more and/or have 8 Animal Units or more.

- Both N and P core NM credit requires yield estimates and cropping plan at field management unit as outlined in the Delaware Nutrient Management Program State Technical Standard ‘Nutrient Management Code 590.’ This standard applies to producers who apply nutrients to ten acres or more and/or have 8 Animal Units or more.

- Both N and P core NM credit requires cropping and manure history at field management unit as outlined in the Delaware Nutrient Management Program State Technical Standard ‘Nutrient Management Code 590.’ This standard applies to producers who apply nutrients to ten acres or more and/or have 8 Animal Units or more.

- Both N and P core NM credit requires manure analysis as outlined in the Delaware Nutrient Management Program State Technical Standard ‘Manure Sampling and Analysis.’ This standard applies to producers who plans to land apply manure or manure and fertilizer.

- Both N and P core NM credit requires a spreader and applicator calibration as outlined in the Delaware Nutrient Management Program State Technical Standards ‘Calibrating Fertilizer Applicators’ and ‘Calibrating Poultry Litter and Other Solid Manure Spreaders.’ The first standard applies to all farming operations that land apply commercial fertilizers, and the second applies to all farming operations that land apply poultry litter and other solid manures.

- P core NM credit additionally requires P soil tests at field management unit level as outlined in the Delaware Nutrient Management Program State Technical Standard ‘Nutrient Management Code 590.’ This standard applies to producers who apply nutrients to ten acres or more and/or have 8 Animal Units or more.

For Placement:

- Both N and P placement NM credit requires either subsurface application or application setbacks. Setbacks are required according to the Delaware Nutrient Management Program State Technical Standard ‘Field Application Setbacks.’ This practice applies to all CAFOs in the state and requires a 100-foot setback or 35-foot vegetated buffer adjacent to water.

For Rate:

- P rate NM credit requires either P based manure rate based on annual crop P removal, P rate less than the LGU recommendations or variable P rate at sub-field management unit level. The Delaware Nutrient Management Program State Technical Standard ‘Nutrient Management Code 590’ requires a Phosphorus Site Index (PSI) test wherever phosphorus levels are higher than agronomic need.

D2.1.4 Manure Relocation (Transport)

D2.1.4.1 Manure Relocation (Transport) Non - Visual Assessment – Annual

Through the Department of Agriculture’s Manure Relocation Cost Share Program, the State can report manure transport inside and outside of the Chesapeake Bay Watershed.  Manure relocation practices are annual practices.  Details regarding verification and validation procedures for manure relocation practices are contained in Table D2.1.4.1 and summarized in the following sections.

2.1.4.1 Table – Manure Relocation

| Jurisdictional Agriculture Verification Protocol Design Table: Non-Visual Assessment BMPs - Annual |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| High | Non-Visual Assessment: Annual | Nutrient Transport | Self- Certification | Annual | Trained DDA Staff | Cost assistance receipts | Single Year | 100% | Not Eligible for Cost Share Funds | Single Year | 100% NM Transport Cost Share |

D2.1.4.2 Manure Relocation (Transport) Data Verification

Method

Manure relocation practices are non-regulatory.  Only a portion of the manure transported throughout the state are captured through DDA’s Manure Relocation Cost Share Program.  The Delaware Nutrient Management Relocation Program is available annually on a first come, first served basis. Current funding is provided to the DDA through CWA Section 319 Grant and Chesapeake Bay Implementation grant funds, and state cost share funds.  To apply for cost assistance, landowners submit an application to the Delaware Nutrient Management Program. Once the application has been approved, the landowner will receive a letter of approval and a Claim for Payment form. After completion of the manure transport, the landowner must send in the Claim for Payment form and the weight slips for payment. Payment of cost assistance is contingent upon funding availability.

Data are gathered annually through the Nutrient Management Relocation Program cost assistance program. Information obtained from this program is the only verifiable and reportable data for relocation.  The BMPs featured in this section meet state standards and Chesapeake Bay Program definitions. Practices captured through this process include: Manure Transport Outside and Inside the Chesapeake Bay Watershed.  This section does not include resource improvement practices.

Verifiers

DDA staff is trained in Nutrient Management (NM) Relocation Program standards and specifications necessary to perform the inspections. The written guidance and documentation on the data collection and verification systems is found on DDAs Nutrient Management Relocation Program website: http://dda.delaware.gov/nutrients/nm_reloc.shtml

Documentation of Verification

NM Relocation Program information is recorded and maintained on NM Relocation Program application forms.  Data recorded includes the transport agent, eligible sender and eligible receiver/nutrient destination. Data is transferred from a paper copy to an Excel spreadsheet and maintained on DDA’s servers.

End of contract/project lifespan

Manure relocation BMPs are annual practices and thus have a lifespan of one year. New data are reported annually by DDA.

### D2.1.4.3 Manure Relocation (Transport) Data Validation

Quality Assurance

All (100%) cost-shared manure relocation practices are recorded annually.  Forms are verified by DDA staff and compared to manure handler reports before payment is made.  All records are provided to DNREC’s Nonpoint Source Program (NPS) for inclusion in Delaware’s existing DE BMP Tracking and Reporting Tool and submission to the CBP through NEIEN.

The data are entered into the agriculture Excel template (Appendix I) for upload into the NPS BMP Database. To confirm that the correct information has been copied into the templates from the raw data, the total area (e.g., acres) or numbers for each BMP type are summed and compared to the original dataset to be sure there were no errors in translating the data. Data are also compared to previous years’ submissions to see if numbers are consistent (i.e., no extreme increases or decreases in acreage or count). The methodology for entering data for each BMP type is recorded every year so that the same methodology is consistently used year to year.

The BMP progress data are submitted to CBP every year by DNREC.

Data Entry

Data are collected and entered into DDA’s manure relocation database by trained staff.   Data are provided to NPS to input into the DE BMP Tracking and Reporting Tool using the NEIEN input template with the correct NEIEN BMP names (see QAPP Section B10 for additional details on the parties involved in data submission to NEIEN). DE BMP Tracking and Reporting Tool is mapped to provide the data required to NEIEN and the CBP.

Double counting is avoided because DDA is the only agency that submits manure relocation data.

External Data

Data are provided from DDA. The data are reviewed for accuracy – correct reporting period and that all necessary fields for NEIEN have been included.

Historic Data Verification

Historic data were verified for this practice - see NEIEN Methodology for Historical Data Clean Up (Appendix H).  Data quality assurance and data entry were conducted as discussed in the above sections.

BMP Performance

Manure relocation BMPs are annual practices.  All data are entered annually and will supersede any previous records.

D2.1.5 Conservation Tillage and High-Residue Tillage

D2.1.5.1 Conservation Tillage and High-Residue Tillage Visual Assessment – Annual Transect Survey

This section incorporates two, high priority BMPs captured through the statistically valid state-wide transect survey including conservation tillage and high-residue minimum soil disturbance (HRMSD) tillage.

In October 2015, EPA’s statistical team reviewed Delaware’s approach for generating and verifying BMP data for both conservation tillage and cover crops. The team verified that Delaware’s survey will accurately estimate tillage and cover crop BMPs with the proposed 100% verification protocol and 100% statistical sampling.

In 2019 -2021 Transect Survey visual assessment was modified due to COVID-19 State of Emergency Restrictions.

2.1.5.1 Table - Conservation Tillage, High-Residue Tillage, and Cover Crops

| Jurisdictional Agriculture Verification Protocol Design Table: Visual Assessment BMPs - Single Year |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| High | Visual Assessment: Single Year | Non-cost Shared Programs: Conservation Tillage, High-Residue Tillage, and Cover Crops | Visual Assessment: Residue Cover and Cover Crop Establishment | Yearly survey along statistically valid transect route | Trained DDA Staff, County Conservation Districts, and UD Agronomists | Delaware's “Procedures for Cropland Roadside Transect Survey for Obtaining Tillage/Crop Residue Data” | Single Year | 10% Sub-sample of all observations along statistically valid transect route | Review all previously recorded data from current year’s survey | Single Year | 10% Compliance Checks by State Agency/NPS Database/Transect Survey Application |

D2.1.5.2 Conservation Tillage and High-Residue Tillage Data Verification

Method

Tillage practices are annual, non-regulatory and collected using a statistically valid driving transect survey method. In addition to tillage practices, this survey also incorporates the observation of annual cover crop practices across the state.  Practices collected from this state-wide transect survey are reported at the state-wide level. Conservation Tillage and High Residue Minimum Soil Disturbance (HRMSD) Tillage are reported as a percent implemented throughout the entire state. Cover crops are collected as an observation percentage, which later get converted to an acreage based on available cropland. The collection process of this survey is part of a statistically valid transect that targets agricultural areas. The methodology for this survey was adopted by Delaware with help from the Chesapeake Bay Program from the Conservation Technology Information Center (CTIC), the original suppliers of Conservation Tillage data for the Chesapeake Bay States.

This cropland driving transect survey procedure provides a high degree of confidence in the data summaries. Users can have 90% or more confidence in the accuracy of the results. The driving route is required to be at least 110 miles long in each of Delaware’s three counties. The routes do not double-back along the same road more than once. The survey is conducted after the majority of the main crops have been planted, but before the crop canopy closes or the first row cultivation takes place. To obtain a statistically reliable data set, approximately 460 cropland sites are observed along the driving route, in each county. The survey team stops and checks field conditions at particular intervals to ensure correct estimates are being made for different crop, tillage, and residue conditions. The team re-calibrates their visual estimates when entering a region of the county with different soil surface conditions due to changes in moisture, organic matter levels, stoniness, or crops grown. In addition to the original survey team, a Quality Assurance and Quality Control team retraces the original routes after the initial survey is conducted to ensure the data captured is consistent. The QA/QC team consists of members that did not participate in initial survey, but from the same organizations. Using the same GPS coordinates as marked in the initial survey run-through, the team checks and confirms or rejects the initial observations on at least 10% of the observations. Members on the QA/QC team have access to the original observations and are able to compare them with their own judgments. The QA/QC team begins immediately after the initial observations are made. The team is able to verify a random sample of the initial observations; at most, two days after the initial observations are made. This ensures that the conditions originally observed are as close as possible to what was viewed in the QA/QC runs. In addition to the immediacy of the quality assurance and quality control review, the lead observer ground truths and interviews the land manager of several of the fields with their permission. The lead observer may utilize the bead-and-line residue estimation method in several cases to verify that correct observations are recorded.

Verifiers

The agricultural partners associated with this survey are invested and willing to partake in the survey on an annual basis. These associated parties include the Conservation Districts, University of Delaware Cooperative Extension, NRCS, FSA, and Delaware Dept. of Agriculture. Two teams are utilized to verify the presence/absence of residue and make observations – the Observation Team and Quality Assurance Team.

The observation team consists of two observers, a driver, navigator, and recorder.  Everyone conducting the survey is trained prior to making observations and as needed for recalibration.  This survey targets data collection shortly after producers have planted their main crops in the spring, but before canopy closure on these fields has been reached. This allows for “windshield observations” from the survey vehicles. An initial driving transect is conducted by the observation team and followed by the QA/QC team.  The QA/QC teams are able to retrace the original driving route for verification purposes almost immediately after initial observations are made, in most cases approximately two days later. This ensures that the conditions observed by the QA/QC team are as close as possible to those conditions viewed by the initial observation team. The observation teams conduct a second survey run mid-summer to observe double-cropping systems, in order to capture crops planted after the early spring crop. A third pilot survey is conducted in early winter in order to capture winter-planted cover crops.

The data collectors and verifiers have worked closely throughout the development of the data collection application and have been properly trained. Training for the data collection and verification was part of the initial training held in conjunction with University of Maryland’s Chesapeake Bay Representative Mark Dubin to practice residue estimation techniques, calibration of the observer’s eye for estimation, as well as data entry and examination. For subsequent survey years, trainings will be held prior to the actual survey to introduce new survey members and serve as a refresher for past members.

Documentation of Verification

The written guidance and documentation on the data collection and verification systems is found in Delaware’s “Procedures for Cropland Roadside Transect Survey for Obtaining Tillage/Crop Residue Data” (Appendix J), which was approved by the Chesapeake Bay Program Workgroup in December 2014.

Data are uploaded real-time into an iOS supported device utilizing Esri’s ArcCollector software. This allows for a more streamlined and reliable collection process, utilizing a tablet device rather than previous methods of paper data sheets. The driving route is preloaded onto the application for each county and by using the GPS feature; the team can track their driving progress throughout the day and follow the predetermined path. As the team arrives at their observation locations, a list of selectable fields appear for the data recorder to enter exactly what the observer notates. Data is automatically backed up through a cellular network to minimize chance of data loss. The addition of GPS technology ensures that the teams can return to the exact observation point, whether that is for QA/QC verification or for subsequent survey years. Keeping the data in this digitized form also allows for the randomized selection of the 10% QA/QC checks.  Data are maintained on private servers within Delaware’s Department of Technology and Information.  Information collected within the ArcCollector software is tied to a locked and secure ESRI account.

End of contract/project lifespan

Tillage and Cover Crop BMPs are annual practices and thus have a lifespan of one year. Therefore, reported practices are implemented and credited for that submission period only.

D2.1.5.2 Conservation Tillage and High-Residue Tillage Data Validation

Quality Assurance

All (100%) tillage and cover crop practices are surveyed annually by the observation team as described above.  Additionally, a 10% sub-sample is made by the Quality Assurance and Quality Control Team also described above. Data entry has proven to be a very simple and streamlined process. Predetermined locations have been identified and are able to be selected from a drop-down menu within the ArcCollector software to minimize the chance of errors. There have been no issues associated with the complicatedness of data entry.  Updates to the system are easily completed within the Esri Arc Suite of tools.

Upon survey completion, data is downloaded by a GIS Specialist housed within the Nonpoint Source Program of DNREC.  Raw data are reviewed and categorized based on Chesapeake Bay Program definitions for conservation tillage and high residue minimum soil disturbance.  Data are summarized and entered into the agriculture Excel template (Appendix I) for upload into the DE BMP Tracking and Reporting Tool.

Acres of cover crops based on the CTIC survey are reported utilizing the methodology that was approved by the Agricultural Workgroup at the September 2015 meeting. The cover crop observation percentages that were made during the survey are categorized by species, planting time, and planting method based on the NEIEN appendix for approved cover crop BMPs. The observation percentages for each of the cover crop categories are then applied the 2012 NASS county-wide harvested cropland acreages, yielding estimated acreages for traditional cover crops based on the survey. The acreages for each cover crop category reported by the county conservation districts are then subtracted from the matching cover crop category calculated from the CTIC-based survey. The acres left are submitted through NEIEN at the county-wide level, where model simulation calculates acreages within the watershed. NRCS acres of cover crops are not reported in order to prevent double counting of cover crops.

To confirm that the correct information has been copied into the templates from the raw data, the total area (e.g., acres) or numbers for each BMP type are summed and compared to the original dataset to be sure there were no errors in translating the data. Data are also compared to previous years’ submissions to see if numbers are consistent (i.e., no extreme increases or decreases in acreage or count). The methodology for entering data for each BMP type is recorded every year so that the same methodology is consistently used year to year.

Data Entry

Conservation tillage is not reported by any other agency. Therefore, double counting is not an issue.  Conservation tillage practices were previously reported by CTIC for historical data submissions; however, data from CTIC are no longer used. Data collected from this survey serves as a replacement for the CTIC dataset. At the conclusion of the survey, the percentage of observations for each residue BMP type (High Residue Tillage and Conservation Tillage including High Residue Tillage) are reported county wide to NEIEN. These percentages are then simulated by the model for acreage calculation. The cover crop acreages are compared to the cover crop submissions by NRCS and FSA for accuracy. These acres submitted from the survey replace the acres that have historically been submitted by NRCS and FSA to prevent double counting.

Data are entered into the agriculture Excel template (Appendix I) for upload into the NPS BMP Database. The agriculture Excel template is provided to NPS to input into the DE BMP Tracking and Reporting Tool using the NEIEN input template with the correct NEIEN BMP names (see QAPP Section B10 for additional details on the parties involved in data submission to NEIEN). DE BMP Tracking and Reporting Tool is mapped to provide the data required to NEIEN and the CBP.

The lifespan or credit duration of each BMP is also entered in the template based on the CreditDurations05222015.xlxs spreadsheet provided by CBP. The code in the DE BMP Tracking and Reporting Tool has been modified so that the lifespan/credit duration is added to the implementation date of a particular BMP to calculate the Lifespan End Date. The BMP will be tagged as “retired once the Lifespan End Date has passed.

External Data

Historic data was submitted to the CBPO by CTIC. Since 2014, all data for tillage practices are provided by DNREC NPS. The data are checked to be sure that they have been provided for the correct time period and that all necessary fields for NEIEN have been included.

Historic Data Verification

Historic data were provided by CTIC. Data quality assurance and data entry were conducted the same way as in the past (as discussed in the above sections).

BMP Performance

Tillage and cover crop practices are annual practices. Performance is only noted during the transect survey.  All tillage and cover crop data are entered annually and will supersede any previous records.

D2.1.6 Animal Waste Management Systems and Mortality Composting

D2.1.6.1 Animal Waste Management Systems and Mortality Composting -Visual Assessment – Cumulative/Structure

These two (2) BMPs – Animal Waste Management Systems and Mortality Composting were grouped together because they follow similar verification and validation protocols. Table B10.3.1 in Section B.10 of this QAPP provides definitions for each of these BMP types. Details regarding verification and validation procedures for these two practices are contained in Table D2-1 and summarized in the following sections.

| Table B-3. Delaware Agriculture Protocol Design Table: Visual Assessment BMPs – Cumulative/Strutures |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| High | Visual Assessment Multi-Year: Cost Shared | Animal Waste Management Systems | Field Visit/Visual Inspection | All Cost-Shared Animal Waste Management Systems | Trained NRCS and Conservation District Staff | Inspection forms and Database | 1-3 Years Following Implementation | All Cost-Shared Animal Waste Management Systems | Contact Landowner if out of compliance; Possible removal from program | 10 Years in Kent County, 15 Years in Sussex County | Maintain database for current and expired structures |
| High | Visual Assessment Multi-Year: Permit and Regulation | Animal Waste Management Systems | Field Visit/Visual Inspection | All Permitted Structures Under CAFO at Application | Trained DDA Staff | Inspection Report | 3-5+ Years Following Implementation | All Permitted Structures Under CAFO | Contact Landowner if out of Compliance | 5 Years (Duration of CAFO Permit) | Maintain Database for CAFO Permits and Inspections |
| Low | Visual Assessment Multi-Year: Cost Shared | Mortality Composting | Field Visit/Visual Inspection | All Cost-Shared Mortality Composters | Trained NRCS and Conservation District Staff | NRCS Standard | 1-3 Years Following Implementation | All Cost-Shared Mortality Composters | Contact Landowner if out of Compliance | 15 Years | Conservation Desktop |
| Low | Visual Assessment Multi-Year: Permit and Regulation | Mortality Composting | Field Visit/Visual Inspection | All Permitted Structures Under CAFO at Application | Trained DDA Staff | Inspection Report | 3-5+ Years Following Implementation | Percentage, Subsample, Targeted | Contact Landowner if out of Compliance | 5 Years (Duration of CAFO Permit) | Maintain Database for CAFO Permits and Inspections |

2.1.6.1 Table - Animal Waste Management Systems and Mortality Composting

D2.1.6.2 Animal Waste Management Systems and Mortality Composting -Data Verification

Method

The BMPs featured in this section meet NRCS standards, state standards, and Chesapeake Bay Program definitions. Practices that will be captured through this evaluation include: livestock waste management systems, animal waste management systems, and poultry waste management systems, and mortality composters.  This section does not include resource improvement practices. One hundred percent of all practices are inspected at initial implementation.  Follow-up inspections (10%) will be randomly chosen, while ensuring no successive duplication, by the Quality Assurance Officer from practices entered into the NPS BMP database.  Additionally, inspections may be conducted by the implementing agency for contract requirements; those inspections will be reported to the Quality Assurance Officer. All practices (100%) will be inspected prior to end of lifespan.

One hundred percent (100%) of all animal waste management and mortality composting facilities are inspected during the lifespan of the contract or permit.  BMPs are collected by multiple agencies - USDA, NRCS, Conservation Districts, and DDA.  The BMPs meet NRCS, State, and Chesapeake Bay Program definitions for agricultural practices. All are inspected by visual on-site inspection during the time of BMP implementation as required by NRCS contracts and DNREC/DDA permits.  Funding for these BMPs are provided by USDA programs or state cost share funding.  Additional funding is available through CWA Section 319 Grant and Chesapeake Bay Implementation (Section 117) Grants funds.

Structural BMPs are inspected by a Conservation Planner.  The goal is to inspect all structural BMPs every year.  The BMP inspection form is used to collected BMP related information during the inspection including: date of inspection; tract number; owner name; farm name; watershed name; whether or not the farmer was contacted; BMP implementation date; whether the BMP is being used; contents of BMP (if applicable); compliance status; and cost-share program (funding source).

Data collection includes implementation date, project type, animal type and project size. Projects submitted by NRCS are reported at the county level.  An outline of practice data submissions can be found in section B10 of Delaware’s QAPP. Additionally, in 2010, an agreement (Basinwide 1619 Agreement) was reached to have federal agencies, such as the USDA’s NRCS, report practices directly to the USGS for CBP modeling rather than have jurisdictions report on their behalf.  All other submitted practices are reported at the site-level, with corresponding latitude-longitude coordinates. All BMPs currently reported are approved by CBP for inclusion in model application.

Verifiers

All practices (100%) are inspected by visual on-site inspection after implementation and once during the lifespan of the BMP or as required by the cost sharing agency. BMP data are collected for several years by the aforementioned agencies to determine if BMPs are functioning properly.  Verification efforts occur year round and NRCS verification timing will be at the organization’s discretion.  BMP inspectors are trained NRCS, District or DDA agency employees. Training is ongoing as all new personnel are trained in the collection of BMP data; however, there is no “certification requirement” for staff collecting BMP data. If any of the data collectors have questions regarding functionality, contact is usually made with USDA NRCS.

Documentation of Verification

BMPs are inspected and entered into databases by trained NRCS, District planners or DDA staff.  Each Agency has separate verification documentation:

- NRCS Conservation Desktop – NRCS cost shared practices

- DDA– animal waste management systems and composting facilities are captured in hard copy forms housed at DDA in NMP or CAFO files.  Additional QA is performed DDA staff during NMP inspections. During that time, DDA staff inspect for compliance of their enforceable regulations.

- Conservation Districts – New Castle, Kent, and Sussex document the existence of practices during conservation plan and nutrient management plan development.  Each district uses excel spreadsheets to record BMPs.  Sussex completes inspection forms and inputs the data into their own Compliance Database.

Data regarding the parcel location of each BMP, visual functionality, and whether or not the practice is meeting standards and specifications are recorded in written files as well as respective databases.

End of contract/project lifespan

DDA will continue on-farm assessments through the nutrient management program and will review waste and composting system functionality.  Additionally, the conservation districts will continue to work with farmers to include animal waste and composting facilities in their on-farm inspections.  This data will be submitted, as requested, to the DNREC Quality Assurance Officer for inclusion in Delaware’s reporting and tracking database.

D2.1.6.3 Animal Waste Management Systems and Mortality Composting -Data Validation

Quality Assurance

All (100%) facilities are inspected within the contract/permit lifespan by various agencies. Additionally, checks are made upon implementation and before contract end dates by the funding agency; hence, BMPs are verified for functionality.  Inspection records (pass/fail) will be provided to DNREC’s Nonpoint Source Program (NPS) for inclusion in Delaware’s existing NPS BMP Database and submission to the CBP through NEIEN.

The DE BMP Tracking and Reporting Tool is an online database that serves as a means of reporting and tracking BMPs in the state of Delaware. The use of this tool allows for a more streamlined approach for generating reports needed for water quality assessment and monitoring purposes. This database is used to submit data for inclusion in the Chesapeake Bay Watershed Model. The DNREC-DWS-NPS Program staff enter all practices and are the only ones permitted to review and edit data.

The data are entered into the agriculture Excel template (Appendix I) for upload into the NPS BMP Database. To confirm that the correct information has been copied into the templates from the raw data, the total area (e.g., acres) or numbers for each BMP type are summed and compared to the original dataset to be sure there were no errors in translating the data. Data are also compared to previous years’ submissions to see if numbers are consistent (i.e., no extreme increases or decreases in acreage or count). The methodology for entering data for each BMP type is recorded every year so that the same methodology is consistently used year to year.

The BMP progress data are submitted to CBP every year by DNREC.

Data Entry

Data are collected and entered into the respective cost sharing entity’s database by trained staff.   Data are provided to NPS to input into the DE BMP Tracking and Reporting Tool using the NEIEN input template with the correct NEIEN BMP names (see QAPP Section B10 for additional details on the parties involved in data submission to NEIEN). DE BMP Tracking and Reporting Tool is mapped to provide the data required to NEIEN and the CBP.

BMPs are aggregated by the funding agency at the County level.  DDA and Conservation District is summarized and NRCS data are subtracted by animal type for each BMP type.  Any remaining units are added to the current year’s progress submission.  These data are entered with the current year’s implementation date and re-set with new lifespan.  Double counting is avoided because NRCS provides the active contracted BMPs and the Districts and DDA submit all on-the-ground BMPs.

External Data

Data are provided from external government agencies as mentioned above.  The data are checked to be sure that they have been provided for the correct time period and that all necessary fields for NEIEN have been included.

Historic Data Verification

Historic data were verified for this practice - see NEIEN Methodology for Historical Data Clean Up (Appendix H).  Data quality assurance and data entry were conducted as discussed in the above sections.

BMP Performance

During visual field assessments, the BMPs are inspected for compliance or failure by the inspecting agency. If a BMP is not performing up to its standards and specifications, a maintenance inspection report or letter is provided to the landowner. Agency staff work with landowners to bring the BMP back into compliance or the landowner must pay back the funds used to implement the BMP. A BMP is retired from the system if it is not brought into compliance.

D2.1.7 Grass Buffers, Land Retirement, and Water Control Structures.

D2.1.7.1 Grass Buffers, Land Retirement, and Water Control Structures. Assessment – Cumulative

These three (3) BMPs –Grass Buffers, Land Retirement, and Water Control Structures were grouped together because they all follow similar verification and validation protocols. Table B10.3.1 in Section B.10 of this QAPP provides definitions for each of these BMP types. Details regarding verification and validation procedures for these three practices are contained in Table D2-1 and summarized in the following sections.

2.1.7.1 Table - Grass Buffers, Land Retirement, and Water Control Structures

| Table B-3. Delaware Agriculture Protocol Design Table: Visual Assessment BMPs – Cumulative |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| Low | Visual Assessment Multi-Year: Cost Shared | Water Control Structures | Field Visit/Aerial/Visual Inspection | All Cost-Shared Water Control Structures | Trained NRCS and Conservation District Staff | NRCS Standard, DNREC Restoration Database | 1-5+ Years Following Implementation | All Cost Shared Water Control Structures | Contact Landowner if out of Compliance | 5 years | Districts Maintain Spreadsheets; DE BMP Tracking and Reporting Tool |
| Moderate | Visual Assessment: Annual | Grass Buffers | Visual Assessment | Annual | Trained District Conservation Planners | Kent Conservation District or DNREC DFW BMP Compliance Inspection Forms | Multi- Year | 100% | Verbal Warning, letter of compliance, return of cost share funds | 10 years | 100% Tracked within District BMP database; DNREC Restoration Database; DE BMP Tracking and Reporting Tool |
| Low | Visual Assessment | Land Retirement | Visual and Aerial Inspection | Annually – FSA 3-5 years - DNREC | Trained DNREC Staff | GIS shapefiles, DNREC Restoration Database | Targeted | 100% every 5 years | Verbal Warning, letter of compliance, return of cost share funds | 3 years | 100% Tracked within Kent Conservation District BMP database; DNREC Restoration Database; DE BMP Tracking and Reporting Tool |

D2.1.7.2 Grass Buffers, Land Retirement, and Water Control Structures. Data Verification

Method

The BMPs featured in this section meet NRCS standards, state standards, and Chesapeake Bay Program definitions. Practices that will be captured through this evaluation include: Water Control Structures, Grass Buffers, and Land Retirement. One hundred percent of all practices are inspected at initial implementation.  Follow-up inspections (10%) will be randomly chosen, while ensuring no successive duplication by the Quality Assurance Officer from practices entered into the NPS BMP database.  Additionally, inspections may be conducted by the implementing agency for contract requirements; those inspections will be reported to the Quality Assurance Officer. All practices (100%) will be inspected prior to end of lifespan.

One hundred percent (100%) of all the grouped BMPs are inspected during the lifespan of the practice.  BMPs are collected by multiple agencies - USDA, NRCS, Conservation Districts, and DNREC.  The BMPs meet NRCS, State, or Chesapeake Bay Program definitions for agricultural practices. All are inspected by visual on-site inspection during the time of BMP implementation, as required by NRCS contracts.  Additionally, DNREC inspects and reports practices funded through the agency.  Funding for these BMPs are provided by USDA programs or state cost share funding.  Additional funding is available through CWA Section 319 Grant and Chesapeake Bay Implementation (Section 117) Grants funds.

Structural BMPs, like water control structures, are inspected by a DNREC or Conservation Planners.  The BMP inspection form is used to collected BMP related information during the inspection including date of inspection; tract number; owner name; farm name; watershed name; BMP implementation date; compliance status; and cost-share program (funding source).  An outline of practice data submissions can be found in section B10 of Delaware’s QAPP. Additionally, in 2010, an agreement (Basinwide 1619 Agreement) was reached to have federal agencies, such as the USDA’s NRCS, report practices directly to the USGS for CBP modeling rather than have jurisdictions report on their behalf.  All other submitted practices are reported at the site-level, with corresponding latitude-longitude coordinates. All BMPs currently reported are approved by CBP for inclusion in model application.

Verifiers

All practices are inspected by visual on-site inspection during the lifespan of the BMP as required by the cost sharing agency. BMP data are collected for several years by the aforementioned agencies to determine if BMPs are functioning properly.  Verification efforts occur year-round and NRCS verification timing will be at the organization’s discretion.  BMP inspectors are trained NRCS, District or DNREC agency employees. Training is ongoing as all new personnel are trained in the collection of BMP data; however, there is no “certification requirement” for staff collecting BMP data. If any of the data collectors have questions regarding functionality, contact is usually made with USDA NRCS.

Documentation of Verification

BMPs are inspected and entered into databases by trained NRCS, District planners or DDA staff.  Each Agency has separate verification documentation:

- NRCS Conservation Desktop – NRCS cost shared practices

- DNREC – land retirement, water control structure, and grass buffer practices are entered into the Restoration Database.  Additional QA is performed by DNREC staff. During that time, DNREC staff inspect for functionality.

- Conservation Districts – New Castle, Kent, and Sussex document the existence of practices during conservation plan and nutrient management plan development.  The written guidance and documentation on the data collection and verification systems is found in the District Cost Share Compliance documents individually drafted by each of the three Districts.

Data regarding the parcel location of each BMP, visual functionality, and whether the practice is meeting standards and specifications are recorded in written files as well as respective databases by the funding agency.

End of contract/project lifespan

Once the practice contract expires with NRCS, the District will report and inspect.  DNREC reports their own practices and therefore are never double counted with NRCS practices.  The implementing agency will submit updated data to the DNREC Quality Assurance Officer for inclusion in Delaware’s reporting and tracking database.

D2.1.7.3 Grass Buffers, Land Retirement, and Water Control Structures. Data Validation

Quality Assurance

All (100%) facilities are inspected within the practice lifespan by various agencies. Additionally, checks are made upon implementation and before contract end dates by the funding agency; hence, BMPs are verified for functionality.  Inspection records (pass/fail) will be provided to DNREC’s Nonpoint Source Program (NPS) for inclusion in Delaware’s existing NPS BMP Database and submission to the CBP through NEIEN.

The DE BMP Tracking and Reporting Tool is an online database that serves as a means of reporting and tracking BMPs in the state of Delaware. The use of this tool allows for a more streamlined approach for generating reports needed for water quality assessment and monitoring purposes. This database is used to submit data for inclusion in the Chesapeake Bay Watershed Model. Individual organizations are responsible for entering their practices with their provided login information and are only permitted to review their own data.

The data are entered into the agriculture Excel template (Appendix I) for upload into the NPS BMP Database. To confirm that the correct information has been copied into the templates from the raw data, the total area (e.g., acres) or numbers for each BMP type are summed and compared to the original dataset to be sure there were no errors in translating the data. Data are also compared to previous years’ submissions to see if numbers are consistent (i.e., no extreme increases or decreases in acreage or count). The methodology for entering data for each BMP type is recorded every year so that the same methodology is consistently used year to year.

The BMP progress data are submitted to CBP every year by DNREC.

Data Entry

Data are collected and entered into the respective cost sharing entity’s database by trained staff.   Data are provided to NPS to input into the DE BMP Tracking and Reporting Tool using the NEIEN input template with the correct NEIEN BMP names (see QAPP Section B10 for additional details on the parties involved in data submission to NEIEN). DE BMP Tracking and Reporting Tool is mapped to provide the data required to NEIEN and the CBP.

External Data

Data are provided from external government agencies as mentioned above.  Double counting is avoided since BMPs are reported by the funding agency.  The data are checked to be sure that they have been provided for the correct time period and that all necessary fields for NEIEN have been included.

Historic Data Verification

Historic data were verified for this practice - see NEIEN Methodology for Historical Data Clean Up (Appendix H).  DNREC conducted an extensive historical clean-up for water control structures see Appendix C.  Data quality assurance and data entry were conducted as discussed in the above sections.

BMP Performance

During visual field assessments, the BMPs are inspected for compliance or failure by the inspecting agency. If a BMP is not performing up to its standards and specifications, a maintenance inspection report or letter is provided to the landowner. Agency staff work with landowners to bring the BMP back into compliance or the landowner must pay back the funds used to implement the BMP. A BMP is retired from the system if it is not brought into compliance.

D2.1.8 Low Priority BMPs

Section D1 of the QAPP discusses that verification protocols for additional BMPs with lower anticipated contributions to the overall load reductions will be developed but at a slower pace, given the reduced reliance on these practices to Delaware’s reduction strategy. All of the lower priority agricultural BMPs are either (1) accounted for in NRCS inspections at 5% per year; (2) have numeric milestones established but have not been reported; or (3) are not approved BMPs.

The low priority BMPs that are accounted for in NRCS inspections at 5% per year, but not specifically listed in the Verification Program include:

- Pasture alternative watering

- Prescribed grazing

- Precision intensive rotational grazing

- Stream access control with fencing

- Barnyard runoff control

- Loafing lot management

Carbon sequestration has numeric milestones but is currently not reported. Cropland irrigation management and heavy use area pads are not approved BMPs, therefore, they are not included in the Verification program. If the aforementioned BMPs become high priority BMPs in the future, Delaware will develop a timeframe for verification. Delaware will develop a verification program for cropland irrigation management (currently using methodology listed in Appendix B) and heavy use area pads, once approved by CBP.

### D2.2 Forestry Sector Verification Protocol

This section of the verification protocol represents the BMP groupings for Forestry Source Sector. Table F-2 in Appendix F provides a sector-specific checklist of Delaware BMP verification protocol components and maps them to the relevant QAPP sections where they are documented.

D.2.2.1 Forest Buffers, Tree Planting, and Urban Tree Planting

D.2.2.1.1 Forest Buffers, Tree Planting, and Urban Tree Planting Visual Assessment

These three BMPs – forest buffers, tree planting, and urban tree planting - were grouped together because they all follow the same verification and validation protocol. Table B10.3.1 in Section B.10 of this QAPP provides definitions for each of these BMP types. Details regarding verification and validation procedures for these three practices are contained in Table 2.2.1.1 and summarized in the following sections.

2.2.1.1 Table - Forest Buffers, Tree Planting, and Urban Tree Planting

| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| High | Visual Assessment: Multi-Year | Cost-Shared Agricultural Forest Buffers | Visual Inspection | 100% post installation | NRCS staff | Inspection form and signed contract | Visual assessment – FSA | 5% annually or 100% by end of contract lifespan | Bring back into compliance or loss of money – must pay back funds. | Contract Lifespan 15 yrs | Conservation Desktop, 319 Program Database; if fail inspection documented and filed at FSA. If reenrolled information is updated in NPS BMP database. |
| High | Visual Assessment | CREP RFB | Field Inspection | 100% post installation (1 year after) | Delaware CREP Coordinator (Partnership between USDA FSA and DNREC 319 Program) | 319 Program Data Tracking | Mid - Contract | 100% Initial by DDA, 100% post installation, 100% Mid-Contract, 100% sampled by end of contract | Compliance Letter | 10-15 yrs. | 100% Tracked by FSA and 319 Program database |
| High | Visual Assessment: Multi-Year | Cost-Shared Agricultural Tree Planting – NRCS EQIP | Visual Inspection | 100% post installation | NRCS staff | Inspection form and signed contract | Follow-up inspection conducted via aerial imagery thru CBPO. No inspection required due to Land Use change. | GIS analysis; 100% | Bring back into compliance or loss of money – must pay back funds. | 10 yrs – Land Use change in Model | Toolkit. If reenrolled information is updated in NPS BMP database. |
| High | Visual Assessment: Multi-Year | Cost-Shared Agricultural Tree Planting – Delaware Forest Service (DFS) | Visual Inspection | 100% during installation and post installation (1 year after) | DDA foresters | Forestry database entered using cost share forms (DDA funding) | Survival Checks conducted one year after implementa-tion | 100% | Work with landowners to bring into compliance | 10 yrs – Land Use change in model | DDA Forestry Database – files sent to NPS BMP database. |
| High | Visual Assessment Multi year | Cost Share Ag Tree Planting – DFW | Visual Inspection | 100% during instllation and post installation (1 year after) | DFW biologists | Spreadsheet form – LIP inspections | Annual Inspection for each project in contract | 100% Initial; 100% Mid-Contract; 100% by end of contract lifespan | Work with landowner to resolve | 10 yrs | LIP database to NPS BMP database. |
| High | Visual Assessment Multi Year | Ag Tree Planting –public lands | Visual Inspection | 100% during installation and post installation | DFW biologists | DFW database | Survival Checks conducted one year after implementa-tion | 100% | Work to establish per planting specification | 10 yrs – Land Use change in model | DFW Database – files sent to NPS BMP database. |

D.2.2.1.2 Forest Buffers, Tree Planting, and Urban Tree Planting Data Verification

Method

One hundred percent of all forest buffers, tree planting and urban tree plantings are inspected during the lifespan of the contract (ag tree planting and buffers) or project (urban). BMPs are collected by multiple agencies:

  - Ag forest buffers – USDA standard - implemented by USDA and DNREC

  - Ag tree planting – USDA standard – implemented by USDA, DNREC and DDA

  - Urban tree planting – CBP definition – 300 trees/acre – implemented by DDA

The listed BMPs meet NRCS, State, and Chesapeake Bay Program definitions for forestry practices. All are inspected by visual on-site inspection during the time of BMP implementation as required by NRCS and CREP contracts, and DNREC/DDA projects. Funding for these BMPs are provided by USDA programs or state cost share funding. Additional funding is available through CWA Section 319 Grant, US Forest Service, and Chesapeake Bay Implementation (Section 117) Grants funds.

In Delaware, DDA DFS installs most large-scale buffer and tree planting projects. DFS inspects 100% of plantings post installation (following the Forestry Workgroup’s verification guidance of inspection at implementation). Approximately 1 year after planting, DFS inspects the projects again (100%) (following the Forestry Workgroup’s inspection recommendation for 1-4 years). One hundred percent of the buffer and tree planting projects are spot checked on average every 7.5 years by the cost sharing agency (NRCS, DFW, DFS, etc.) (following the Forestry Workgroup’s inspection recommendation of 5-10 years). During this time, projects are assessed for water quality impacts. NRCS verification timing will be at the organization’s discretion. A final inspection is completed at contract or lifespan expiration (approximately 10-15 years) (following the Forestry Workgroup’s inspection recommendation of 5-10 years). Once project lifespan or contract expires, Delaware will use high resolution imagery and Light Detection and Ranging (LiDAR) data to determine the existence of riparian forest buffer and tree planting practices.

Data collection includes implementation date, project type, and project size. Projects submitted by NRCS are reported at the county level. An outline of practice data submissions can be found in section B10 of Delaware’s QAPP. Additionally, in 2010, an agreement (Basinwide 1619 Agreement) was reached to have federal agencies, such as the USDA’s NRCS and FSA, report practices directly to the USGS for CBP modeling rather than have jurisdictions report on their behalf. All other submitted practices are reported at the site-level, with corresponding latitude-longitude coordinates. All BMPs currently reported are approved by CBP for inclusion in model application.

Verifiers

All practices are inspected by visual on-site inspection during the lifespan of the BMP as required by the cost sharing agency. BMP data are collected for several years by the aforementioned agencies to determine if BMPs are functioning properly. The time and frequency of sampling has a large influence on quality of information gained. While forestry practices are present year-round, most of the verification will occur during the growing season.

BMP inspectors are trained NRCS, DDA, or DNREC agency employees. Training is ongoing as all new personnel are trained in the collection of BMP data; however, there is no “certification requirement” for staff collecting BMP data. If any of the data collectors have questions regarding functionality, contact is usually made with USDA NRCS.

DFS Urban foresters have made great strides to capture urban forestry grant funded tree planting projects and verify historical practices. For urban tree planting, DFS urban foresters have reviewed historical grant files and visited tree projects and evaluated the location of trees.  For each tree planting project, site observations are made and geolocated – presence and absence of trees are noted.

Documentation of Verification

BMPs are inspected and entered into databases by trained NRCS, DFS Foresters, or DNREC Scientists or Biologists. Each Agency has separate databases for their reportable BMPs:

- DDA DFS Planting Database – agricultural tree planting data implemented by DDA DFS is geolocated and stored in GIS shapefiles. DFS foresters are responsible for entering their own planting data. Additional QA is performed by a senior forester and GIS manager on data entries. Planting projects performed on State Forest owned lands are also entered into the Planting database.

- DDA DFS Urban Database – urban tree planting data implemented by DDA DFS urban foresters are geolocated and stored in an Urban Database. The Urban forestry coordinator enters and stores information in the database provided by the urban foresters.

- DNREC Landowner Incentive Program (LIP) Database – DNREC DFW biologists maintain spreadsheets for those landowners enrolled in LIP. The LIP manager maintains the database and submits data for CB progress.

- USDA Conservation Desktop – NRCS cost shared practices

- 319 USDA FSA CREP Program – riparian forest buffer data are maintained on private servers within DNREC in excel and GIS databases at the Delaware 319 NPS Program Office. The CREP coordinator maintains data and submits for CB progress.

Data regarding the location of each BMP, visual functionality, and whether or not the practice is meeting standards and specifications are recorded in written files as well as the respective databases.  Due to the large number of contracts held by cost sharing agencies like USDA NRCS and FSA, Delaware is confident that 90% of all riparian forest buffers are avoiding water quality impacts because landowners comply with contract requirements to receive monetary payment.

End of contract/project lifespan

All CREP contracts will be fulfilled to contract end date; federal funding is secure and payments will continue through the active period of the contract up until re-enrollment. After contract end date, if State funding is secured, contracts will be re-enrolled for another term; however, if State funding is not secure, the acreage will not be re-enrolled. The CREP Coordinator will inspect riparian forested buffers if the landowners will continue to allow the BMP to function.  If contracts are renewed or inspected without incentive payments, the DNREC CREP coordinator will submit updated data to the DNREC Quality Assurance Officer for inclusion in Delaware’s reporting and tracking database.

All other forestry BMPs will be reviewed after contract end date. All implementers will work together to inspect tree planting projects after contract end date.

In order to prioritize forestry needs, it is essential to have a good understanding of the current resource.  Once project lifespan or contract expires, Delaware will use high resolution imagery and Light Detection and Ranging (LiDAR) data to determine the existence of riparian forest buffer and tree planting practices.  The Delaware Forest Service has quantified the spatial extent of forests and urban tree canopy coverage in the State. Periodically, DNREC and DFS will use these original datasets developed by DFS to determine if a net gain occurs over time and practices exist upon expiration. Additionally, the U&CF with DNREC will utilize Davey i-Tree software to set tree canopy goals and facilitate proper urban forest management strategies.  Furthermore, the CBP Forestry Workgroup will release new High-Resolution Land Cover dataset for the Phase 6 Watershed Model.  This imagery will allow jurisdictions to further enhance verification programs.

D.2.2.1.3 Forest Buffers, Tree Planting, and Urban Tree Planting Data Validation

Quality Assurance

All (100%) forestry practices are inspected within the contract/project lifespans. Additionally, checks are made upon implementation and before contract end dates – hence, BMPs are verified for functionality. Inspection records (pass/fail) will be provided to DNREC’s Nonpoint Source Program (NPS) for inclusion in Delaware’s existing NPS BMP Database and submission to the CBP through NEIEN.

The DE BMP Tracking and Reporting Tool is an online database that serves as a means of reporting and tracking BMPs in the state of Delaware. The use of this tool allows for a more streamlined approach for generating reports needed for water quality assessment and monitoring purposes. This database is used to submit data for inclusion in the Chesapeake Bay Watershed Model. Individual organizations are responsible for entering their practices with their provided login information and are only be permitted to review their own data.

The data are entered into the agriculture Excel template (Appendix I) for upload into the NPS BMP Database. To confirm that the correct information has been copied into the templates from the raw data, the total area (e.g., acres) or numbers for each BMP type are summed and compared to the original dataset to be sure there were no errors in translating the data. Data are also compared to previous years’ submissions to see if numbers are consistent (i.e., no extreme increases or decreases in acreage or count). The methodology for entering data for each BMP type is recorded every year so that the same methodology is consistently used year to year. See QAPP Section B10.3 for specific urban BMP submittal methodology.

The BMP progress data are submitted to CBP every year by DNREC.

Data Entry

Data are collected and entered into the respective cost sharing entity’s database by trained staff.   Forestry data are provided to NPS to input into the DE BMP Tracking and Reporting Tool using the NEIEN input template with the correct NEIEN BMP names (see QAPP Section B10 for additional details on the parties involved in data submission to NEIEN). DE BMP Tracking and Reporting Tool is mapped to provide the data required to NEIEN and the CBP.

The lifespan or credit duration of each BMP is also entered in the template based on the CreditDurations05222015.xlxs spreadsheet provided by CBP. The code in the DE BMP Tracking and Reporting Tool has been modified so that the lifespan/credit duration is added to the implementation date of a particular BMP to calculate the Lifespan End Date. Once the Lifespan End Date has been passed, that BMP will be tagged as “retired” unless that BMP been inspected, or maintenance has been performed. If an inspection or maintenance has occurred and the BMP is functioning properly, the BMP is credited with a new lifespan.

Double counting is unlikely to occur for these forestry practices because they are being tracked by the funding agency.

The DNREC BMP Tracking and Reporting Application (https://bmptracker.dnrecapps.net/)  was revised in 2019 as the means of collecting and managing the nonpoint source sector best management practices (BMPs) implemented in the State of Delaware. It replaced the DE NPS BMP Database on December 1, 2019 as the central repository of BMP data from multiple agencies and departments allows a streamlined approach to generate reports needed for BMP tracking, BMP reporting, and water quality assessments. DNREC personnel use the BMP Tool to submit data to the Chesapeake Bay Program as required for the Bay TMDL. The BMP tool also facilitates analyses like evaluating BMP trends and targeting.

This Excel spreadsheet contains a template for reporting and tracking best management practices (BMPs). The BMP template helps agencies in the identification and regular reporting of best management practices to the Chesapeake Bay Program. Reported practices receive credit towards meeting the goals of the Chesapeake Bay nutrient and sediment total maximum daily loads (TMDLs).

External Data

Data are provided from external government agencies as mentioned above. The data are checked to be sure that they have been provided for the correct time period and that all necessary fields for NEIEN have been included.

Historic Data Verification

Historic data were provided by some of the cost sharing agencies.  Data quality assurance and data entry were conducted the same way as in the past (as discussed in the above sections).   For additional information on historic data collection, please see Appendix H.

BMP Performance

During the visual field assessment, the BMPs are inspected for compliance or failure by implementing agency. If a BMP is not performing up to its standards and specifications, a maintenance inspection report or letter is provided to the landowner. Agency staff work with landowners to bring the BMP back into compliance or the landowner must pay back the funds used to implement the BMP.

D2.2.2 Forest Harvesting Practices

D2.2.2.1 Forest Harvesting Practices Assessment

Through the Erosion and Sediment Program, the Delaware Forest Service ensures forest management activities follow Best Management Practices (BMPs), and thus comply with the Forest Harvesting Practices via the Forest Practices Erosion and Sedimentation Law (Title 3, Chapter 10, Subchapter VI). Loggers or operators submit a permit prior to commencing forest management activities, and DFS staff reviews the site during the operation. Forest harvesting practices are temporary, while the other BMPs have a much longer lifespan. Details regarding verification and validation procedures for erosion and sediment control practices are contained in Table 2.2.2.1 and summarized in the following sections.

2.2.2.1 Table – Forest Harvesting Practices

| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| Low – CB WIP High - Forestry | Visual Assessment Regulatory | Forest Harvesting Practices | Visual Inspection | 100% inspected initially; upon receipt of E&S Permit from landowner/ logger | DDA FS Staff | Timber Harvest Inspection Reports | Possibly multiple times during the permit cycle. At least 1 inspection-done after job is completed. | 100% | Work with loggers to address concerns – verbal warnings & remediation plan. If Severe – have regulatory action (fines). | 3 yrs. | Database – GIS polygons provided at parcel level to NPS BMP db. |

D2.2.2.2 Forest Harvesting Practices Data Verification

Method

Forest harvesting practices are regulatory and 100% of all practices are inspected during the duration of an Erosion and Sediment Control permit. The forest harvesting practices comply with the Forest Practices Erosion and Sedimentation Law (Title 3, Chapter 10, Subchapter VI). All BMPs are inspected by visual on-site inspection during the time of permit application, during silviculture operations. Operators provide written notification to DFS at least five (5) business days prior to initiation of covered silviculture operations greater than one acre.

Site operators are responsible for following BMPs as indicated on the Erosion and Sediment Law Notification Form and Permit until a forester has made a final inspection of the site and issued a final inspection report. Forest Harvesting BMPs are collected by the Delaware Forest Service.

All practices are reported at the site-level, with implementation date (date of permit), and corresponding latitude-longitude coordinates. All BMPs currently reported are approved by CBP for inclusion in model application.

Verifiers

DFS Foresters visit silviculture operation tracts, perform BMP inspections, and record total acreage of forest harvesting BMPs. During the course of the inspection, a forester will determine the status of the effectiveness of BMPs in protecting water quality and record this determination on the inspection forms. If a potential water quality problem exists, as defined by the law, the forester will document the problem on the BMP inspection forms.

Documentation of Verification

BMPs are inspected by a DFS forester and documented on the BMP inspection form. Water Quality (WQ) classifications are used to determine severity of problems:

- No WQ problem

- Potential WQ problem – a typical problem that would cause excessive sedimentation and erosion during a normal rainfall.

- Severe WQ problem – any silvicultural activity which is causing sediment deposition or will immediately create serious sediment deposition in a rainfall event.

If no WQ problem exists, the landowner and operator are notified on site, if possible, and in writing within five (5) business days following the inspection. However, if a potential WQ problem exists on an initial field visit, the Forester will note the problem on the BMP inspection form, including written directions to alleviate the potential problem, to the operator and landowner, and a time limitation of up to five (5) business days to correct the problem. The Forester will notify his/her immediate supervisor of the existence of a potential WQ problem. When the time limitation specified in the recommendation for a potential WQ problem has elapsed, the Forester will return for a second visit. If the problem persists with no extenuating circumstances such as bad weather, all operations will be halted until specified corrective actions have been made to the satisfaction of the Forester.

If a severe WQ problem exists, such as skidding logs across a stream or ditch with no bridge, the Forester will cause all operations to cease immediately, issue a written warning containing instructions how to immediately correct the problem.  If WQ problems are not resolved, the Department will take actions on the operator including no further issuance of permits or a fine.

Forest harvesting practices are entered into the DFS GIS database by DFS Foresters. Data regarding the location of each BMP, visual functionality, and whether or not the practice is meeting standards and specifications are recorded in written files.

End of contract/project lifespan

Harvest permits are annual practices and thus have a lifespan of one year.

D2.2.2.3 Forest Harvesting Practices Data Validation

Quality Assurance

All (100%) forestry practices are inspected within the contract/project lifespans. Additional, checks are made upon implementation and before contract end dates – hence, BMPs are verified for functionality. Inspection records (pass/fail) will be provided to DNREC-DWS-NPS for inclusion in Delaware’s existing NPS BMP Database and submission to the CBP through NEIEN.

The DE BMP Tracking and Reporting Tool is an online database that serves as a means of reporting and tracking BMPs in the state of Delaware. The use of this tool allows for a more streamlined approach for generating reports needed for water quality assessment and monitoring purposes. This database is used to submit data for inclusion in the Chesapeake Bay Watershed Model. Individual organizations are responsible for entering their practices with their provided login information and are only be permitted to review their own data.

The data are entered into the agriculture Excel template for upload into the NPS BMP Database. To confirm that the correct information has been copied into the templates from the raw data, the total area (e.g., acres) or numbers for each BMP type are summed and compared to the original dataset to be sure there were no errors in translating the data. Data are also compared to previous years’ submissions to see if numbers are consistent (i.e., no extreme increases or decreases in acreage or count). The methodology for entering data for each BMP type is recorded every year so that the same methodology is consistently used year to year. See QAPP Section B10.3 for specific urban BMP submittal methodology.

The BMP progress data are submitted to CBP every year by DNREC.

Data Entry

Data are collected and entered into the respective cost sharing entity’s database by trained staff.   Forestry data are provided to NPS to input into the DE BMP Tracking and Reporting Tool using the NEIEN input template with the correct NEIEN BMP names (see QAPP Section B10 for additional details on the parties involved in data submission to NEIEN). DE BMP Tracking and Reporting Tool is mapped to provide the data required to NEIEN and the CBP.

The lifespan or credit duration of each BMP is also entered in the template based on the CreditDurations05222015.xlxs spreadsheet provided by CBP. The code in the DE BMP Tracking and Reporting Tool has been modified so that the lifespan/credit duration is added to the implementation date of a particular BMP to calculate the Lifespan End Date. Once the Lifespan End Date has been passed, that BMP will be tagged as “retired” unless that BMP been inspected, or maintenance has been performed. If an inspection or maintenance has occurred and the BMP is functioning properly, the BMP is credited with a new lifespan.

Double counting is unlikely to occur for these forestry practices because they are being provided by one agency (DDA) and there are no cost-share practices.

The DNREC BMP Tracking and Reporting Application (https://bmptracker.dnrecapps.net/)  was revised in 2019 as the means of collecting and managing the nonpoint source sector best management practices (BMPs) implemented in the State of Delaware. It replaced the DE NPS BMP Database on December 1, 2019 as the central repository of BMP data from multiple agencies and departments allows a streamlined approach to generate reports needed for BMP tracking, BMP reporting, and water quality assessments. DNREC personnel use the BMP Tool to submit data to the Chesapeake Bay Program as required for the Bay TMDL. The BMP tool also facilitates analyses like evaluating BMP trends and targeting.

This Excel spreadsheet contains a template for reporting and tracking best management practices (BMPs). The BMP template helps agencies in the identification and regular reporting of best management practices to the Chesapeake Bay Program. Reported practices receive credit towards meeting the goals of the Chesapeake Bay nutrient and sediment total maximum daily loads (TMDLs).

External Data

Data are provided from external government agencies as mentioned above. The data are checked to be sure that they have been provided for the correct time period and that all necessary fields for NEIEN have been included.

Historic Data Verification

Historic data were previously captured by a joint project with DFS and NPS – see Appendix D.   Data quality assurance and data entry were conducted as discussed in the above sections.  For additional information on historic data collection, please see Appendix H.

BMP Performance

During the visual field assessment, the BMPs are inspected for compliance or failure by implementing agency. If a BMP is not performing up to its standards and specifications, a maintenance inspection report or letter is provided to the landowner. Agency staff work with landowners to bring the BMP back into compliance or the landowner must pay back the funds used to implement the BMP.

### D2.3 Restoration Sector Verification Protocol

This section of the verification protocol represents the BMP groupings for stream and wetland restoration. Table F-3 in Appendix F provides a restoration-specific checklist of Delaware BMP verification protocol components and maps them to the relevant QAPP sections where they are documented.

D2.3.1 Stream Restoration

D2.3.1.1 Stream Restoration Assessment – Multi-Year

This part of the verification protocol incorporates all stream restorations that are implemented and accounted for within Delaware’s WIP, including non-urban stream restoration and urban stream restoration. Table B10.1.1 in Section B.10 of this QAPP provides definitions for each of these BMP types. Details regarding verification and validation procedures for these practices are contained in Table D2.3.1.1 and summarized in the following sections.

2.3.1.1 Table Stream Restoration Protocol Design Table: Visual Assessment BMPs – Multi

| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| High | Visual Assessment: Multi-Year | Cost-Shared Stream Restorations | Visual On-site | 100% of All Tracked and Reported Stream Restoration | Non-regulator Agency | Projects plans and specifications sealed by Delaware Professional Engineer | Inspect every year first 3 years, once every 5 years after established and/or after major storm events | 10% a year | Structures to be fixed if funding available. If cannot be achieved remove from database | 10 years | 10% compliance checks |

D2.3.1.2 Stream Restoration Data Verification

Method

The Stream Restoration Verification Protocol incorporates all stream restorations that are implemented and accounted for in Delaware’s Chesapeake Bay Watershed Implementation Plan (WIP). All stream restorations (100%) will be inspected every year for first three years of the project establishment. Following the first three years, projects will be inspected at least once every five years as well as after heavy storm events, which are defined as a 10-year storm recurrence interval. This will entail verifying at least 10 percent of reported practices every year.

The reported BMPs (non-urban stream restoration and urban stream restoration) meet NRCS, State, and Chesapeake Bay Program (CBP) definitions for stream restorations practices and have been approved by the CBP. Resource improvement practices are not reported. Reported BMPs, whether by NRCS or the State of Delaware, all have an implementation date, project type, and project size. Projects submitted by NRCS are reported at the county level. All other implemented practices are reported at the site-level with corresponding latitude-longitude coordinates.

The selected collection method is the Rapid Stream Restoration Monitoring Protocol methodology established by the Chesapeake Bay Field Office and U.S. Fish and Wildlife Service (June 2014).

This methodology is approved by the CBP workgroup as well as NRCS and the state agencies reporting stream restorations. Delaware has decided to adopt the use of the Rapid Stream Restoration Monitoring Protocol, including the use of the associated data recording sheets found in Appendix A of the USFWS document. A link to the Rapid Stream Restoration Monitoring Protocol is provided here:

http://www.fws.gov/chesapeakebay/Stream/StreamsPDF/Restoration%20Monitoring%20Protocol%20FINAL%20%206-30-14.pdf

The purpose of the protocol is to allow for the rapid assessment of these restoration projects and the determination of potential shortfalls in project design that may lead to failure. The following list identifies the main objectives of the protocol:

- Develop a function-based rapid and standardized method to evaluate the stability and functional success of a restored stream

- Establish a minimum standard necessary to evaluate the stability and functional success of a restored stream

- Promote consistent and reproducible results

- Identify situations that require additional monitoring

- Identify potential causes for impairment

- Identify potential corrective actions

This monitoring and verification protocol can be applied to almost any type of stream restoration, making it an ideal fit for stream restorations within the state. Observations will be made to evaluate the functional stability of stream restoration projects which focuses on vertical stability, lateral stability, riparian condition and instream structures, as well as vegetative stability. The Rapid Stream Restoration Monitoring Protocol report consists of eight main sections: A) design approach, B) bank-full determination, C) limits of investigation, D) rapid stream restoration monitoring form, E) evaluation attribute definitions, F) monitoring procedures, G) limited stream measurements, and H) monitoring recommendations.

The evaluator must be familiar with the various design approaches associated with stream restoration projects including analytical based, regenerative storm conveyance, sand berm seepage systems, Natural Channel Design, and valley/base flow approaches, all of which have unique criteria and standards.

During the verification process, the evaluator will examine bankfull field indicators, such as a significant slope break or floodplain feature, along the stream banks as a critical component to assess stream stability and function. The evaluation of the project will start and end at the points where the restoration has no visual influence on the stream, typically beginning upstream and working their way downstream.

Observations will be recorded on the Rapid Stream Restoration Monitoring Form found in the USFWS Rapid Stream Restoration Monitoring Protocol, Appendix A. This monitoring form is divided into six sections: 1) project information, 2) station identification, 3) problem description, 4) recommended actions, 5) measurements, and 6) evaluation attributes. An in-depth review of each of these sections in the monitoring form can be found in Section E of the USFWS Rapid Stream Restoration Monitoring Protocol.

The procedures for field monitoring are as follows. The evaluator will review all relevant materials prior to the site visit including assessment and design reports and/or plans, as well as the rapid stream restoration monitoring protocol and monitoring forms. These materials should also be brought into the field for reference, as well as for filling out the monitoring field form. In order to document observations, a measuring tape, survey rod, and camera should also be utilized. The evaluator will determine the start and end points of the evaluation, as well as determine the monitoring stations. These locations should be documented for subsequent inspections. All problems that are present will be documented and addressed separately.

If significant potential structural or functional failures are identified, the evaluator should conduct a more intensive stream survey. Measurements should be made to compare to the initial design criteria to determine if remediation should occur.

Verification for stream restoration practices will occur annually throughout the first 3 years of project establishment, specifically during the practice installation and following severe storm events. It will take several years to determine if a BMP is properly functioning. Monitoring continues throughout the determined monitoring period as established in the contract, which will be at least once every 5 years.

Verification Team

As outlined in the Rapid Stream Restoration Monitoring Protocol, the evaluator must be knowledgeable of fluvial geomorphic and watershed processes and be well trained in the design approach used for the stream restoration.

The evaluations will be conducted through DNREC’s Division of Watershed. The appropriate staff will be trained in person to ensure the verification protocol is being followed and the correct information is being collected and reported back to the responsible agency. There will be no certification requirement beyond the initial training for those collecting data. The verification collectors will call the project-sponsoring organization if questions arise about the specific project.

Documentation of Verification

Stream restoration data are requested on an annual or more frequent basis from numerous agencies that implement, track, and/or maintain stream restoration practice data. An outline of practice data submissions can be found in Section B10 of Delaware’s QAPP. Additionally, in 2010, an agreement (Basinwide 1619 Agreement) was reached to have federal agencies, such as the USDA’s NRCS and FSA, report practices directly to the USGS for CBP modeling rather than have jurisdictions report on their behalf. DNREC–DWS-NPS maintains a restoration database that captures restoration practices like stream restoration, tree plantings, forest and grass buffers are compiled from various projects throughout DNREC. More information can be found in Section B10.1 of Delaware’s QAPP.

The guidance and documentation on the data collection and verification systems can be found in Rapid Stream Restoration Monitoring Protocol, as well as in Delaware’s approved QAPP.

Independent Verification

The chosen system allows for verification by the agency responsible for implementation, with the possibility of hiring additional conservation district staff that would be responsible for practice verification. In addition, the permitting authority provides post construction approval upon project completion.

D2.3.2 Wetland Restoration

D2.3.2.1 Wetland Restoration - Assessment

This Verification Protocol incorporates all wetland related BMPs that are implemented and accounted for within Delaware’s WIP, including wetland restorations and creations. Table B10.1.1 in Section B.10 of this QAPP provides definitions for each of these BMP types. Details regarding verification and validation procedures for these practices are contained in Table D2.3.2.1 and summarized in the following sections.

2.3.2.1 Table – Wetland Restoration

Wetland Restoration Protocol Design Table: Visual Assessment BMPs – Multi-Year

| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| High | Visual Assessment: Multi-Year | Cost-Shared Wetland Restorations | Visual Assessment: On-site | 100% of All Tracked and Reported Wetland Restorations | Non-regulator Agency | Meets NRCS Code (657, 656) | <5 years | 10% per year | Work with landowner to achieve compliance. If cannot be achieved; remove from database | 10 years | 10% compliance checks by non-regulatory database |

D2.3.2.2 Wetland Restoration – Data Verification

Method

The Wetland Restoration Verification Protocol incorporates all wetland restorations that are implemented and accounted for in Delaware’s Chesapeake Bay Watershed Implementation Plan (WIP). All wetland restorations (100%) will be inspected through onsite visits while the restoration is being implemented. These onsite inspections will ensure the proper implementation of conservation practices, components, measures, or activities based on topography, groundwater, and stream water flow assessments. These visits will be conducted as often as needed, but at least annually.  Once restoration has been implemented all monitoring will occur at least once every 5 years through an onsite visit. The site will be monitored through offsite or remote sensing methods the other four years. This will entail verifying at least 10 percent of reported practices every year.

While wetland restoration practices are present year-round, most of the verification will occur during the fall, winter, and spring seasons to avoid scheduling conflicts during the busy summer season. NRCS verification timing will be at the organization’s discretion. It will take several years to determine if the BMP is properly functioning. Monitoring will continue throughout the determined monitoring period as established in the contract.

The wetland restoration BMP meets NRCS, State, and Chesapeake Bay Program (CBP) definitions for wetland restorations practices and have been approved by the CBP. Resource improvement practices are not reported. Reported BMPs, whether by NRCS or the state of Delaware, all have an implementation date, project type, and project size. Projects submitted by NRCS are reported at the county level. All other implemented practices are reported at the site-level with corresponding latitude-longitude coordinates.

The state has decided to adopt the NRCS methodology for collecting and monitoring wetland restoration projects. This methodology is approved by the CBP workgroup as well as NRCS and the state agencies reporting wetland restorations. The permitting authority provides post construction approval upon project completion.  All wetland restorations will be monitored in accordance with the following schedules, using the standard Wetland Reserve Program (WRP) monitoring worksheet. While the restoration is being implemented, all enrollments will be monitored through onsite visits to ensure the proper implementation of conservation practices, components, measures, or activities. These visits will be conducted as often as needed, but at least annually.

Once the restoration project has been implemented, all projects will be monitored at least once every five years through an onsite visit. The site will be monitored through offsite aerial imagery or remote sensing methods the other 4 years. Certain circumstances may also warrant more frequent onsite visits than the minimum 1 in 5 year requirement. Onsite visits must occur as described below in circumstances that include but are not limited to:

- Projects that have active compatible use authorizations (CUA) will have annual onsite visits for the first 2 years of the CUA to ensure compliance with and effectiveness of the CUA activity.

- Projects should have an onsite visit after each significant weather event or other potentially damaging event, including but not limited to flooding, forest fire, or other major storms.

- Projects will have onsite monitoring at least 2 consecutive years following a complete change in ownership.

- Projects will have onsite monitoring for at least 2 consecutive years following a documented violation.

During the monitoring process, the evaluator will record observations based on the questions found on the NRCS Wetlands Reserve Program (WRP) Monitoring Worksheet. A link to the worksheet can be found in the supporting documentation. The WRP Monitoring Worksheet aims to ensure restoration requirements are being met, evaluate progress, determine what restoration repairs or enhancements may be needed, and to maintain contact with the landowner. Photographs should also be taken to ensure accurate observation points. A summary of the general monitoring observations during a site visit is found below.  Each implementing agency uses this checklist below for field verification:

- Is the landowner present during the review?

- Has the landowner changed?

- Is the restoration boundary clearly marked and identifiable?

- Are the contract and agreement conditions being met?

- Are restoration practices being properly operated and maintained? (If not, what maintenance is needed? Fill in maintenance practice and cost worksheet.)

- Is the planned hydrology present? (ie. saturation or inundation. If no, what actions are needed?)

- Are maximum wildlife habitat objectives being achieved? (e.g. adequate hydrology, nesting cover, etc.)

- Are planned vegetation restoration goals being achieved (e.g. is desired vegetation being established, are invasive or noxious species a problem)? (If no, what modifications are necessary?)

- Are restoration practices being properly operated and maintained? (If no, what maintenance is needed?)

- Are there opportunities to enhance wildlife habitat components?

- Does the landowner have any concerns or suggestions for improvement of the project site?

- Identify concerns or suggestions from partners involved with the restoration and management of the restoration project.

- Additional observations or comments.

These practices will continue to be inspected once every five years while still in the initial lifespan and inspection dates will be updated in the database. Once the practice is no longer in its lifespan, the state will inspect the possibility of hiring a seasonal employee through DNREC’s Wetland Assessment and Monitoring Program to inspect existing practices and update the inspection dates in the database upon successful functional verification.

A more in depth summary of the protocol for verification of wetland restoration practices can be found in the NRCS Manual Title 440 – Part 514.66 – Wetland Reserve Program: http://directives.sc.egov.usda.gov/RollupViewer.aspx?hid=17111

The observations made during the verification process will utilize a similar verification sheet as NRCS: http://directives.sc.egov.usda.gov/OpenNonWebContent.aspx?content=28984.wba

Verification Team

BMPs are inspected and verified by trained NRCS, conservation district and state personnel. The appropriate staff will be trained in person to ensure the verification protocol is being followed and the correct information is being collected and reported back to the responsible agency. Special training and certification requirements can also be found in Delaware’s QAPP section A8 – Special Training/certification. There will be no certification requirement beyond the initial training for those collecting data. The verification collectors will call the project-sponsoring organization if questions arise about the specific project.

Documentation of Verification

Wetland Restoration data are requested on an annual or more frequent basis from numerous agencies that implement, track, and/or maintain wetland restoration practice data. BMPs are inspected and entered into databases by trained NRCS, USFWS, or DNREC staff.  Each Agency has separate verification databases and maintains files for implemented projects.  These data are provided to the Quality Assurance Officer annually for Chesapeake Bay progress submissions.

An outline of practice data submissions can be found in section B10 of Delaware’s QAPP. Additionally, in 2010, an agreement (Basinwide 1619 Agreement) was reached to have federal agencies, such as the USDA’s NRCS and FSA, report practices directly to the USGS for CBP modeling rather than have jurisdictions report on their behalf. DNREC –DWS-NPS maintains a restoration database that captures restoration practices like wetland restoration, tree plantings, forest buffers, and grass buffers. These practices are compiled from various projects throughout DNREC. More information can be found in Section B10.1 of Delaware’s QAPP.

Guidance and documentation on the data collection and verification systems can be found in NRCS’s Title 440 Part 514 – Wetland Reserve Program, as well as in Delaware’s approved QAPP.

Independent Verification

The chosen system allows for verification by the agency responsible for implementation, with the possibility of hiring additional conservation district staff that would be responsible for practice verification. In addition, the permitting authority provides post construction approval upon project completion.

D2.3.2.3 Wetland Restoration – Data Validation

Quality Assurance

Since all (100%) of stream restoration practices are inspected every year for the first 3 years and then once every 5 years and/or after major storm events and all (100%) of wetland restoration practices are inspected every 5 years or more, there is a consistent visual field check to see that the BMPs are still in place and functioning properly. These data are provided to DNREC-DWS-NPS for inclusion in Delaware’s existing NPS BMP Database and submission to the CBP through NEIEN.

The DE BMP Tracking and Reporting Tool is an online database that serves as a means of reporting and tracking BMPs in the state of Delaware. The use of this tool allows for a more streamlined approach for generating reports needed for water quality assessment and monitoring purposes. This database is used to submit data for inclusion in the Chesapeake Bay Watershed Model. Individual organizations are responsible for entering their practices with their provided login information and are only be permitted to review their own data.

The data are entered into the agricultural Excel template for upload into the NPS BMP Database. To confirm that the correct information has been copied into the templates from the raw data, the total area (e.g., acres, feet) or numbers for each BMP type are summed and compared to the original dataset to be sure there were no errors in translating the data. Data are also compared to previous years’ submissions to see if numbers are consistent (i.e., no extreme increases or decreases in acreage or count). The methodology for entering data for each BMP type is recorded every year so that the same methodology is consistently used year to year. See QAPP Section B10.1 for specific stream restoration BMP submittal methodology.

The BMP progress data are submitted to CBP every year by DNREC.

Data Entry

DNREC –DWS-NPS maintains a restoration database that captures restoration practices like wetland restoration, tree plantings, forest buffers, and grass buffers. These practices are compiled from various projects throughout DNREC. The restoration database links DNREC BMPs to NRCS practice codes. The DNREC BMP Tracking and Reporting Application (https://bmptracker.dnrecapps.net/) was revised in 2019 as the means of collecting and managing the nonpoint source sector best management practices (BMPs) implemented in the State of Delaware. It replaced the DE NPS BMP Database on December 1, 2019 as the central repository of BMP data from multiple agencies and departments allows a streamlined approach to generate reports needed for BMP tracking, BMP reporting, and water quality assessments. DNREC personnel use the BMP Tool to submit data to the Chesapeake Bay Program as required for the Bay TMDL. The BMP tool also facilitates analyses like evaluating BMP trends and targeting.

This Excel spreadsheet contains a template for reporting and tracking best management practices (BMPs). The BMP template helps agencies in the identification and regular reporting of best management practices to the Chesapeake Bay Program. Reported practices receive credit towards meeting the goals of the Chesapeake Bay nutrient and sediment total maximum daily loads (TMDLs).

The lifespan or credit duration of each BMP is also entered in the template based on the CreditDurations05222015.xlxs spreadsheet provided by CBP. This spreadsheet includes credit durations for each BMP type approved by the Ag Workgroup on May 21, 2015. The lifespan, or credit duration, of most stream restoration practices is 10 years. The lifespan of wetland restoration is 15 years. The code in the DE BMP Tracking and Reporting Tool has been modified so that the lifespan/credit duration is added to the implementation date of a particular BMP to calculate the Lifespan End Date. Once the Lifespan End Date has been passed, that BMP will be tagged as “retired”.

Double counting is avoided by submitting data by the primary funding source or the primary implementing agency. For example, BMP implementation data that is cost-shared with NRCS is submitted by NRCS. Non-cost shared data are submitted by the state or conservation districts.

The DNREC BMP Tracking and Reporting Application (https://bmptracker.dnrecapps.net/)  was revised in 2019 as the means of collecting and managing the nonpoint source sector best management practices (BMPs) implemented in the State of Delaware. It replaced the DE NPS BMP Database on December 1, 2019 as the central repository of BMP data from multiple agencies and departments allows a streamlined approach to generate reports needed for BMP tracking, BMP reporting, and water quality assessments. DNREC personnel use the BMP Tool to submit data to the Chesapeake Bay Program as required for the Bay TMDL. The BMP tool also facilitates analyses like evaluating BMP trends and targeting.

This Excel spreadsheet contains a template for reporting and tracking best management practices (BMPs). The BMP template helps agencies in the identification and regular reporting of best management practices to the Chesapeake Bay Program. Reported practices receive credit towards meeting the goals of the Chesapeake Bay nutrient and sediment total maximum daily loads (TMDLs).

External Data

Data are provided from external government agencies as mentioned above. The data are checked to be sure that they have been provided for the correct time period and that all necessary fields for NEIEN have been included.

Historic Data Verification

Historic data were provided by DNREC and NRCS. Data quality assurance and data entry were conducted the same way as in the past (as discussed in the above sections).

BMP Performance

During the visual field assessment of all stream and wetland restoration BMPs every 1 to 5 years, the BMPs are checked for signs of failure. If a stream restoration BMP is not performing up to its standards and specifications it is repaired if funding is available. If funding is not available, the BMP is removed from the database and will no longer be reported to NEIEN. To date, most stream restoration projects are completed on tax ditches that have a permitted easement for maintenance by the tax ditch organization, which is a governmental subdivision of the state. The implementing agency will work with the landowner to achieve compliance if a wetland restoration BMP is not performing up to its standards and specifications. If compliance cannot be achieved, the BMP is removed from the database.

Stream and Wetland Restoration Verification and Validation Summary

Stream restoration data are collected by both DNREC and NRCS following the Rapid Stream Restoration Monitoring Protocol methodology established by the Chesapeake Bay Field Office and U.S. Fish and Wildlife Service (June 2014). The Rapid Stream Restoration Monitoring Protocol can be found at the following location:

http://www.fws.gov/chesapeakebay/Stream/StreamsPDF/Restoration%20Monitoring%20Protocol%20FINAL%20%206-30-14.pdf

Wetland restoration data are collected by both DNREC and NRCS following the NRCS methodology for collecting and monitoring wetland restorations as part of the WRP. The WRP manual can be found here: http://directives.sc.egov.usda.gov/RollupViewer.aspx?hid=17111. This methodology is approved by the CBP workgroup as well as NRCS and the state agencies reporting wetland restorations. All wetland restorations will be monitored using the standard WRP monitoring worksheet:

### D2.4 Stormwater

This section of the verification protocol represents the BMP groupings for urban stormwater. Table F-4 in Appendix F provides a sector-specific checklist of Delaware BMP verification protocol components and maps them to the relevant QAPP sections where they are documented.

D2.4.1 Stormwater BMP Groups

D2.4.1.1 Wetponds and Wetlands, Infiltration Practices, Filtering Practices, Bioretention, and Bioswales

Wetponds and wetlands, infiltration practices, filtering practices, bioretention, and bioswales were grouped together because they all follow the same verification and validation protocol. Table B10.3.1 in Section B.10 of this QAPP provides definitions for each of these BMP types. Details regarding verification and validation procedures for these five practices are contained in Table D2.4.1.1 and summarized in the following sections.

2.4.1.1 Table – Stormwater Protocol Design Table: Wetponds and Wetlands, Infiltration Practices, Filtering Practices, Bioretention, and Bioswales

| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| High | Source sector | Structural | Visual Assessment: On-site | 100% of All Tracked and Reported at project closeout | Regulatory agency Self | Meets specs Visual functioning Location | Periodic inspections based on O&M plan (owner inspection) every 3-5 years – Section 7.3 of the DSSR | No – 100% inspected | Maintenance inspection report given to owner (delegated agency). Owners shall comply (Sections 7.3.3 and 7.4.1) | Not currently. They’re all still there. | QA Plan in place. Program checked and amended to ensure. Tri-annual reviews with delegated agencies (a few sites randomly picked to review) |

D2.4.1.2 Erosion and Sediment Control

Erosion and sediment control practices were grouped in their own section because they follow a slightly different verification and validation protocol than wet ponds and wetlands; infiltration practices; filtering practices; bioretention; and bioswales. Erosion and sediment control practices are temporary, while the other BMPs have a much longer lifespan. Table B10.3.1 provides definitions for each of these BMP types. Details regarding verification and validation procedures for erosion and sediment control practices are contained in Table D2.4.2.1 and summarized in the following sections.

Table 2.4.1.2 Stormwater Protocol Design Table: Erosion and Sediment Control

| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| High | Source sector | Structural Management | Visual Assessment: On-site | 100% of All Tracked and Reported – at project initiation (100% of all projects > 1 acre) | Regulatory agency Self Independent party | Meets specs Visual functioning | Weekly while under active construction | N/A (100%) | Compliance assistance Enforcement when needed | No longer needed after project closeout | QA Plan in place. Program checked and amended to ensure. |

D2.4.1.3 Street Sweeping

Street sweeping only covers a very small portion of the land in Delaware’s portion of the Chesapeake Bay watershed. Delaware Department of Transportation (DelDOT) only has permit requirements for street sweeping in New Castle County, not in the Chesapeake Bay watershed portions of Kent or Sussex counties. Although DelDOT does report their street sweeping results on an annual basis, it does not represent a significant reduction of sediment and nutrients in the watershed. Individual cites and/or townships also conduct street sweeping, but the data are not typically collected and reported to the state or the Bay Program. Details regarding verification and validation procedures for street sweeping are contained in Table D2.4.3.1 and summarized in the following sections.

Table 2.4.3.1 Stormwater Protocol Design Table: Street Sweeping

| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| Low/Moderate | Source sector | Stormwater management | Targeted monitoring | 1-3 years | Self | Location | NA | NA | NA | NA | QA Plan in place. |

D2.4.1.4 Stormwater BMP Data Verification

Method

One hundred percent of all wetponds and wetlands, infiltration practices, filtering practices, bioretention, and bioswales are inspected by visual on-site inspection every 3 to 5 years and one hundred percent of all erosion and sediment control practices are inspected by visual on-site inspection at the project initiation and then weekly during active construction based on the operation and maintenance (O&M) plan required by DNREC’s sediment and stormwater regulations. See: http://regulations.delaware.gov/AdminCode/title7/5000/5101.shtml . The inspection frequencies recommended in the regulations are shorter than the maximum inspection frequencies in this BMP Verification Plan.  The O&M Plan identifies the required maintenance for stormwater management systems. All of these BMPs must meet state standards and specifications. The data on the wetponds and wetlands, infiltration practices, filtering practices, bioretention, and bioswales are collected for several years (life of the BMP) to determine if the BMPs are functioning properly, while erosion and sediment control practices are collected during active construction until project closeout.

An implementation date is collected and reported for all of these BMPs. MudTracker was used from 2009 – 2019 to track BMP implementation. In 2020, the DNREC-DWS-SSP is requiring biannual submission of relevant BMP and maintenance data to be submitted by the nine Delegated Agencies, including DelDOT. Street Sweeping data should be reported separately by DelDOT. Further elements for enhanced BMPs will be collected in the future. All BMPs currently reported have also been approved by CBP for inclusion in the model application.

The CBP’s Urban Stormwater Workgroup recommended that the CBP develop BMP removal efficiencies for street sweeping. In March 2011, an Expert Panel came up with a set of recommendations for street sweeping, which is a CBP-approved credit. A description of the CBP’s street sweeping recommendations can be found at this link: http://chesapeakestormwater.net/bay-stormwater/urban-stormwater-workgroup/urban-street-sweeping/. Delaware’s Department of Transportation (DelDoT) and the City of Laurel follows these recommendations for street sweeping in the Chesapeake Bay Watershed.

According to the CBP’s recommendations, localities can use one of two methods to compute the projected nutrient reduction associated with street sweeping: 1) the mass loading approach and 2) the qualifying street lanes approach.

The preferred method is the mass loading approach and this is the approach that Delaware uses. The mass of street dirt collected during street sweeping operations is measured (in tons) at the landfill or ultimate point of disposal. The mass is then multiplied by factors to determine sediment and nutrient reduction credits.

Using the qualifying street lanes approach the locality reports the number of qualifying lane miles they have swept during the course of the year. This is then converted into total acres swept.

The pre-sweeping annual nutrient load for the swept acres is defined using the Simple Method (Schueler, 1987). The locality would multiply the total acres swept by the annual nutrient load to arrive at a baseline load. The baseline load is adjusted to determine the load reduction associated with street sweeping. This methodology is described in greater detail in the Expert Panel’s March 2011 memo: http://chesapeakestormwater.net/bay-stormwater/urban-stormwater-workgroup/urban-street-sweeping/

The sediment and nutrient reductions only apply to a street sweeping program conducted by a municipality that has the following characteristics:

- An urban street with a high average daily traffic volume located in commercial, industrial, central business district, or high intensity residential setting.

- Streets are swept at a minimum frequency of 26 times per year (every 2 weeks), although a municipality may want to bunch sweepings in the spring and fall to increase water quality impact.

- The reduction is based on the sweeping technology in use, with lower reductions for mechanical sweeping and higher reductions for vacuum assisted or regenerative air sweeping technologies.

- Localities need to document the length of lane miles swept using their traditional routes

Verification Team

The wetponds and wetlands, infiltration practices, filtering practices, bioretention, and bioswales are inspected by regulatory agencies. The BMP inspectors are trained DNREC or delegated agency employees, such as county conservation district employees. These employees will have completed DNREC Blue Card Training for Certified Construction Reviewer certification.  Inspections of erosion and sediment control practices are conducted by regulatory agencies, self-inspection, and by independent third-party inspectors who have completed the DNREC Certified Construction Reviewer (CCR) training. Training is ongoing as all new personnel are trained in the collection of BMP data; however, there is no “certification requirement” for staff collecting BMP data. If any of the data collectors have questions, they contact DNREC’s Sediment and Stormwater Program.

Currently, the only street sweeping conducted, in Delaware, which meets Chesapeake Bay Program standards are done through DelDOT staff.

Documentation of Verification

Wetponds and Wetlands, Infiltration Practices, Filtering Practices, Bioretention, and Bioswales

Data regarding the location of each wetpond and wetland, infiltration practice, filtering practice, bioretention area, and bioswale, their visual functioning, and whether or not the practice is meeting standards and specifications (Lucas 2005) are recorded in written files as well as in the Delegated Agencies’ local databases.  Each Delegated Agency provides a Stormwater BMP Implementation and Stormwater BMP Maintenance Verification report to DNREC Sediment and Stormwater Program on a bi-annual basis.

All wetpond and wetland, infiltration practice, filtering practice, bioretention area, and bioswale data are recorded using a maintenance inspection checklist. The maintenance inspection checklists can be found here: http://www.dnrec.delaware.gov/swc/Drainage/Pages/BMP-Maintenance-Review-Checklists.aspx

Erosion and Sediment Control

Data regarding the visual functioning of an erosion and sediment control practice and whether or not the practice is meeting standards and specifications (in the current Delaware Erosion and Sediment Control Handbook) are also recorded in written files, as well as in Delegated Agencies’ local databases and in CCR reports. The URL below contains a link to the list of NOIs for stormwater discharges associated with construction activities under a NPDES general permit: http://apps.dnrec.state.de.us/noi/

All erosion and sediment control practice data are recorded using a construction inspection checklist. The construction inspection checklists for various BMPs can be found here: http://www.dnrec.delaware.gov/swc/Drainage/Pages/BMP-Construction-Checklists.aspx

Street Sweeping data are recorded by DelDOT and housed in an internal database.  DNREC records data from the Town of Laurel’s street sweeping program into the NPS BMP database.

Independent Verification

Each delegated agency is subject to a triennial review by DNREC’s Sediment and Stormwater Program for each of their delegated elements related to stormwater BMPs, including maintenance inspections. A list of delegated agencies is available at the following link:

http://www.dnrec.delaware.gov/swc/Drainage/Pages/DelegatedAgencies.aspx. Some of the delegated agencies include the Delaware Department of Transportation (DelDOT), and New Castle, Kent, and Sussex County Conservation Districts.

D2.4.1.5 Stormwater BMP Data Validation

Quality Assurance

Since all (100%) of wetponds and wetlands, infiltration practices, filtering practices, bioretention, and bioswales are inspected every 3-5 years and all (100%) of erosion and sediment control practices are tracked and reported at the project initiation and then inspected weekly until project closeout, there is a consistent visual field check to see that the BMPs are still in place and functioning properly. These data are provided to DNREC-DWS-NPS for inclusion in Delaware’s existing NPS BMP Database and submission to the CBP through NEIEN.

DelDOT reports all street sweeping weights to the Quality Assurance Officer to be included in Delaware’s NPS BMP database.  DNREC records mileage for the Town of Laurel’s street sweeping program and data are handled by the Quality Assurance Officer and stored in the NPS BMP database.

The DE NPS BMP Database is an online database that serves as a means of reporting and tracking BMPs in the state of Delaware. The use of this tool allows for a more streamlined approach for generating reports needed for water quality assessment and monitoring purposes. This database is used to submit data for inclusion in the Chesapeake Bay Watershed Model. Individual organizations are responsible for entering their practices with their provided login information and are only be permitted to review their own data.

The data are entered into the urban Excel template for upload into the NPS BMP Database. To confirm that the correct information has been copied into the templates from the raw data, the total area (e.g., acres) or numbers for each BMP type are summed and compared to the original dataset to be sure there were no errors in translating the data. Data are also compared to previous years’ submissions to see if numbers are consistent (i.e., no extreme increases or decreases in acreage or count). The methodology for entering data for each BMP type is recorded every year so that the same methodology is consistently used year to year. See QAPP Section B10.3 for specific urban BMP submittal methodology.

The BMP progress data are submitted to CBP every year by DNREC.

Data Entry

Data are collected and entered Delegated Agencies’ local databases then submitted to DNREC-DWS-SSP bi-annually. Urban stormwater and street sweeping data are provided to NPS to input into the DE NPS BMP Database using the NEIEN input template with the correct NEIEN BMP names (see QAPP Section B10 for additional details on the parties involved in data submission to NEIEN). DE BMP Tracking and Reporting Tool is mapped to provide the data required to NEIEN and the CBP.

The lifespan or credit duration of each BMP is also entered in the template based on the CreditDurations05222015.xlxs spreadsheet provided by CBP. This spreadsheet includes credit durations for each BMP type approved by the Urban Stormwater Workgroup on March 17, 2015. Erosion and Sediment Control BMPs will have a lifespan only during construction.  Once the project is closed out of the NOI database, then the E&S BMP reaches the end of its life.  Street sweeping BMPs will be reported in lane miles annually and pounds.  All other BMPs will remain until replaced with another BMP or the site is developed.

Double counting is unlikely to occur for these stormwater practices because they are being provided by one agency (DNREC) and there are no cost-share practices.

The DNREC BMP Tracking and Reporting Application (https://bmptracker.dnrecapps.net/)  was revised in 2019 as the means of collecting and managing the nonpoint source sector best management practices (BMPs) implemented in the State of Delaware. It replaced the DE NPS BMP Database on December 1, 2019 as the central repository of BMP data from multiple agencies and departments allows a streamlined approach to generate reports needed for BMP tracking, BMP reporting, and water quality assessments. DNREC personnel use the BMP Tool to submit data to the Chesapeake Bay Program as required for the Bay TMDL. The BMP tool also facilitates analyses like evaluating BMP trends and targeting.

This Excel spreadsheet contains a template for reporting and tracking best management practices (BMPs). The BMP template helps agencies in the identification and regular reporting of best management practices to the Chesapeake Bay Program. Reported practices receive credit towards meeting the goals of the Chesapeake Bay nutrient and sediment total maximum daily loads (TMDLs).

External Data

This is not applicable to these stormwater BMPs as there are no external data collected. All data for wet ponds and wetlands; infiltration practices; filtering practices; bioretention; bioswales; and erosion and sediment control are provided by DNREC’s Sediment and Stormwater Program. The data are checked to be sure that they have been provided for the correct time period and that all necessary fields for NEIEN have been included.

DelDOT is a delegated agency, but street sweeping is conducted as part of their MS4 permit, not as a delegated element.

Historic Data Verification

Historic data were provided by DNREC’s Sediment and Stormwater Program. Data quality assurance and data entry were conducted the same way as in the past (as discussed in the above sections).

BMP Performance

During the visual field assessment of all wetponds and wetlands, infiltration practices, filtering practices, bioretention, and bioswales every 3 to 5 years, the BMPs are checked for signs of failure by DNREC and/or one of the Delegated Agencies. If a BMP is not performing up to its standards and specifications, the maintenance inspection report is provided to owner who has been designated on the Operation and Maintenance Plan as being responsible for maintenance of the stormwater management facilities.  The Owner of the stormwater management system shall comply with the conditions of the maintenance review within the timeframe specified by the Department or Delegated Agency.

During the weekly visual field assessment of all erosion and sediment control practices, the BMPs are checked for signs of failure through Owner self-inspections or by CCRs. One of the conditions of the Construction General Permit is weekly inspections.  If the site does not require a CCR to perform this function, the owner must keep a log of weekly self-inspections by a responsible person, typically the job foreman.  DNREC and/or delegated agencies conduct periodic oversight inspections to verify the accuracy of the inspection reports. If a BMP is not performing up to its standards and specifications, compliance assistance is provided and enforcement is applied when necessary according to Regs 7.3.3 The Owner of the stormwater management system shall comply with the conditions of the maintenance review within the timeframe specified by the Department or Delegated Agency; and Regs 7.4.1 The Department may seek enforcement action against an Owner deemed negligent in fulfilling the requirements of Section 7 of these regulations. If corrective measures have not been taken by the next inspection cycle, the BMP credit would be downgraded.

D2.4.3 Urban Nutrient Management and Industrial Stormwater

Urban nutrient management follows the same protocol as agricultural nutrient management and is included in the agriculture verification protocol section (Section D2.1). Section D1 of the QAPP discusses that verification protocols for additional BMPs with lower anticipated contributions to the overall load reductions will be developed but at a slower pace, given the reduced reliance on these practices to Delaware’s reduction strategy. For stormwater, this practice includes industrial stormwater. The design matrix table for industrial stormwater is included in Table 2.4.3.1 but additional details for industrial stormwater BMPs are not included in this verification protocol. Both of these practices only cover a very small portion of the land in Delaware’s portion of the Chesapeake Bay watershed and do not result in a significant reduction to nutrients or sediment to the Bay (see watermelon charts in Appendix G). Also note that Delaware Department of Transportation (DelDOT) only has permit requirements for street sweeping in New Castle County, not in the Chesapeake Bay watershed portions of Kent or Sussex counties. Although DelDOT does report their street sweeping results on an annual basis, it does not represent a significant reduction of sediment and nutrients in the watershed. Individual cites and/or townships also conduct street sweeping, but the data are not typically collected and reported to the state or the Bay Program.

2.4.3.1 Table - Stormwater Protocol Design Table: Industrial Stormwater

| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| Low | Source sector | Structural Management (SWIP) | Targeted onsite visual assessment Targeted monitoring | 100% of All Tracked and Reported at start of business operation | Regulatory agency Self Independent party | Water quality data Visual functioning Location | Annually | No | Compliance assistance Enforcement when needed | As long as business is operating. No tracking of lifespan. | QA Plan in place. |

D2.4.3.2 Urban Nutrient Management and Industrial StormwaterVerification and Validation Summary and Future Plans

Both groups of stormwater BMPs (1. wetponds and wetlands, infiltration practices, filtering practices, bioretention, and bioswales and 2. erosion and sediment control) already have BMP verification procedures in place that are fully operational and routinely carried out through the state of Delaware’s current sediment and stormwater regulations, which can be found at the following location:

http://regulations.delaware.gov/AdminCode/title7/5000/5101.shtml

Stormwater verification data are collected by DNREC Sediment and Stormwater staff and their delegated agencies, such as the New Castle, Kent, and Sussex County conservation districts and DelDOT.

Stormwater BMP maintenance checklists for wetponds and wetlands, infiltration practices, filtering practices, bioretention, and bioswales can be found at the following link:

http://www.dnrec.delaware.gov/swc/Drainage/Pages/BMP-Maintenance-Review-Checklists.aspx

The construction inspection checklists for various BMPs can be found here:

http://www.dnrec.delaware.gov/swc/Drainage/Pages/BMP-Construction-Checklists.aspx

A list of NOIs for stormwater discharges associated with construction activity under a NPDES general permit can be found at the following link:

http://apps.dnrec.state.de.us/noi/

Additional information regarding the Sediment Stormwater program can be found at:

http://www.dnrec.delaware.gov/swc/pages/sedimentstormwater.aspx

Delaware’s methodology for street sweeping can be found at: http://chesapeakestormwater.net/bay-stormwater/urban-stormwater-workgroup/urban-street-sweeping/

### D2.5 Wastewater Sector Verification Protocol

This section of the verification protocol represents the BMP groupings for wastewater practices. Table F-5 in Appendix F provides a sector-specific checklist of Delaware BMP verification protocol components and maps them to the relevant QAPP sections where they are documented.

D2.5.1 Septic Connections, Septic Denitrification, Septic Pumping

These three wastewater BMPs (septic connections, septic denitrification, septic pumping) were grouped together because they all follow the same verification and validation protocol.  Details regarding verification and validation procedures for these three practices are contained in Table D2.5.1.1 and summarized in the following sections.

Data reported in 2021 and prior years will be re-verified and updated in 2022/2023 by the Division of Water staff to properly reflect new installations and replacement systems in all categories.

2.5.1.1 Table - Septic Connections, Septic Denitrification, Septic Pumping

| Table B-3. Jurisdictional Wastewater Protocol Design Table: Onsite Wastewater |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| Low | Structural/ Multi Year Assessment | Septic Pumping (Convention-al systems) | Regulatory Requirement | Per State Regulations as described in Permit Conditions. (Once every 3 Years or 30% solids capacity) | Licensed Service Provider (Class F Liquid Waste Hauler) | Class F Pump-out Reports | Upon Complaint | 100% of complaints received | Yes – meet with landowner | 1/3 years | Email and web application submission. Working on improving process to database. |
| Low | Structural/ Multi Year Assessment | Septic Connections to Sewer | Targeted | Once | Regulatory Agency (County/ City/ Political Subdivision/Utility) | Septic System Abandonment Report from County/WWTP owner/Licensee | Upon Complaint | 100% of complaints received | Yes – meet with landowner | Perpetual | Onsite Database |
| Low | Structural/ Multi Year Assessment | Advanced Onsite Treatment Systems | Targeted | Per State Regulations | Licensed Service Provider | Inspection Reports | Upon Complaint | 100% of complaints received | Yes – meet with landowner /follow up system inspection | 25 Years (approved lifespan of advanced treatment system) | Written Inspection Reports/tracked in database Inspections are performed twice/year. |

D2.5.1 Septic Connections, Septic Denitrification, Septic Pumping Data Verification

Method

The Delaware Onsite Wastewater Program, through regulations, requires that onsite septic systems be installed and inspected by certified installers and inspectors (Section 4.0, Delaware Onsite Wastewater Regulations). Existing small on-site treatment systems that are less than 2,500 gallons per day are inspected at the transfer of a property by DNREC licensed Class H System Inspector. Upon completion of an inspection the completed inspection form is sent to the Department for review and placed in the program data base. If a system receives an unsatisfactory report then property owner or buyer is required to bring the system into compliance by repairing or replacing the system (Section 5.4.6.3, Delaware Onsite Regulations). A permit is required for repair or replacement and is issued by the Department. An additional inspection may be required to ensure proper installation or repair. If a cesspool or seepage pit is discovered during an inspection, the system is required to be replaced within one year of the property transfer in accordance with section 3.31.12 of Delaware’s Onsite Wastewater Regulations.

All large and community on-site treatment systems over 2,500 gallons per day are required to have a licensed operator and are inspected at least once a year. (Section 6.5.3.2.3 operating permit conditions; Section 6.7 General Operation and Maintenance Requirements and 6.7.2.2.4 Monitoring Program of Delaware Onsite Wastewater Regulations).

Onsite systems are required to be pumped on a triennial basis by licensed Class F Liquid Waste Haulers (Section 4.1.6, Delaware Onsite Wastewater Regulations) with pumpouts reported to the Onsite Wastewater Program at DNREC for tracking.

All new and/or replacement systems within 1000 feet of tidal waters in the watershed are also required to have septic denitrification systems/advanced treatment installed by Licensed Class E certified installers and follow an operation and maintenance program. Septic abandonments and connections to central systems are reported by the service provide and/or county.

Septic Pumpout data collection is currently submitted and tracked through emails, as well as mobile and electronic web submission portals.

Septic connections, pumpouts and denitrification systems are tracked in the Delaware Environmental Network (DEN). All of the elements required for CBP model application are currently being reported. All BMPs currently reported have also been approved by CBP for inclusion in the model application.

Verification Team

Septic pumpouts and installation of advanced treatment/denitrification systems are required by regulation to be installed/performed by licensed professionals. Septic haulers, inspectors, installers receive certification and licenses in accordance with regulations and most must receive ongoing training.

Documentation of Verification

Data regarding the location of each BMP, pump out records and system types are recorded in written files as well as in the DEN.

D2.5.1 Septic Connections, Septic Denitrification, Septic Pumping Data Validation

Quality Assurance

All septic inspections for property transfers are required to be performed by licensed Class H Inspectors. Each of these inspections includes a pumpout performed by licensed Class F Waste-haulers.  Once completed report is submitted to the Department for review and entry into database. In addition, DNREC Groundwater Discharges Section staff conduct inspections/audits of systems to ensure compliance by licensed professionals. Staff also respond to any complaints or concerns by system owners.

Data Entry

Data are collected and entered into the DEN by regulatory agency staff or an independent external party (i.e., contract employee). Wastewater data are provided to NPS to input into the DE BMP Tracking and Reporting Tool using the NEIEN input template with the correct NEIEN BMP names (see QAPP Section B10 for additional details on the parties involved in data submission to NEIEN). DE BMP Tracking and Reporting Tool is mapped to provide the data required to NEIEN and the CBP. In the future, it is expected that the data in DEN may be formatted into an XML file that will be linked to the required fields in the NEIEN template.

Double counting is unlikely to occur for these wastewater practices because they are being provided by one agency (DNREC) and there are no cost-share practices.

The DNREC BMP Tracking and Reporting Application (https://bmptracker.dnrecapps.net/)  was revised in 2019 as the means of collecting and managing the nonpoint source sector best management practices (BMPs) implemented in the State of Delaware. It replaced the DE NPS BMP Database on December 1, 2019 as the central repository of BMP data from multiple agencies and departments allows a streamlined approach to generate reports needed for BMP tracking, BMP reporting, and water quality assessments. DNREC personnel use the BMP Tool to submit data to the Chesapeake Bay Program as required for the Bay TMDL. The BMP tool also facilitates analyses like evaluating BMP trends and targeting.

This Excel spreadsheet contains a template for reporting and tracking best management practices (BMPs). The BMP template helps agencies in the identification and regular reporting of best management practices to the Chesapeake Bay Program. Reported practices receive credit towards meeting the goals of the Chesapeake Bay nutrient and sediment total maximum daily loads (TMDLs).

External Data

This is not applicable to these wastewater BMPs as there are no external data collected. All data for onsite septic systems are provided by DNREC’s Groundwater Discharges Section.  The data are checked to be sure that they have been provided for the correct time period and that all necessary fields for NEIEN have been included.

Historic Data Verification

In 2013, DNREC and DNREC Groundwater Discharges group updated GIS coverage for onsite sewer connections in the Chesapeake Bay watershed (DNREC QAPP 2015, Appendix E).  This project focused on data verification for reporting purposes. Data were verified by DNREC Groundwater Discharges staff and updated in the DEN database for onsite systems.

BMP Performance

Advanced Treatment/Denitrification systems are required to have an operations and maintenance contract and/or be performed by certified O&M provider. Inspection reports are required to be submitted to DNREC staff for tracking and reporting. System owners are provided with an inspection report and may become certified to provide O&M on their systems.

State or local authorities will verify, track and report proper installation and operation and maintenance (O&M) of on-site BMP systems. Verification may also occur through inspections performed by a certified design professional. Delaware does not require annual inspections for shallow placed pressure dosed, or elevated sand mound systems because they are confident in the performance of these technologies based on decades of experience. Additionally, there are other requirements in place, such as an inspection of any on-site system when a property is sold, that act as sufficient verification mechanisms for these technologies.

The design and installation of onsite BMP systems will be performed and reported by certified service providers and verified in the permitting process. All construction of on-site BMP systems are inspected by DNREC and system designer. Certificate of Satisfactory Completion is not issued until specific conditions and requirements are met which includes an O&M contract with a certified service provider.

The maintenance and inspection of on-site BMP systems will be conducted and reported annually by certified providers and tracked by the authorities. For some technologies, state or local authorities may stipulate an inspection frequency that is less than annual.  I/A (Nitrogen removal) systems less than or equal to 2,500 GPD. Systems permitted after 2/1/2007 inspected every 6 mos. by certified service provider. Systems installed prior to 2/1/2007 do not have to follow O&M requirements and are inspected by DNREC every three years. On-site systems must also be inspected when a property is sold.

Tracking and reporting through databases managed by state agencies. Delaware will maintain its own tracking database.

Reference:

Delaware’s Revised Septic System Regulations (effective Jan. 11, 2014). Delaware Department of Natural Resources and Environmental Control (DNREC), Division of

Water, Groundwater Discharges, Section 7 Delaware Code Chapter 60, Delaware Regulations Governing the Design, Installation, Operation of On-Site Wastewater Treatment and Disposal System (amended Jan. 11, 2014). http://www.dnrec.delaware.gov/wr/Information/GWDInfo/Pages/GWDS%20Design%20Install%20Operate%20Info%20For%20Proposed%20Wastewater%20Treatment%20Regulations.aspx.

D2.5.2 Wastewater Treatment Facilities

Delaware has four significant wastewater treatment facilities which discharge to land and surface waters within the Delaware Chesapeake Bay Watershed:

- Bridgeville Wastewater Treatment Plant. NPDES Permit effective April 1, 2014. As of 2021 Bridgeville's flows were transferred to Seaford Wastewater Treatment Plant and the aging facility will be taken off-line in 2022.

- Seaford Wastewater Treatment Plant – NPDES permit effective November 1, 2015 has been administratively extended until a new permit is issued. The new permit is expected to be issued in 2022/2023. Facility is currently operating within TMDL permit limits (with the trade agreement in effect).  Seaford’s current permit includes an approved trade agreement with INV Performance Materials, LLC (Formerly Invista).  The agreement is expected to become permanent and is reflected in the Phase III WIP.

- INV Performance Materials, LLC (Formerly Invista) Wastewater Treatment Plant – NPDES permit issued in 2021. Facility is currently operating within TMDL permit limits. INV Performance Materials, LLC’s current permit includes an approved trading agreement with Seaford. The agreement is expected to become permanent and is reflected in the Phase III WIP.

- Laurel Wastewater Treatment Plant – NPDES permit effective November 1, 2016. Laurel is currently operating within TMDL permit limits.

2.5.2.1 Table – Wastewater Treatment Facilities

| Table B-3. Jurisdictional Wastewater Protocol Design Table: Wastewater Treatment Facilities |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. BMP Priority | B. Data Grouping | C. BMP Type | D. Initial Inspection (is the BMP there?) |  |  |  | E. Follow-up Check |  |  | F. Lifespan/Sunset (Is the BMP no longer there?) | G. Data QA, Recording & Reporting |
|  |  |  | Method | Frequency | Who Inspects | Documentation | Follow-up Inspection | Statistical Sub-Sample | Response if Problem |  |  |
| Low | Wastewater | Treatment Technology | DMRs, Annual Reports | Monthly DMRs | DNREC | Monthly DMRs, Annual Reports | DNREC conducts annual inspection of each facility as well as data review | NA – All BMPs are inspected per state regulation | Enforcement action | NA – Annual BMP | DNREC conducts annual inspection of each facility. DNREC also reviews data and quality assurance review of data generated by the facility. Each facility has quality assurance procedures and protocols in place. |

Methodology

Facilities conduct monitoring and monthly reporting of flows and loads via DMRs. In addition, (a) annual loading reports are also submitted where trading or general permit conditions apply to a facility, and/or; (b) annual WIP reporting also applies. Reports are submitted to DNREC’s Surface Water Discharges Section.

NPDES permits and CFR 40 dictates procedures and protocols for monitoring flows and pollutants, sampling protocol and data collection.  Each facility is required to participate in annually in a DMR quality assurance analysis with DNREC Surface Water Discharges Section and EPA. As part of the QA study, each facility analyzes a blind sample and submits data to an EPA approved provider. The provider provides a report card to EPA and the Department. If an unacceptable report card is received, then the facility may be required to make corrective actions and retest.

DNREC also conducts an annual inspection of each facility as well as a data review and quality assurance review of data generated by the facility. Each facility has quality assurance procedures and protocols in place. Any issues identified by the annual inspection may require corrective action and DNREC Compliance and Enforcement staff provide follow up as needed to ensure compliance with the permit conditions and limits.

D2.5.3 Combined Sewer Overflows (CSOs)

Delaware has recently identified one CSO in the Town of Laurel. The town, through inspection and testing, has identified one section of the community which has storm drains connected to the wastewater treatment plant system. During heavy rain events (greater than 2 inch event) there is a possibility for discharge to occur. The Town of Laurel has a plan in place to separate the sewer system at five locations within town to prevent untreated overflows to the local waters. Construction is expected to begin by September 2016 and completed by May 2017.

• Construction Verification: properly designed, installed, and maintained by the certified service providers. The project has been designed by certified engineers and reviewed by DNREC staff engineers. Permits for construction will be obtained as part of the construction plan and will require regular inspections during the construction period. Sussex Conservation District will approve the construction site stormwater plan and conduct inspections.

• Post construction monitoring and inspection. The Town of Laurel Public Works Department will be responsible for long term maintenance and will enter into contracts with certified providers as required. Sussex Conservation District may provide additional inspections as needed.

• Existing compliance and enforcement procedures. The Town of Laurel is currently in compliance with their NPDES permit. Any enforcement action required will be conducted by the DNREC Division of Water Surface Water Discharges Section and/or EPA.

• Tracking and reporting. – Laurel is and will continue to track and report wastewater system flow in compliance with their NPDES permit. Discharge of Pollution is required to be reported to DNREC in accordance with state law.

# E References

CBP (Chesapeake Bay Program). 2014. Strengthening Verification of Best Management

Practices Implemented in the Chesapeake Bay Watershed: A Basinwide Framework. Annapolis, MD.

CBP (Chesapeake Bay Program). 2015. Credit Durations for Phase 6 Watershed Model.

Annapolis, MD.

DDA (Delaware Department of Agriculture). 2005. Nutrient Best Management Practices.  Delaware Department of Agriculture, Dover, DE. http://dda.delaware.gov/nutrients/D17762%20Best%20Mgmt%20booklet.pdf

DDA (Delaware Department of Agriculture). 2012. Delaware Nutrient Management Commission 2012 Annual Report.  Delaware Department of Agriculture, Dover, DE.  http://dda.delaware.gov/nutrients/downloads/2012_NMAnnualReport.pdf

DNREC (Department of Natural Resources and Environmental Control). 1999. Delaware Nonpoint Source Management Plan.  Department of Natural Resources and Environmental Control, Dover, DE.  http://www.dnrec.state.de.us/dnrec2000/Divisions/Soil/NPS/NPS%20management%20plan.pdf

DNREC (Department of Natural Resources and Environmental Control). 2004. State of Delaware Surface Water Quality Standards, as amended July 11, 2004. Department of Natural Resources and Environmental Control, Dover, DE.  http://www.dnrec.state.de.us/DNREC2000/Divisions/Water/WaterQuality/WQStandard.pdf

DNREC (Department of Natural Resources and Environmental Control). 2006. Delaware Sediment and Stormwater Regulations.  Department of Natural Resources and Environmental Control, Dover, DE. Projects prior to 1/1/14.  http://www.dnrec.state.de.us/DNREC2000/Divisions/Soil/Stormwater/Regs/SSRegs_4-05.pdf

DNREC (Department of Natural Resources and Environmental Control). 2013. Delaware Sediment and Stormwater Regulations. Department of Natural Resources and Environmental Control, Dover, DE.  Effective January 1, 2014.

http://regulations.delaware.gov/AdminCode/title7/5000/5101.pdf

DNREC (Department of Natural Resources and Environmental Control). 2006. State of Delaware 2006 Combined Watershed Assessment Report (305(b)) and Determination for the Clean Water Act Section 303(d) List of Waters Needing TMDLs. Department of Natural Resources and Environmental Control, Dover, DE.  http://www.dnrec.state.de.us/water2000/Sections/Watershed/TMDL/2006%20305(b)Final.pdf

DNREC (Department of Natural Resources and Environmental Control). 2007. Delaware Ambient Statewide Surface Water Quality Monitoring Program Quality Assurance Project Plan.

NRCS (Natural Resources Conservation Service). 2012. Delaware Quality Assurance Plan and Corrective Action Plan FY2012. Dover, DE.

NWA (Nanticoke Watershed Alliance). May 2015. Quality Assurance Project plan for Nanticoke Watershed Alliance’s Water Quality Monitoring Program Chemical and Physical Properties, Vienna, MD.

H.R. 6124 (110th): Food, Conservation, and Energy Act (Farm Bill) of 2008.

# Appendix A:  BMP Assessment for Delaware

# Appendix B Irrigated Land Methodology

Irrigated Land Area Update Methodology – Summer 2013 Work Group:

DNREC: Bryan Bloch, Tyler Monteith, Regina Kukola UD Extension Office: James Adkins (adkins@udel.edu)

Objective:

The acreage of irrigated land was calculated in July 2010 based on Google Earth Imagery by James Adkins. This project was an update to this dataset based on 2012 imagery in ArcGIS.

Methodology:

- An original dataset of irrigated land was established based on 2010 imagery by James Adkins

  - Polylines were drawn to identify irrigation systems on Delaware lands

  - These polylines were converted to polygon features in ArcGIS in order to calculate the acreage of these areas

  - These polygons were labeled as “July 3/4 2010” in the Imagery field of the databse

- As an update, a new data layer was created using 2012 imagery to track more current irrigation area

  - A grid was overlaid on the 2012 state land imagery to establish easier areas of examination (figure 1)

  - The 2010 data set of polygons was pulled in for reference

  - At a 1:4000 scale, each grid area was examined to determine where current irrigation practices existed

    - Irrigation practices that were still in existence from the 2010 set were copied to the 2012 Irrigation layer (the “July 3/4 2010” in the Imagery field remained to allow for a query of 2010 data and newly created 2012 data)

    - New irrigation practices not found in the 2010 layer were created as new polygon features. These were tagged with “2012” in the Imagery field to allow for a query of new data

    - A “Source” field was created to indicate who inputted the data (figure 3)

    - Some of the original 2010 data was adjusted via clipping/cutting tools in order to eliminate overlapping polygons

  - A geometry calculation was run in order to update the acreage of irrigated land based on the 2012 update.

Reporting:

- The updated geodatabase of 2012 irrigated land was sent to James Adkins at the UD Extension office at the end of August

- This data will be submitted as part of the Chesapeake Bay Submissions

Figure 1. ArcMap layers of 2010 and 2012 irrigation areas overlaid on 2012 imagery, as seen at a county level scale.

Figure 2. ArcMap layers of 2010 and 2012 irrigation areas overlaid on 2012 imagery, as seen as a single grid for identification.

Figure 3. The attribute table of the 2012 irrigation layer showing the imagery year used, acreage, and source of who inputted the data.

# Appendix C Water Control Structure Methodology

Report for Water Control Structures Project – Summer 2013

Team Members:

Bryan Bloch (Initial Database Creation and GIS work)

Regina Kukola (Site Prioritization, Site Visit Scheduling, Field Work) Tyler Monteith (Field Work, GIS work for updated GPS points)

Ryan Hendry (Field Work)

Contact info:

Delaware Dept. of Natural Resources and Environmental Control Division of Watershed Stewardship; Watershed Assessment Section 302-739-9939

Objective:

The purpose of this project was to update a database of water control structures (WCS) from the Sussex County Conservation District (SCD). These structures were implemented and funded by the SCD and therefore, have been verified in the past.  This project focused on data verification for reporting purposes. Primarily, we were interested in ground-truthing the GPS data for the structures. Our goal for the summer was to visit all 42 WCS listed in the database that were located within the Chesapeake Bay Basin.

Summary:

Water control structures provide controlled drainage to tax ditches in agricultural fields throughout the state of Delaware. Controlling water drainage from fields has important water quality implications. Discharge waters from fields with drainage control have been observed contain significantly less nitrates than discharge waters from fields with uncontrolled drainage. There are two mechanisms for this reduction in nitrate concentrations: 1. Water control structures reduce the total output of water leaving a field by 20 to 30% on average, and 2. the installation of water control structures raises the water table, and increases denitrification, which results in lower nitrate concentration in drainage waters (Osmund et al. 2002). All the structures discussed in this project report were funded by the Sussex County Conservation District.

By using a database supplied by the SCD and an ArcMap of the SCD database created by Bryan Bloch, we were able to create a list of 42 WCS in the Chesapeake Bay Basin in Sussex County that we needed to ground truth with GPS data. We met with Kip Foskey, a Planner at the SCD. He provided us with contact information for the private landowners that owned the land on which the 42 WCS were located. We called these private landowners to get their permission to visit the WCS. Of 23 landowners, we were able to successfully contact 20: 17 landowners were willing to give us permission to enter their properties this August, 2 were willing to give us permission after their summer crop seasons were over, and 1 did not give us permission this summer.

The 17 landowners that gave us permission to visit their properties owned 21 of the 42 WCS in the Chesapeake Basin in Sussex County.  We attempted to visit a total of 21 WCS over 4 field days in August and were able to successfully locate and obtain GPS information for 16 WCS. We then created a GIS document containing the GPS data we had collected.

If we contact landowners who were willing to allow us access in the fall or winter later this year, we can increase our site visits from 21 to 34. Also, Senior Conservation Planner, Kip Foskey (302-856-3990, ext. 114,kip.foskey@de.nacdnet.net)  is trying to get in touch with the 3 landowners we were not able to contact this summer. If these landowners give us permission to enter their property, we could increase our site visits by 5. The only landowner who did not give us permission to enter their property wanted to talk to Kip about our visit first. There is a chance that he might decide to allow us access to his 3 WCS after speaking with Kip.

The focus of this summer was to get information for all of the WCS in the Chesapeake Bay Basin. However, by repeating our methodology, information could be obtained for WCS statewide. If this methodology were to be repeated, we would recommend getting in touch with landowners ASAP and setting up field days to visit sites at least 2-3 weeks in advance to when phone calls are first made. Successfully making contact with landowners frequently took multiple calls, sometimes over the span of several weeks.

Contents of Expanded Narrative:

- Late June – Met with SCD to discuss project

- Mid July – Obtained ArcMap version of SCD Database

- Mid July – Developed Prioritization system for visiting WCS

- Mid July – Contacted Debbie Absher from SCD to obtain contact info for SCD planner to assist with contacting landowners

- End of July – Debbie provided contact info for SCD planner Kip Foskey

- Early August – Met with Kip to discuss contacting landowners

- Early August – Began contacting Landowners to visit WCS

- August – Visited WCS’s & Results

Expanded Narrative:

Late June – Met with SCD to discuss project

We met with Chip and Director of Agriculture Programs, Debbie Absher (302-856-3990, ext. 110; Debbie.Absher@de.nacdnet.net)  from the Sussex County Conservation District to discuss the project. All of the water control structures were on private land, so we needed to contact the land owners individually to ask for permission to visit the structure. They showed us a paper filing system of information about the water control structures and landowner contact information in the District’s office.  Debbie suggested that working with a planner from the Conservation District would be the easiest way to get access to the water control structures on private land. Consulting the paper database ended up not being necessary, because we were able to obtain contact information for landowners from Kip Foskey, the SCD Planner with whom we collaborated.

Mid July – Obtained ArcMap version of SCD Database

We consulted with Bryan Bloch about the map he created from the Sussex County Conservation District’s water control structure database. (J:\ChesBayProj\WCS\WCSMap1). The sum for the entire county was 169.  Debbie and Bryan could only locate 114 of the 169 (SCD_WCS

layer).  The number is low because some of the properties were located and point placed on the property but not the individual WCS’s since some properties have multiple WCS on them or location was not found at all. Bryan also went through the database to try and aerially determine the location of some structures. In the SCD_WCSMap1 attribute table field named “20” any point that reads Bryan Bloch was moved from its original location to a place that appeared more likely to have a water control structure by Bryan. Points that read original were not moved from their initial locations. For points that have read either “Bryan Bloch – check” or “original- check”, Bryan was not able to determine the placement of the structure aerially.

Mid July – Developed Prioritization system for visiting WCS

From Bryan’s work, we were able to determine the HUCs of the different WCS. We created a system to prioritize our visits of the structures, because we knew it would logistically be very difficult to visit every WCS in the database by the end of summer. Our prioritized list of WCS can be found at (F:\Watershed\2013InternDataUpdates\WCS_Verification\WCS priorities). A key to understanding the color coding in the document is below:

1st Priority – In the Chesapeake Bay and Bryan was unable to aerially infer structure’s location

2nd Priority – In the Chesapeake Bay and Bryan was able to aerially infer structure’s location

3rd Priority – Outside the Chesapeake Bay and Bryan was unable to aerially infer structure’s location

4th Priority – Outside the Chesapeake Bay and Bryan was able to aerially infer structure’s location

Our goal for the summer was to ground truth all of our 1st and 2nd priority structures (N = 42).

Mid July to End of July - Contacted Debbie Absher from SCD to obtain contact info for SCD planner to assist with contacting landowners, and Debbie provided contact info for SCD planner Kip Foskey.

In mid-July, we contacted Debbie for contact information for a SCD planner to assist us in gaining permission to WCS on private landowners’ properties. Due to state fair, she was unable to supply us with contact information for a SCD planner until the end of July.

Early August – Met with Kip to discuss contacting landowners

At the SCD office, Kip was able to supply us with Sussex Count mapping system maps of the different WCS. He also provided us with phone numbers of the landowners with WCS in the Chesapeake Bay Basin. An updated spreadsheet that reflects this contact info that Kip gave us can be found at F:\Watershed\2013InternDataUpdates\WCS_Verification\WCS information. (Note: WCS outside of the Chesapeake Basin are hidden rows. Rows without color fill are WCS we were able to visit.)

Early August – Began contacting Landowners to visit WCS

Overall, landowners were very willing to allow us to come on their property and take GPS data points.  However, there were a few landowners we were either unable to reach or could not give us access:

- We are still waiting on permission from WCS 6 owner.

- We attempted to contact WCS 8 owner on 8/2, 8/5, 8/14, 8/19, and 8/26 with no response

- We attempted to call WCS 13 owner on 8/2, 8/5, 8/14, 8/19, and 8/26 with no response

	We attempted to call WCS #17 owner on 8/2, 8/5, 8/13, 8/19, 8/21, 8/26

- WCS 17 is currently not accessible because of soy bean planting. In November, it will be accessible, and the owner would be willing to schedule a time for someone to come out to the WCS

- WCS 2 & 5 are currently not accessible because of corn planting. Once harvested, the owner would be willing to schedule a time for someone to come out to the WCS. This should be a higher priority because the SCD database has both properties listed as having 6 separate WCS each.

August – Visited WCS & Results (the data used to create these graphs is in WCS information.xls):

Overall, we spent 4 days in the field visiting a total of 21 WCS. Most commonly, we weren’t able to visit sites because of accessibility issues due to plantings (Fig. 1).

We were able to obtain GPS data points for 16 of the 21 sites we visited (Fig. 2).

3, 14%

1, 5%

2, 10%

15, 71%

WCS Found, Active, and GPS point taken

WCS Found, Inactive, GPS point taken

WCS Not Found

WCS Not Found, Inactive

13, 31%

5, 12%

3, 7%

21, 50%

Visited Successfully

Unable to visit because permission denied

Unable to visit because of current plantings

Unable to visit because no contact was established

Figure 1: Pie Graph showing successful data point collection

References:

Osmond, D.L., J.W. Gilliam and R.O. Evans. 2002. Riparian Buffers and Controlled Drainage to Reduce Agricultural Nonpoint Source Pollution, North Carolina Agricultural Research Service Technical Bulletin 318, North Carolina StateUniversity, Raleigh, NC.

Figure 2: Pie Graph showing successful site visits

# Appendix D Forest Harvesting Methodology

Forestry Harvested Area Update Methodology – Summer 2013 Work Group:

DNREC: Bryan Bloch, Tyler Monteith, Regina Kukola Forest Service: Sam Topper (sam.topper@state.de.us)

Objective:

The purpose of this project was to update forest harvest area data collected by the Delaware Forest Service to include ArcGIS coverage through the digitization of harvested forest areas. The digitization of these harvest areas are then linked to an Access database containing all permit information, creating a spatial reference. These files are located on a server at the Redden State Forest Office. This will also allow for the reporting of these harvests for inclusion in the Chesapeake Bay Model.

Methodology:

- Examined the current status of harvested forest areas comparing the contents of the Access Database (containing all information on the harvest permits) to the attribute table of the DDAForest_HarvestArea layer (containing the shapefiles of harvested areas already in existence)

  - Permits were categorized as being in the Access database but without a shapefile (our main task), those in both the Access File and had a shapefile (what is up-to- date), and those that had a shapefile but did not exist in the Access database (DDA’s task to update)

  - An excel file of the Access Database can be found at (F:)Watershed/2013InternDataUpdates/TimberHarvestPermits/Harvest_permits

- Shapefiles were created for harvest permits in Access Database

  - Identified all permit numbers lacking shapefiles

  - Used the hard copy of the harvest permit for reference. These documents were housed in the Forest Service office in Redden State Forest.

  - Used information from the permit and ArcMap layers in order to spatially locate the harvested area including:

    - Parcel/tax ID, Forest cover, historic aerial photography, hardcopy map of harvested area, nearest intersection, etc., as seen in figure 2.

  - Started an editing session in ArcMap using DDAForest_HarvestArea as the target and outlined the harvested area as identified in permit, using a scale of approximately 1:4,000

  - Once that shapefile was created, the attribute table for that shapefile was edited to include information on the updated shapefile, as seen in figure 3.

    - LinkField was added, composed of capital letter county followed by 4 digit year, 2 digit month, and 2 digit day based from the permit (ex. S20130701)

      - This field links the shapefile to the Access Database and automatically populates the remaining fields

    - Forester Initials – initials of forester responsible for the permit, found in permit

    - Year of permit

    - County permit was issued

    - Date that the shapefile was entered (day/month/year)

    - Username of person entering the data

    - The acres field will be populated through a calculated geometry calculator function after all shapefiles have been created

  - Once all possible shapefiles were created, the calculate geometry tool was used in order to calculate the acreage of each harvested area for reporting purposes

  - HUC12 codes were determined by importing a HUC12 data layer to do an intersect for determining which HUC12 each shape file was located in

    - Once determined, these locations were joined to the DDAForest_HarvestArea layer

- Some permits lacked sufficient information to effectively locate harvested area

  - A “nearest intersection” field was used to attempt to identify the harvested area

    - Some fields were able to be estimated based on size and historical land imagery changes between years

    - For those with too vague of descriptions, HUC12 Codes were generated

o A list of HUC Codes for these parcels can be found at (F:)Watershed/2013InternDataUpdates/TimberHarvestPermits/ Forestry_HUC_codes

    - 51 files were unable to, at minimum, determine a HUC code due to lack of sufficient information, as seen in figure 4.

- 12 permit shapefiles (.5% of all permits) were found in the DDAForest_harvestarea GIS layer, but do not exist in the Access Database.

  - A list of these permits was created and given to the Sam Topper for them to correct

  - Since the files exist as shapefiles in GIS, it will not affect our results

- An excel file of the progress of the project containing a list of permits divided by county, and the status of those parcels is located (F:)Watershed/2013InternDataUpdates/TimberHarvestPermits/Forestry_database_pro gress

Reporting:

For our purpose of reporting these practices for inclusion in the Bay Model, the template found at (F:)\Watershed\Chesapeake Bay\ContractorSupport\Tetra Tech\FY12 Deliverables\NEIEN methodology\2012_NEIEN Data.zip was used as a reference for the information needed for reporting, as seen in figure 5. A final version of the reporting spreadsheet can be found at (F:)Watershed/2013InternDataUpdates/TimberHarvestPermits/Timber_harvest_parcel_submissi on. The general template was mirrored, as mentioned above. Some parcels were located in multiple HUCs. For these, the portion of acreage in each corresponding HUC was calculated and reported in the Measure_value column. For fields that we were unable to create a shapefile, but were able to locate the associated HUC, the acreage reported came from the “Treated Area” recorded on the harvesting permit. The date located in the “BMP_EVENT_STATUS_CODE_DATE” column came from the implementation date found on the original harvest permits. If no implementation date was on the permit, the date that the permit was processed was used.

Figure 1: The ArcMap layer “DDAForest_HarvestArea” contains the shapefiles of harvested forest, indicated by the red outlines.

Figure 2: ArcMap layers including county parcels and historic aerial photography were used to locate the harvested area.

Figure 3: The DDAForest_HarvestArea attribute table containing the fields that need entering after a shapefile for the harvested area has been created.

Figure 4: Some parcels had missing information that made their location too vague to effectively locate, such as missing tax ID’s or property location descriptions.

Figure 5: This file was used as the basis for what information was needed for reporting purposes to the Bay Program.

# Appendix E Septic Connection Methodology

Septic System Abandonment and Count for the Chesapeake Bay Geographic Information Systems Methodology – November 2013

Delaware Department of Natural Resources and Environmental Control (DNREC) Watershed Assessment and Management Section (WAS): Bryan Bloch Groundwater Discharges Section (GWDS): Ron Graeber and Dave Schepens

Objective: The purpose of this project was to update the septic connection data collected by the Ground Water Discharges Section to include ArcGIS coverage.  This analysis is based on the assumption that anyone paying for sewer service is using central sewer; therefore, anyone who pays for sewer should be connected to central sewer. Billing data was acquired from municipalities and cross-referenced with GWDS septic database - Delaware Environmental Network (DEN).

Methodology:

- Examined current data to compare septic counts within the Chesapeake Bay Watershed.

Data used includes:

  - GWDS DEN query (including system abandonment reason connection to central sewer)

  - 2012 Imagery

  - Google Imaging Services

  - Municipal Sewer Districts/Area (06/2013)

  - Grid 1.5 mile X 1.5 mile (489 total cells to verify)

  - County parcels-vacant/non-vacant

  - Billing Addresses or Parcel Provided by

    - Bridgeville

    - Seaford

    - Sussex County

    - Kent County-EDU’S attached

    - Still in need of data from the following municipalities:

      - Laurel

      - Delmar

      - New Castle

      - Middletown

      - Farmington

      - Greenwood

      - Harrington

- Linked municipal or county sewer billing data to tax parcels-geocoded addresses:

  - If EDU’S were attached to data that was amended to parcel attribute

o If not, assumed dwelling was one EDU, or count of dwellings on one parcel

- Using the DEN onsite point data, areas within sewer billing area, assumed connected to central sewer. EDU data was attached.

- Points were created for those parcels found to be paying for sewer services and were not found in DEN. Using 2012 imagery, points were created on the sewer service parcels.

- These data will be compiled and submitted for inclusion in the 2013 Chesapeake Bay Submission.

- Some issues were encountered when analyzing the data but were rectified:

  - Some parcels did not completely match county/municipal data

  - Geocoding addresses made it tough at time to figure which dwelling was on sewer (Bridgeville)

  - Abandoned dwellings viewing with aerial imagery, use imagery and google and parcel data if applicable to determine vacant or not

  - Data has yet to be mapped from billing data from some areas. DEN Issues:

- The DEN database is used to track permits

- Duplicates must be removed

- Other permit statuses must be updated

- Cannot determine if prior septic systems have been abandoned or connected to sewer.

- can’t assume a specific permit status since data has not been entered in completely for all parcels, so must look one by one or digitize)

Future Recommendations:

- The State of Delaware needs a central septic tracking database.  Not just for permits but for septics, sewer connections, abandonments, and pump-outs.

- Each septic record should be recorded with lat/long and number of EDU’S connected to sewer.

- GPS actual septic system location when installed or when a Class H inspection is done if applicable rather than a point being created based on the centroid of a given parcel.

Billing Contact Information: (Updated 2022)

| Municipality | Contact Name | Phone | Email |
| --- | --- | --- | --- |
| Bridgeville | Jesse Savage- Town Manager April Buckler-Billing | 302-337-7135 | jsbridgeville@gmail.com abuckler@ddmg.net |

| Seaford | Sharon Drugash-Payroll Berley Mears-Director of Public Works | 302-629-8307 | sdrugash@seafordde.com |
| --- | --- | --- | --- |
| Sussex County | Hans Medlarz-County Engineer | Public Works 302-854-5396 | hmedlarz@sussexcountyde.gov |
| Kent County | Zach Lawson-GIS | 302-744-2430 | publicworks@co.kent.de.us Zach.Lawson@co.kent.de.us |
| Laurel | James Foskey-Public Works Jamie Smith- Operations Manager | 302-875-2277 | laurelpwd@comcast.net laurelop@comcast.net |
| Delmar |  |  |  |
| New Castle |  |  |  |
| Middletown |  |  |  |
| Farmington |  |  |  |
| Greenwood |  |  |  |
| Harrington |  |  |  |

# Appendix F Mapping of Jurisdiction BMP Verification Protocol Components to the Relevant QAPP Sections

Table F-1. Mapping of Jurisdiction BMP Verification Protocol Components to the Relevant QAPP Sections – Agriculture.

| Sector: | Agriculture |  |
| --- | --- | --- |
|  | BMP Verification Component | QAPP Section |
| 1 | BMP's Collected |  |
|  | Type (structural, management, annual, etc.) | Verification Design Protocol Tables Cover Crops 2.1.1.1 Conservation Plans 2.1.2.1 Nutrient Management Plans 2.1.3.1 Manure Relocation 2.1.4.1 Conservation Tillage 2.1.5.1 Multi-Year Animal BMPs 2.1.6.1 Multi-Year Land BMPs 2.1.7.1 |
|  | BMP Funding/Cost shared (federal, state, NGO, non-cost shared) | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Distinct state standards/specifications | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Matching CBP BMP definition/efficiencies | Spreadsheet: NEIEN NPS BMP CBP Section 10.1.1 |
| 2 | Method/System of Verification/Assessment |  |
|  | Description of methods/systems to be used | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Documentation of procedures used to verify BMPs | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Instruction manual for system users | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
| 3 | Who Will Complete the Verification |  |
|  | Qualification requirements | Cover Crops Conservation Plans Nutrient Management Plans Manure Relocation Conservation Tillage Multi-Year Animal BMPs Multi-Year Land BMPs |
|  | Training requirements | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | Certification requirements | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | CEU follow-up training requirements in the future | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
| 4 | Documentation of Verification Finding |  |
|  | Date of installation | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Location (lat/long if applicable) | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Level of reporting (watershed, HUC, county, sitespecific, etc.) | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Units (number, acres, length, etc.) needed for NEIEN | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Ownership (public, private) | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Documentation: | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Pictures | N/A |
|  | Worksheets | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Multi-Year Animal BMPs 2.1.6.2 |
|  | Electronic Tool | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Aerial Photos | N/A |
|  | Maps | N/A |
|  | Other | N/A |
|  | Report Generator | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
| 5 | How Often Reviewed (Cycle of review) |  |
|  | 1-2 years | Verification Design Protocol Tables Cover Crops 2.1.1.1 Conservation Plans 2.1.2.1 Nutrient Management Plans 2.1.3.1 Manure Relocation 2.1.4.1 Conservation Tillage 2.1.5.1 |
|  | 5 years | Verification Design Protocol Tables Multi-Year Animal BMPs 2.1.6.1 Multi-Year Land BMPs 2.1.7.1 |
|  | 10 years | Verification Design Protocol Tables Multi-Year Animal BMPs 2.1.6.1 Multi-Year Land BMPs 2.1.7.1 |
|  | Other | N/A |
| 6 | Independent Verification of Finding |  |
|  | Is this a requirement? | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | Internal Independent | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | External Independent | Cover Crops 2.1.1.2 Conservation Plans 2.1.2.2 Nutrient Management Plans 2.1.3.2 Manure Relocation 2.1.4.2 Conservation Tillage 2.1.5.2 Multi-Year Animal BMPs 2.1.6.2 Multi-Year Land BMPs 2.1.7.2 |
|  | BMP Data Validation |  |
| 7 | Quality Assurance/Spot Checking |  |
|  | Who-qualifications/training/certification | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | Method to select BMP for follow-up check | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | Method to select the number of BMPs to review | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | Other | N/A |
| 8 | Data Entry of BMP Implementation |  |
|  | What is the system? | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | Who enters data (training/certification)? | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | Does the system connect to NEIEN? | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | System in place prevent double counting | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
| 9 | External Provided Data Validation Meeting CBP Partnership Guidance |  |
|  | Method to validate data | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | Who will validate data (training/certification)? | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
| 10 | Historic Data Verification |  |
|  | System to re-certify or remove | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | Who will verify historic data training/certification)? | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | Documentation of action | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | BMP Performance |  |
| 11 | Does state collect data to assess BMP Performance? |  |
|  | System used to collect BMP performance data? | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | Who collects BMP performance data? | Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
|  | Who analyzes collected data and report to CBP? | Figure 1 Cover Crops 2.1.1.3 Conservation Plans 2.1.2.3 Nutrient Management Plans 2.1.3.3 Manure Relocation 2.1.4.3 Conservation Tillage 2.1.5.3 Multi-Year Animal BMPs 2.1.6.3 Multi-Year Land BMPs 2.1.7.3 |
| Source: Derived from Table 7 in CBP 2014. |  |  |

Table F-2. Mapping of Jurisdiction BMP Verification Protocol Components to the Relevant QAPP Sections – Forestry

| Sector: | Forestry |  |
| --- | --- | --- |
|  | BMP Verification Component | QAPP Section |
| 1 | BMP's Collected |  |
|  | Type (structural, management, annual, etc.) | Tables D2.2.1.1 and D2.2.2.1 Verification Design Protocol |
|  | BMP Funding/Cost shared (federal, state, NGO, non-cost shared) | Inspection fee. But may need additional database development funding with increasing data requirements. |
|  | Distinct state standards/specifications | Section D2.2.1.2 |
|  | Matching CBP BMP definition/efficiencies | Spreadsheet: NEIEN NPS BMP CBP Data Flow (Appendix8.26_01032014) |
| 2 | Method/System of Verification/Assessment |  |
|  | Description of methods/systems to be used | Tables D2.2.1.1 and D2.2.2.1 - Verification Design Protocol |
|  | Documentation of procedures used to verify BMPs | Section D2.2.1.2 |
|  | Instruction manual for system users | Section D2.2.1.2 |
| 3 | Who Will Complete the Verification |  |
|  | Qualification requirements | Section D2.2.1.2 |
|  | Training requirements | Section D2.2.1.2 |
|  | Certification requirements | Section D2.2.1.2 |
|  | CEU follow-up training requirements in the future | Section D2.2.1.2 |
| 4 | Documentation of Verification Finding |  |
|  | Date of installation | Section D2.2.1.2 |
|  | Location (lat/long if applicable) | Section D2.2.1.2 - see maintenance checklist |
|  | Level of reporting (watershed, HUC, county, sitespecific, etc.) | Section D2.2.1.2 - see maintenance checklist |
|  | Units (number, acres, length, etc.) needed for NEIEN | Section D2.2.1.2 |
|  | Ownership (public, private) | Section D2.2.1.2 - see maintenance checklist |
|  | Documentation: |  |
|  | Pictures | n/a |
|  | Worksheets | Section D2.2.1.2 - see maintenance checklist |
|  | Electronic Tool | n/a |
|  | Aerial Photos | n/a |
|  | Maps | n/a |
|  | Other | Section D2.2.1.2 - see maintenance checklist |
|  | Report Generator | Section D2.2.1.2 |
| 5 | How Often Reviewed (Cycle of review) |  |
|  | 1-2 years | Tables D2.2.1.1 and D2.2.2.1 Verification Design Protocol |
|  | 5 years | Tables D2.2.1.1 and D2.2.2.1 Verification Design Protocol |
|  | 10 years | Tables D2.2.1.1 and D2.2.2.1 Verification Design Protocol |
|  | Other | Tables D2.2.1.1 and D2.2.2.1 - Verification Design Protocol |
| 6 | Independent Verification of Finding |  |
|  | Is this a requirement? | Section D2.2.1.2 |
|  | Internal Independent | Section D2.2.1.2 |
|  | External Independent | Section D2.2.1.2 |
|  | BMP Data Validation |  |
| 7 | Quality Assurance/Spot Checking |  |
|  | Who-qualifications/training/certification | Section D2.2.1.3 |
|  | Method to select BMP for follow-up check | Section D2.2.1.3 |
|  | Method to select the number of BMPs to review | Section D2.2.1.3 |
|  | Other | Section D2.2.1.3 |
| 8 | Data Entry of BMP Implementation |  |
|  | What is the system? | Section D2.2.1.3 |
|  | Who enters data (training/certification)? | Section D2.2.1.3 |
|  | Does the system connect to NEIEN? | Section D2.2.1.3 |
|  | System in place prevent double counting | Section D2.2.1.3 |
| 9 | External Provided Data Validation Meeting CBP Partnership Guidance |  |
|  | Method to validate data | Section D2.2.1.3 |
|  | Who will validate data (training/certification)? | Section D2.2.1.3 |
| 10 | Historic Data Verification |  |
|  | System to re-certify or remove | Section D2.2.1.3 |
|  | Who will verify historic data training/certification)? | Section D2.2.1.3 |
|  | Documentation of action | Section D2.2.1.3 |
|  | BMP Performance |  |
| 11 | Does state collect data to assess BMP Performance? |  |
|  | System used to collect BMP performance data? | Section D2.2.1.3 |
|  | Who collects BMP performance data? | Section D2.2.1.3 |
|  | Who analyzes collected data and report to CBP? | Section D2.2.1.3 |
| Source: Derived from Table 7 in CBP 2014. |  |  |

Table F-3. Mapping of Jurisdiction BMP Verification Protocol Components to the Relevant QAPP Sections – Restoration.

| Sector: | Restoration |  |
| --- | --- | --- |
|  | BMP Verification Component | QAPP Section |
| 1 | BMP's Collected |  |
|  | Type (structural, management, annual, etc.) | Tables D2.3.1.1 and D2.3.2.1 Verification Design Protocol |
|  | BMP Funding/Cost shared (federal, state, NGO, non-cost shared) | Section D2.3.1.2 and D2.3.2.2 |
|  | Distinct state standards/specifications | Section D2.3.1.2 and D2.3.2.2 |
|  | Matching CBP BMP definition/efficiencies | Spreadsheet: NEIEN NPS BMP CBP Data Flow (Appendix8.26_01032014) |
| 2 | Method/System of Verification/Assessment |  |
|  | Description of methods/systems to be used | Tables D2.3.1.1 and D2.3.2.1 Verification Design Protocol |
|  | Documentation of procedures used to verify BMPs | Section D2.3.1.2 and D2.3.2.2 |
|  | Instruction manual for system users | Section D2.3.1.2 and D2.3.2.2 |
| 3 | Who Will Complete the Verification |  |
|  | Qualification requirements | Section D2.3.1.2 and D2.3.2.2 |
|  | Training requirements | Section D2.3.1.2 and D2.3.2.2 |
|  | Certification requirements | Section D2.3.1.2 and D2.3.2.2 |
|  | CEU follow-up training requirements in the future | Section D2.3.1.2 and D2.3.2.2 |
| 4 | Documentation of Verification Finding |  |
|  | Date of installation | Section D2.3.1.2 and D2.3.2.2 |
|  | Location (lat/long if applicable) | Section D2.3.1.2 and D2.3.2.2 |
|  | Level of reporting (watershed, HUC, county, sitespecific, etc.) | Section D2.3.1.2 and D2.3.2.2 |
|  | Units (number, acres, length, etc.) needed for NEIEN | Section D2.3.1.2 and D2.3.2.2 |
|  | Ownership (public, private) | Section D2.3.1.2 and D2.3.2.2 |
|  | Documentation: | Section D2.3.1.2 and D2.3.2.2 |
|  | Pictures | N/A |
|  | Worksheets | Section D2.3.1.2 and D2.3.2.2 |
|  | Electronic Tool | Section D2.3.1.2 and D2.3.2.2 |
|  | Aerial Photos | n/a |
|  | Maps | n/a |
|  | Other | n/a |
|  | Report Generator | Section D2.3.1.2 and D2.3.2.2 |
| 5 | How Often Reviewed (Cycle of review) |  |
|  | 1-2 years |  |
|  | 5 years |  |
|  | 10 years | Tables D2.3.1.1 and D2.3.2.1 Verification Design Protocol |
|  | Other |  |
| 6 | Independent Verification of Finding |  |
|  | Is this a requirement? | Section D2.3.1.2 and D2.3.2.2 |
|  | Internal Independent | Section D2.3.1.2 and D2.3.2.2 |
|  | External Independent | Section D2.3.1.2 and D2.3.2.2 |
|  | BMP Data Validation |  |
| 7 | Quality Assurance/Spot Checking |  |
|  | Who-qualifications/training/certification | Section 2.3.2.3 |
|  | Method to select BMP for follow-up check | Section 2.3.2.3 |
|  | Method to select the number of BMPs to review | Section 2.3.2.3 |
|  | Other | Section 2.3.2.3 |
| 8 | Data Entry of BMP Implementation |  |
|  | What is the system? | Section 2.3.2.3 |
|  | Who enters data (training/certification)? | Section 2.3.2.3 |
|  | Does the system connect to NEIEN? | Section 2.3.2.3 |
|  | System in place prevent double counting | Section 2.3.2.3 |
| 9 | External Provided Data Validation Meeting CBP Partnership Guidance |  |
|  | Method to validate data | Section 2.3.2.3 |
|  | Who will validate data (training/certification)? | Section 2.3.2.3 |
| 10 | Historic Data Verification |  |
|  | System to re-certify or remove | Section 2.3.2.3 |
|  | Who will verify historic data training/certification)? | Section 2.3.2.3 |
|  | Documentation of action | Section 2.3.2.3 |
|  | BMP Performance |  |
| 11 | Does state collect data to assess BMP Performance? |  |
|  | System used to collect BMP performance data? | Section 2.3.2.3 |
|  | Who collects BMP performance data? | Section 2.3.2.3 |
|  | Who analyzes collected data and report to CBP? | Section 2.3.2.3 |

Table F-4. Mapping of Jurisdiction BMP Verification Protocol Components to the Relevant QAPP Sections – Stormwater.

| Sector: | Stormwater |  |
| --- | --- | --- |
|  | BMP Verification Component | QAPP Section |
| 1 | BMP's Collected |  |
|  | Type (structural, management, annual, etc.) | Tables D2.4.1.1 and D2.4.2.2 Verification Design Protocol |
|  | BMP Funding/Cost shared (federal, state, NGO, non-cost shared) | Inspection fee. But may need additional database development funding with increasing data requirements. |
|  | Distinct state standards/specifications | Section D2.4.1 and D2.4.2 |
|  | Matching CBP BMP definition/efficiencies | Spreadsheet: NEIEN NPS BMP CBP Data Flow (Appendix8.26_01032014) |
| 2 | Method/System of Verification/Assessment |  |
|  | Description of methods/systems to be used | Tables D2.4.1.1 and D2.4.2.2 Verification Design Protocol |
|  | Documentation of procedures used to verify BMPs | Section D2.4.1.2 and D2.4.2.2 and D2.4.3.2 |
|  | Instruction manual for system users | Section D2.4.1.2 and D2.4.2.2 and D2.4.3.2 |
| 3 | Who Will Complete the Verification |  |
|  | Qualification requirements | Section D2.4.1.2 and D2.4.2.2 and D2.4.3.2 |
|  | Training requirements | Section D2.4.1.2 and D2.4.2.2 and D2.4.3.2 |
|  | Certification requirements | Section D2.4.1.2 and D2.4.2.2 and D2.4.3.2 |
|  | CEU follow-up training requirements in the future | Section D2.4.1.2 and D2.4.2.2 and D2.4.3.2 |
| 4 | Documentation of Verification Finding |  |
|  | Date of installation | Section D2.4.1 and D2.4.2 |
|  | Location (lat/long if applicable) | Section D2.4 - see maintenance checklist |
|  | Level of reporting (watershed, HUC, county, sitespecific, etc.) | Section D2.4 - see maintenance checklist |
|  | Units (number, acres, length, etc.) needed for NEIEN | Section D2.4 |
|  | Ownership (public, private) | Section D2.4 - see maintenance checklist |
|  | Documentation: |  |
|  | Pictures | n/a |
|  | Worksheets | Section D2.4 - see maintenance checklist |
|  | Electronic Tool | n/a |
|  | Aerial Photos | n/a |
|  | Maps | n/a |
|  | Other | Section D2.4 - see maintenance checklist |
|  | Report Generator | Section D2.4 |
| 5 | How Often Reviewed (Cycle of review) |  |
|  | 1-2 years | Tables D2.4.1.1 and D2.4.2.2 Verification Design Protocol |
|  | 5 years | Tables D2.4.1.1 and D2.4.2.2 Verification Design Protocol |
|  | 10 years | Tables D2.4.1.1 and D2.4.2.2 Verification Design Protocol |
|  | Other | Tables D2.4.1.1 and D2.4.2.2 Verification Design Protocol |
| 6 | Independent Verification of Finding |  |
|  | Is this a requirement? | Section D2.4.1.2 and D2.4.2.2 |
|  | Internal Independent | Section D2.4.1.2 and D2.4.2.2 |
|  | External Independent | Section D2.4.1.2 and D2.4.2.2 |
|  | BMP Data Validation |  |
| 7 | Quality Assurance/Spot Checking |  |
|  | Who-qualifications/training/certification | Section D2.4.1.3 and D2.4.2.3 |
|  | Method to select BMP for follow-up check | Section D2.4.1.3 and D2.4.2.3 |
|  | Method to select the number of BMPs to review | Section D2.4.1.3 and D2.4.2.3 |
|  | Other | Section D2.4.1.3 and D2.4.2.3 |
| 8 | Data Entry of BMP Implementation |  |
|  | What is the system? | Section D2.4.1.3 and D2.4.2.3 |
|  | Who enters data (training/certification)? | Section D2.4.1.3 and D2.4.2.3 |
|  | Does the system connect to NEIEN? | Section D2.4.1.3 and D2.4.2.3 |
|  | System in place prevent double counting | Section D2.4.1.3 and D2.4.2.3 |
| 9 | External Provided Data Validation Meeting CBP Partnership Guidance |  |
|  | Method to validate data | Section D2.4.1.3 and D2.4.2.3 |
|  | Who will validate data (training/certification)? | Section D2.4.1.3 and D2.4.2.3 |
| 10 | Historic Data Verification |  |
|  | System to re-certify or remove | Section D2.4.1.3 and D2.4.2.3 |
|  | Who will verify historic data training/certification)? | Section D2.4.1.3 and D2.4.2.3 |
|  | Documentation of action | Section D2.4.1.3 and D2.4.2.3 |
|  | BMP Performance |  |
| 11 | Does state collect data to assess BMP Performance? |  |
|  | System used to collect BMP performance data? | Section D2.4.1 and D2.4.2 |
|  | Who collects BMP performance data? | Section D2.4.1 and D2.4.2 |
|  | Who analyzes collected data and report to CBP? | Section D2.4.1 and D2.4.2 |

Table F-5. Mapping of Jurisdiction BMP Verification Protocol Components to the Relevant QAPP Sections – Wastewater.

| Sector: | Wastewater |  |
| --- | --- | --- |
|  | BMP Verification Component | QAPP Section |
| 1 | BMP's Collected |  |
|  | Type (structural, management, annual, etc.) | Tables D2.5.1.1 Verification Design Protocol |
|  | BMP Funding/Cost shared (federal, state, NGO, non-cost shared) | Inspection fee. But may need additional database development funding with increasing data requirements. |
|  | Distinct state standards/specifications | Section D2.5.1.2 |
|  | Matching CBP BMP definition/efficiencies | Spreadsheet: NEIEN NPS BMP CBP Data Flow (Appendix8.26_01032014) |
| 2 | Method/System of Verification/Assessment |  |
|  | Description of methods/systems to be used | Tables D2.5.1.1 - Verification Design Protocol |
|  | Documentation of procedures used to verify BMPs | Section D2.5.1.2 |
|  | Instruction manual for system users | Section D2.5.1.2 |
| 3 | Who Will Complete the Verification |  |
|  | Qualification requirements | Section D2.5.1.2 |
|  | Training requirements | Section D2.5.1.2 |
|  | Certification requirements | Section D2.5.1.2 |
|  | CEU follow-up training requirements in the future | Section D2.5.1.2 |
| 4 | Documentation of Verification Finding |  |
|  | Date of installation | Section D2.5.1.2 |
|  | Location (lat/long if applicable) | Section D2.5.1.2 - see maintenance checklist |
|  | Level of reporting (watershed, HUC, county, sitespecific, etc.) | Section D2.5.1.2 - see maintenance checklist |
|  | Units (number, acres, length, etc.) needed for NEIEN | Section D2.5.1.2 |
|  | Ownership (public, private) | Section D2.5.1.2 - see maintenance checklist |
|  | Documentation: |  |
|  | Pictures | n/a |
|  | Worksheets | Section D2.5.1.2 - see maintenance checklist |
|  | Electronic Tool | n/a |
|  | Aerial Photos | n/a |
|  | Maps | n/a |
|  | Other | Section D2.5.1.2 - see maintenance checklist |
|  | Report Generator | Section D2.5.1.2 |
| 5 | How Often Reviewed (Cycle of review) |  |
|  | 1-2 years | Tables D2.5.1.1 Verification Design Protocol |
|  | 5 years | Tables D2.5.1.1 Verification Design Protocol |
|  | 10 years | Tables D2.5.1.1 Verification Design Protocol |
|  | Other | Tables D2.5.1.1 Verification Design Protocol |
| 6 | Independent Verification of Finding |  |
|  | Is this a requirement? | Section D2.5.1.2 |
|  | Internal Independent | Section D2.5.1.2 |
|  | External Independent | Section D2.5.1.2 |
|  | BMP Data Validation |  |
| 7 | Quality Assurance/Spot Checking |  |
|  | Who-qualifications/training/certification | Section D2.5.1.3 |
|  | Method to select BMP for follow-up check | Section D2.5.1.3 |
|  | Method to select the number of BMPs to review | Section D2.5.1.3 |
|  | Other | Section D2.5.1.3 |
| 8 | Data Entry of BMP Implementation |  |
|  | What is the system? | Section D2.5.1.3 |
|  | Who enters data (training/certification)? | Section D2.5.1.3 |
|  | Does the system connect to NEIEN? | Section D2.5.1.3 |
|  | System in place prevent double counting | Section D2.5.1.3 |
| 9 | External Provided Data Validation Meeting CBP Partnership Guidance |  |
|  | Method to validate data | Section D2.5.1.3 |
|  | Who will validate data (training/certification)? | Section D2.5.1.3 |
| 10 | Historic Data Verification |  |
|  | System to re-certify or remove | Section D2.5.1.3 |
|  | Who will verify historic data training/certification)? | Section D2.5.1.3 |
|  | Documentation of action | Section D2.5.1.3 |
|  | BMP Performance |  |
| 11 | Does state collect data to assess BMP Performance? |  |
|  | System used to collect BMP performance data? | Section D2.5.1.3 |
|  | Who collects BMP performance data? | Section D2.5.1.3 |
|  | Who analyzes collected data and report to CBP? | Section D2.5.1.3 |
| Source: Derived from Table 7 in CBP 2014. |  |  |

# Appendix G BMP Targeting and Prioritization (Watermelon Charts)

Relative contribution to WIP-planned Nitrogen load reduction among BMPs

Relative contribution to WIP-planned Phosphorus load reduction among BMPs

Relative contribution to WIP-planned Total Solids load reduction among BMPs

# Appendix H NEIEN Methodology for Historical Data Clean Up

NEIEN Methodology for Historical Data Clean Up

June 2015

Delaware Department of Natural Resources and Environmental Control (DNREC)

Nonpoint Source Program (NPS): Marcia Fox

Tetra Tech: Eugenia Hart

This document provides a summary of the methodology used to clean up the historic NEIEN data. To start, the data submitted from 2010 through 2014 were downloaded from the node. The downloaded data were compared to the input files for year 2010-2014 to confirm that the downloaded data matched the data that were originally submitted. The most recent error report from the 2014 Progress Run (from February 2015) was reviewed to identify any errors. The errors were addressed by updating the BMP name where applicable. Note that ALL previously submitted data were resubmitted (not just data that contained errors or were not previously reported) with new model version of Phase 6 in the header schema.

Credit duration for each BMP was also included as “Lifespan” (in years) for all of the historical data. The lifespan or credit duration of each BMP is based on the values provided in the CreditDurations05222015.xlxs spreadsheet developed by CBP. This spreadsheet includes credit durations for each BMP type approved by the Urban Stormwater Workgroup on March 17, 2015; the Ag Workgroup on May 21, 2015; and the Wastewater Workgroup on June 2, 2015. The code in the DE NPS BMP Database has been modified so that the lifespan/credit duration is added to the implementation date of a particular BMP to calculate the Lifespan End Date. Once the Lifespan End Date has been passed, that BMP will be tagged as “retired”. Credit duration for several practices (mostly NRCS practices) that were not included in the CreditDurations05222015.xlxs spreadsheet were provided by Sally Kepfer at NRCS (Dover, DE).

In addition, any BMP that is known to be implemented in the Chesapeake Bay watershed should have the qualifier code IMNFW so that it is not spread across the county/state. This option is not currently in the data input template, but will be added later. The IMNFW qualifier code was added manually for all historical BMPs identified as “ST” (state) rather than “FED” (federal). The only federal BMPs in Delaware are NRCS and FSA practices. This code was also added to 2007 cover crops from Kent Conservation District provided for the entire county.

Specific methodologies for each of the practices reported are provided for each source sector:

Agricultural BMPs

Animal Waste Management Structures (AWMS)/Waste Storage facilities: provided for the entire county. Submitted as is and indicated N in the in BD watershed column. Note that “roofs and covers” is a type of AWMS. Data provided by DDA for 2014. Previously provided by NRCS.

Conservation Tillage – Percentage provided by county for Conservation Tillage and High Residue Minimum Soil Disturbance. Can be entered as a percentage. Data provided by DNREC WAMS in 2014.

Cover Crops – Where cover crop names were not accepted in NEIEN, the acres were divided evenly between each crop type and submitted individually. If the cover crop names were still not accepted in NEIEN, they were changed to general “cover crop” to receive the minimum credit.

Sussex County – Cover crops for Chesapeake Bay watershed were provided by SCD. Harvested cover crops were identified as “commodity cover crops”. Also, if a specific BMP wasn’t listed (for example: “Cover Crop Early Aerial Wheat”), but “Cover Crop Early Other Wheat” was listed, “Aerial Wheat” was included as “Other Wheat”. Planting dates were provided – used these dates and CBP’s BMP guidelines to determine whether the crops were early/late/standard. These acres were NOT subtracted from the NRCS cover crop acres for Sussex County as the conservation districts submit as CTA and are separate from the NRCS values. Historic Cover Crop data for the Chesapeake Bay watershed portions of Sussex County (2005 through 2010) were also provided by SCD. These cover crops were included as general “cover crops” as no specific planting details were provided.

Kent County –cover crop data were provided by Kent County Conservation District (KCD) for the entire county. Watersheds not in the Chesapeake were removed. Harvested cover crops were identified as “commodity cover crops”. A few records had 2 dates, as though the cover crops were planted over 2 days. For these entries, the later date was used, assuming this was the date the planting was completed. “Late” and “Early” dates were used as indicated by KCD. These acres were NOT subtracted from the NRCS cover crop acres for Kent County as the conservation districts submit as CTA and are separate from the NRCS values.

New Castle County – Cover crop data were provided by the New Castle County Conservation District (NCCD) by HUC 12. Data entry followed same methodology as Sussex and Kent counties. These acres were NOT subtracted from the NRCS cover crop acres for New Castle County as the conservation districts submit as CTA and are separate from the NRCS values.

NRCS and FSA – Note that NRCS and FSA had cover crop data that were included (see NRCS above). These are separate acres and were provided for the entire state/county (not just the CB watershed) so they need to be spread evenly. Any NRCS cover crop acres were subtracted from the FSA cover crop acres and any remaining acres were included as “Commodity Cover Crop Late Other Wheat” for minimum credit.

DDA Manure Relocation – Manure Relocation was provided by DDA as tons of poultry manure. The data included the sending watershed (by name; GIS was used to find the county), receiving watershed (by name; GIS used to identify location), receiving town (by name), receiving state, claim tons, claim date, application #, and whether the relocation was “farm to alternative use” (NMAU). Note that the majority of the Nanticoke watershed is in Sussex County and a small portion is in Kent County. An assumption was made that all manure was coming from Sussex County. Marshyhope watershed is in 2 counties, but it is unknown which county the manure is coming from, so the claim tons were split evenly between the 2 counties. Only manure exported FROM the Chesapeake Bay watershed were included. COUNTY_TO in the Excel sheet was left blank if the manure leaves the Chesapeake Bay watershed or is identified as “farm to alternative use”. The HUCs included were the receiving HUC. Only the most descriptive HUC needs to be included (i.e., include the best level of detail available). HUCs were included where available. Anything outside of Delaware, but inside the Chesapeake watershed doesn’t have HUCs because exact location/watershed is unknown.

- Note that Delaware does not transport any manure besides poultry. The poultry in Delaware are all broilers except for one layer facility, therefore, the Animal Group was labeled as “Poultry”.

- ‘County To’ and ‘County From’ were included for ALL manure transported within the watershed. Even if it went to another state, the FIPS code was identified for that out-of-state location. Unique BMPs IDs for each manure entry (poultry, county to, county from) are the same.

- As of 1/9/2015, DNREC provided all manure transported from Perdue AgriRecylce outside of the watershed. Any transport within a 10-mile radius of the facility isn’t included in the cost-share data so these data had never been included before 2014.

- Additional historical data were provided by DDA for 2002, 2005, 2007, and 2009. The data included County From, County To, Year, and tons.

DDA Nutrient Management Planning – DDA provided total acres in each watershed (by name). Watershed names were matched with the appropriate HUC. All NMPs are done as a 3-year plan (per Bob Coleman at DDA), but those acres are only put in the database for the first year, so the NMP acres for the two previous years are added to the current year (e.g., 2012 and 2013 were added to the 2014 acres) to get the actual acres with NMP for the current year)Nutrient Management acres are provided by DDA’s Nutrient Management Program and are calculated using the total number of acres from the DDA annual reports database with a 5% adjustment.

DNREC Restoration Database (grass buffers, water control structures, land retirement) –  DNREC –DWS-NPS maintains a restoration database that captures restoration practices like grass buffers, tree plantings, stream restoration, wetland restoration and water control structures. These practices are compiled from various projects throughout DNREC. In 2014, DNREC worked closely with scientists, planners and biologists with Fish and Wildlife, Parks, and Watershed to review practices within the database and upload missing practices.  The restoration database links DNREC BMPs to NRCS practice codes. The database is not set to match the BMPs reporting to EPA-CBPO. Therefore, DNREC-DWS-NPS must make judgment calls when assigning acres (or other units) to specific EPA-CBPO BMPs.

Large Animal Disposal – provided by county from the conservation districts, but this practice – Large Animal Mortality – is not accepted in NEIEN. Note that this is only a special program when funding is available.

NRCS/FSA Data -  are provided at the state level and county level to be evenly distributed. These data were entered as-is into the Excel agriculture template. These data came from Olivia Deveraux and included data back to 2007. The Sussex, Kent, and New Castle County Conservation District cover crop acres were NOT subtracted from the NRCS as in past years. The NRCS cover crop acres for years 2010 through 2013 were revisited to include the correct NRCS cover crop acres. The acres are different and not cost-shared. Note that not all FSA and NRCS practices provide a water quality benefit or are accepted by the Chesapeake Bay Program for the Annual Progress Report; however, all are accepted in NEIEN (according to Olivia Devereux). Note that if there is no HUC, FIPS, or lat/long info to identify the location of these BMPs, “DE” was manually added in the xml file as the geographic code until the template is updated (expected late 2015).

Poultry Phytase: Historical Poultry Phytase data were provided by county from DDA for years 1997, 2002, 2005, 2007, and 2005.

Water Control Structures: In 2013, DNREC and SCD updated GIS coverage for water control structures in the Chesapeake Bay watershed (DNREC QAPP 2015, Appendix C).  These structures were implemented and funded by the SCD.  This project focused on data verification for reporting purposes.

Forestry BMPs

DDA Forestry Harvesting: In 2013, DNREC and DFS updated GIS coverage for timber harvest practices in the Chesapeake Bay watershed (DNREC QAPP 2015, Appendix D). The acres were provided in attribute table. The timber harvest coverage was intersected with the USGS HUC12 coverage to determine the HUC12 for each harvested area. If dates were not provided, they were assumed to be 1/1/2014 (or other appropriate year).

DNREC Restoration Database (tree plantings on ag land use) – DNREC –DWS-NPS maintains a restoration database that captures restoration practices like grass buffers, tree plantings, stream restoration, wetland restoration and water control structures. These practices are compiled from various projects throughout DNREC. In 2014, DNREC worked closely with scientists, planners and biologists with Fish and Wildlife, Parks, and Watershed to review practices within the database and upload missing practices.  The restoration database links DNREC BMPs to NRCS practice codes. The database is not set to match the BMPs reporting to EPA-CBPO. Therefore, DNREC-DWS-NPS must make judgment calls when assigning acres (or other units) to specific EPA-CBPO BMPs.

NRCS/FSA Data - are provided at the state level and county level to be evenly distributed. These data were entered as-is into the Excel agriculture template. These data came from Olivia Deveraux and included data back to 2007. Note that not all FSA and NRCS practices provide a water quality benefit or are accepted by the Chesapeake Bay Program for the Annual Progress Report; however, all are accepted in NEIEN (according to Olivia Devereux). Note that if there is no HUC, FIPS, or lat/long info to identify the location of these BMPs, “DE” was manually added in the xml file as the geographic code until the template is updated (expected late 2015). All NRCS “Ag Tree Planting” was also changed to “Riparian forest buffer” – according to FSA all tree plantings through CREP are planted along waterbodies and should be considered riparian.

Riparian Forest Buffer – CREP acres provided by HUC by the DNREC CREP Program Partnership with FSA. The acres were also provided in the FSA data from Olivia, so they were subtracted out of FSA practices CP22, CP4D, CP3A and submitted as “Riparian Forest Buffer”.

Urban Tree Planting (Tree Planting) – These data are provided by DDA and are submitted as number of trees.

Restoration BMPs (Wetland and Stream)

NRCS/FSA Data - are provided at the state level and county level to be evenly distributed. These data were entered as-is into the Excel agriculture template. These data came from Olivia Deveraux and included data back to 2007. The Sussex, Kent, and New Castle County Conservation District cover crop acres were NOT subtracted from the NRCS as in past years. Note that not all FSA and NRCS practices provide a water quality benefit or are accepted by the Chesapeake Bay Program for the Annual Progress Report; however, all are accepted in NEIEN. Note that if there is no HUC, FIPS, or lat/long info to identify the location of these BMPs, “DE” was manually added in the xml file as the geographic code until the template is updated (expected late 2015). The NRCS “shallow wildlife area” practice was changed to “wetland restoration” for all records.

DNREC Restoration Database (wetland and stream) –  DNREC –DWS-NPS maintains a restoration database that captures restoration practices like grass buffers, tree plantings, stream restoration, wetland restoration and water control structures. These practices are compiled from various projects throughout DNREC. In 2014, DNREC worked closely with scientists, planners and biologists with Fish and Wildlife, Parks, and Watershed to review practices within the database and upload missing practices.  The restoration database links DNREC BMPs to NRCS practice codes. The database is not set to match the BMPs reporting to EPA-CBPO. Therefore, DNREC-DWS-NPS must make judgment calls when assigning acres (or other units) to specific EPA-CBPO BMPs.

Stormwater BMPs

Sediment and Stormwater – Received data by lat/long from the DNREC Sediment and Stormwater Program. Data come from MudTracker and the urban NOI database.

Street Sweeping – New Castle, Kent, and Sussex County street sweeping data were provided for calendar by DelDOT. Entered all as 1/1/2014. Converted Total waste (in tons) to lbs for inclusion in the template. Note that 2014 data were provided also, but the year is not complete so those data will be included in 2015 progress.

Wastewater BMPs

Onsite Sewer Connections – In 2013, DNREC and DNREC Groundwater Discharges group updated GIS coverage for onsite sewer connections in the Chesapeake Bay watershed (DNREC QAPP 2015, Appendix E).  This project focused on data verification for reporting purposes.

Wastewater Treatment Plant Data – DNREC DWR(John DeFriece) provided historical clean-up of wastewater treatment plant data.  EPA provided a data dump of all DMR data for all DE. NPDES discharges that go to the Chesapeake Bay, from both ICIS and PCS back to 1989 when DE first started putting data into PCS.  Also, early on had sent permittees their old data, asking them to fill in any species data they have, in addition to the DMR requirements. Combined those into tables of data and missing data for the CBP parameters.

Step 1

- Filled in equations to calculate missing species data (e.g., TKN = TN – NO23), where possible.

- Used Excel to create “Data Tables” that calculate averages from actual data for each site for:

  - Each facility, parameter, and year.

  - Each facility and parameter, averaged over all results over the years (some of the smaller facilities had some data, but not enough to do yearly averages).

- For still missing data, used the Data Tables mentioned above to fill in, prioritized as follows:

  - 1st       Data from same year,

  - 2nd      Data from same facility, and

  - 3rd      Old Ch. Bay Program default values.  Adjusted those defaults for the Non-contact cooling water discharges with water supply from groundwater*

- Graphs were created to verify results.  Values were adjusted based on Best Professional Judgment

- Did not overwrite data before Jan. 1989.

Step 2

- Double-checked the DMR date with the last non-zero flow, replaced any flow data after that with zeroes, and cleared data from the other columns (similar to the way shown results in the PhaseV data).

- Filled in 1984 through 1988 with a 12 month average of the nearest facility results (usually 1989 or 1990). I did not use the PhaseV data for ’84-’88, but did use what I could find of real data for each site.

References

DNREC (Department of Natural Resources and Environmental Control). 2015. State of Delaware, Nonpoint Source Best Management Practice Implementation Data Quality Assurance Project Plan.

# Appendix I Condensed Version of Urban & Agriculture Spreadsheet Used for Upload into NPS BMP Database

# Appendix J Procedures for Using the Revised & Simplified Cropland Roadside Transect Survey for Obtaining Annual Tillage/Crop Residue/Cover Crop Data

Preface and Justification

In talking with the Chesapeake Bay Program following Delaware’s 2013 Progress Run Submissions, it was recommended that the State look into the tracking and reporting of newly approved best management practices that have not been reported and may have been utilizing historic data sets. Conservation tillage practices are one of these practices. Delaware looked deeper into its WIP goals and identified Conservation Tillage practices as accounting for 11.2% of its relative nitrogen load reductions, classifying it as a priority practice. Since the current conservation tillage data utilized in progress runs comes from the Conservation Technology Information Center (CTIC) data set collected in 2004 and is applied to the decreasing cropland land use, the actual implementation of conservation tillage has been decreasing. In talking with the state’s agricultural partners, the general consensus is that conservation tillage practice implementation should be increasing due to greater widespread knowledge of its benefits. Delaware agreed to move forward with the adoption of this statistically valid cropland residue transect survey originally conducted in western states through CTIC, and more recently in the state of Pennsylvania. This survey serves as the first update to the currently utilized conservation tillage data set for the state of Delaware since 2004.

In addition to tillage and crop residue practices, Delaware has decided to incorporate the observation of cover crop practices throughout the survey. The state currently receives cover crop acreages implemented through the County Soil and Water Conservation Districts, but knows actual implementation for this annual practice is greater than the acres received by the Conservation Districts. In order to take full advantage of the opportunity this survey presents, bringing together agricultural experts in a state-wide survey, Delaware has decided to incorporate cover crop observations.  Being a priority practice in Delaware’s WIP, accounting for 19.6% of Delaware’s relative nitrogen load reductions, it is important that the state understands actual on the ground implementation as best as possible.

Throughout the development of the survey, we worked directly with the state’s agricultural partners including our Conservation Districts in each county, Natural Resource Conservation Service, Farm Service Agency, Delaware Department of Agriculture, and University of Delaware’s Cooperative Extension. We utilized the local knowledge from these partners to establish our driving routes through primarily agricultural crop land, as well as in the determining of appropriate timing. Most importantly, our collaborators provided experienced staff to be part in the actual survey teams.  Our lead observer for the initial survey was Ben Coverdale of Delaware’s Department of Agriculture Nutrient Management Program. Ben serves on the Chesapeake Bay Program’s Conservation Tillage Expert Panel, as well as the Poultry Litter Subcommittee. He is also a grain farmer who produces corn, soybeans, wheat and barley in Kent County, Delaware.  Dr. Richard Taylor served as the lead observer for the Quality Assurance and Quality Control (QA/QC) survey. Dr. Taylor is a soil science professor at the University of Delaware for almost 30 years, as well as an Extension Agronomist Specialist for the University’s Extension Program. He has conducted a wide range of agricultural research focusing on cover crops and no-till practices. While in the vehicles, we were also able to utilize the personal relationships and local knowledge of our Conservation District staff who know the actual management techniques implemented on the fields we observed. All of the members in the vehicle also took part in a training held in conjunction with University of Maryland’s Chesapeake Bay Representative Mark Dubin to practice residue estimation techniques and calibrate the observer’s eye for estimation.  Ben Coverdale attended a crop residue transect training on May 15th, 2015 with Mark Dubin in Lancaster, Pennsylvania to consult with members of Pennsylvania’s transect team.  On-farm visual assessment training was conducted during this session and training was provided by Joel Meyers – a member of the Agriculture Workgroup with the Chesapeake Bay Program.

During the actual survey, our team took many precautions and extra steps to ensure accurate observations and record keeping. Since we had numerous trained participants and resources available, we were able to run our QA/QC team almost immediately after the initial observations were made. The team was able to verify a random sample of the initial observations, at most, two days after the initial observations were made. This ensured that the conditions originally observed were as close as possible to what was viewed in the QA/QC runs. In addition to the immediacy of our quality assurance and quality control review, our lead observer Dr. Taylor is also able to ground truth and interview the land manager of several of the fields with their permission. Dr. Taylor utilized the bead-and-line residue estimation method in several cases to verify that correct observations were recorded.

The development of the mobile application utilizing ESRI ArcCollector GIS software also allowed for a much more streamlined data entry process. With the app, the data enterer was able to record the observations more quickly and accurately by selecting from pre-determined drop down fields, as well as the opportunity for free text entry notes. The GPS aspect of the app means that we have created definitive stopping points for our future surveys. Teams can return to the exact same observation location year after year and track how the agricultural landscape has changed. We have proven this by making an additional survey run in July to gather more information than required by the Conservation Technology Information Center (CTIC). This additional run returned to the observation points where small grains or no crops were initially observed, and were indicated within the application. Observers could then record the second crop that had been planted after the small grains had been harvested. A third survey run is also conducted in late November-early December to collect information on fall cover crop plantings. The application also gave us the opportunity to more easily collect additional fields on top of what CTIC originally collected. This information included observations on the presence of cover crops, their planting method, and the type (traditional vs commodity).

Introduction

The cropland roadside transect survey method is designed to gather information on tillage and crop residue management systems, as well as cover crops. In 2014, CTIC welcomes data collected voluntarily by conservation partnerships around the country. Any county in which this survey method is used is encouraged to submit the data using CTIC’s web site, www.crmsurvey.org. Experience has been that counties with a grid road system, those with fields readily visible from the road, where crops are planted in a relatively short period of time, and where conservation tillage is being adopted are the most likely candidates for conducting a transect.

Crops, soils, and climate interaction dictate to some degree the adoption of high residue systems. Adoption of conservation tillage dramatically reduces nonpoint pollution, enhances soil quality, and enhances carbon accumulation in the soil. Some Midwest states have found the data so valuable that a transect survey has been completed on an annual basis by each county for a number of years. These counties can track changes in tillage practices due to changing weather conditions, as well as a means of documenting effective educational programs, equipment rental, and other affiliated activities.

The purpose of the survey is threefold: (1) to provide information that can be used by individual soil and water conservation districts and others in establishing priorities for educational or other programs, (2) to evaluate progress achieved in reaching county, statewide, and watershed wide goals, and (3) to provide accurate data on the adoption of conservation tillage systems by crop for the CTIC National Crop Residue Management Survey, and possibly to the Chesapeake Bay Program for Chesapeake Bay Model calibration. This makes the transect survey an ideal tool for assessment as well as measuring progress for locally led conservation. The transect survey will enable counties to have a higher level of confidence in their data for use in county programs and in the report submitted to CTIC. State and national data will have a correspondingly higher confidence level.

Statistical reliability of the cropland roadside survey method

When conducted properly, this cropland transect survey procedure provides a high degree of confidence in the data summaries. Users can have 90% or more confidence in the accuracy of the results. This level of reliability translates into data summaries that can help guide the local or state decision-making process. Several states have used transect data to allocate cost-share funds, develop new resource management goals, and to provide information to the general public about the positive impact of progress on land use trends. In general, few data sources have such a high level of reliability combined with quick data collection!

Selecting the crops

The crop list for the 2013 CRM survey includes 17 crops. Visit www.crmsurvey.org for more information.

Crops should be selected for each county from the following list:

corn 				edible beans and peas 		sunflowers

soybeans (full season) 		barley				rye

soybeans (double-cropped) 	canola 				potatoes

forage crop (seeding year only) 	permanent pasture	 	oats

vegetables and other crops 	fallow				sorghum

winter wheat 			grain (other)			hay

specialty crops (orchard, sod, etc.)

A worksheet is available from the CTIC Web site www.crmsurvey.org to record transect data.

Procedure:

Step 1 - Establishing and Marking the Route

The first step in conducting the tillage and crop residue management survey was to establish a driving route. For future surveys, counties shall use the same routes as long as no conflicts come about. Utilizing the same route allows for evaluation of cropping system changes over time. A county highway map with cropland was used to draw a route that passes through areas that are heavily used for crop production. Largely urbanized areas, forested land, rangeland, and heavily traveled federal and state highways were avoided when possible. The direction of the route was not important, however, the route was required to be at least 110 miles long in each county. The routes did not double-back along the same road more than once. Prior to the survey, each route received a trial drive-through to ensure the routes would have minimal issues.

Figure 1. Sample county road transect route for Tipton County, Indiana. Note how the route bypasses towns (such as Tipton, Indiana located in the center of the county). This survey is applicable regardless of the layout of the county, i.e. counties need not be square to provide accurate results with this method.

Step 2 - Establishing the Survey Date and Team

Once the route was established and marked, a date was scheduled for conducting the survey. The survey should be conducted after the majority of the main crops have been planted but before the crop canopy closes or the first row cultivation takes place. In order to obtain all of the necessary cropping information to get the most accurate representation of yearly cropping practices, multiple transect runs had to be completed. The survey team makes three survey runs per for a singular cropping season. The first run is conducted in late November into early December to collect cover crop planting data. The second run is in the early spring, depending on weather patterns and the general planting timeline among the agricultural community. This is conducted after a majority of the spring-seeded crops have been planted, but before canopy closure to ensure windshield observations can be made. A third survey run in late summer is made to obtain the second crop in a double-cropping planting method.  Conducting the surveys at these times allows for easy “windshield observations” without stopping at each field. Since the dates for conducting the county survey depend upon local planting progress, flexibility in scheduling is necessary.

Next, a survey team is established of at least 2 individuals. In this case, 4-5 individuals are utilized, each with a specific role; driver, navigator who marks data collection points on the map, and data recorder, and occasionally someone needs to verify field observations (measuring residue, previous crop, etc.). Survey team members involved the following organizations: NRCS district or soil conservationist, county Extension agriculture agent, University of Delaware agriculture extension agent, and DNREC employees. At least one individual on each team is very familiar with tillage systems and estimating residue levels. Ben Coverdale from Delaware Department of Agriculture is the primary observer in the initial survey. Ben served on the Agricultural Workgroup’s Conservation Tillage Panel and has been trained to identify residue cover percentages. These observers remained constant across all 3 counties within the state to ensure consistent data observations. Dr. Richard Taylor is an Extension Specialist for the University of Delaware’s Cooperative Extension as well as a professor for Plant and Soil Sciences and served as our primary observer in the QA/QC runs. By getting a variety of people involved, the ability to assemble a full team for each day of surveying was greatly increased.

In addition to the original survey team, a Quality Assurance and Quality Control team was established to retrace the original routes after the initial survey was conducted to ensure quality data. The QA/QC team consisted of members that did not participate in initial survey, but from the same organizations. Using the same GPS coordinates as marked in the initial survey run-through; the team checked and confirmed or rejected the initial observations on at least 10% of the fields. Members on the QA/QC team had access to the original observations and were able to compare them with their own judgments.

A training was held for all of the members of the observation teams prior to the actual survey. Mark Dubin and other agricultural specialists informed the teams of various measurement techniques used to estimate on-field residue. The attendees then practiced these methods on sample fields at University of Delaware’s Agricultural Research Center in Georgetown DE. This location allowed for observations on fields utilizing various management techniques for different residue levels, crop types, and planting methods.

Step 3 - Collecting the Survey Data

The highway map aids navigation across the county, especially if there are detours or road changes since the last transect.

For counties with 300,000 cropland acres or less, data was collected at one-half mile intervals in Kent and Sussex Counties and 0.2 mile intervals in New Castle County, as indicated by the vehicle odometer. To obtain a statistically reliable data set, approximately 460 cropland sites are to be observed along the route.

For data collection purposes, a mobile application was developed by DTI through use of ESRI’s ArcCollector application. The app allowed for a more streamlined collection process utilizing a tablet device rather than previous methods of utilizing paper data sheets. The driving route is preloaded onto the application for each county. Using GPS location, the team can track their driving progress throughout the day and follow the predetermined path. As the team comes to their interval observations, they are able to drop a point at that location. Once a point is dropped, a list of selectable fields appears for the data recorder to enter exactly what the observer sees for each side of the road. The fields include residue cover, cover crop observations, and landuse categories, as well as a free text entry field for observation notes. This app allows for greater ease of entry, reduces entry errors, and helps with subsequent surveys. Example screen captures from the application can be found below in figures 1-3.

Beginning at the start of the route, the team traveled the predetermined interval distance and stopped. Fields were observed on both sides, and recorded the appropriate information in the mobile application. Since data is being collected from 2 fields, this constitutes as 2 data collection points. The application automatically saves the GPS location of that data observation. This procedure was repeated until the route is completed and the appropriate number of observations had been collected.

Figure 1. Data collection points for the entire state of Delaware during the survey. Each orange dot represents a stop during the survey, in which data was collected.

Figure 2. Participants in the survey followed the blue path in the direction of the arrows, which was the pre-determined route for the survey. Each orange dot represents a data collection point. Other symbology (ex. Larger pink dot) was used later in the survey for identifying QA/QC stops or for follow up observations.

Figure 3. Once a stop was made for data collection, the user was able to enter the observations using drop-down options from predetermined fields. The data point is saved and can be edited if needed.

Important:

(A) If a data point is a cropland field but is not planted to a crop (hayland, CRP, etc) in 2012, then it was noted as unknown for tillage type.

(B) If a cropland field (pasture, farmstead, subdivision, etc.) is not encountered at the stopping point on one side of the road, data was recorded for only the side with cropland. The non-cropland point becomes not applicable (NA).

(C) Only record data for fields where the tillage type/residue level is obvious. For example, if one is conducting a transect in the spring, it is futile to walk into a winter wheat field to try and determine tillage/residue level. The field was simply marked as unknown for tillage/residue level.

(D) If no cropland field is encountered on either side of the road, the team continued driving until cropland was observed on at least one side of the road.

As the transect survey continued, the survey team stopped and checked field conditions on a regular basis to insure correct estimates are being made for different crop, tillage, and residue conditions. Once the team has calibrated their visual estimates to match actual field conditions, were made less frequently. The team re-calibrated their visual estimates when entering a region of the county with different soil surface conditions due to changes in moisture, organic matter levels, stoniness, or crops grown.

Crop residue cover levels will be the most important data category to confirm with field measurements. Therefore, the line-transect method as described in the National Agronomy Manual for confirming percent residue cover was utilized. Visual estimates were confirmed with field measurements in borderline cases. A list of field residue categories can be found in the appendix A and match those described in the latest Conservation Tillage Panel Report.

As the initial observation team completed a county, the Quality Assurance and Quality Control (QA/QC) team followed through along the same route to verify a random selection of initial observations (10% of initial stops). The short turn-over time between the initial and QA/QC observations increased the likelihood of identical conditions and allowed for more accurate confirmations from the QA/QC team. To make these confirmations, the QA/QC team periodically would conduct a line and bead test on the actual field to get an accurate measurement of residue cover.

At the end of the route, the number of cropland sites where data was recorded was counted. Fields were not counted twice if the transect crossed over its previous route. The totals for route mileage, vehicle stops, and actual cropland observations are found in the table below.

|  | Route (miles) | Vehicle Stops | Cropland Observations |
| --- | --- | --- | --- |
| New Castle County | 133 | 315 | 470 |
| Kent County | 206 | 341 | 504 |
| Sussex County | 202 | 331 | 497 |

Step 4 - Crop Acreage and Percentage Calculation

The number of observations were summed for each residue/tillage category and then summed for each crop. Dividing the sum in each category by the total for the crop will provide the percentage for each tillage system. For example, if there were 36 observations for no-till corn, 22 for mulch-till corn, 28 for reduced-till corn, and 14 for conventional corn, the sum would be 100. So this county would have 36% no-till corn, 22% mulch-till corn, 28% reduced-till corn, and 14% conventional-till.

For producing an acreage of CTIC-based survey cover crops, acreages are reported utilizing the methodology that was approved by the Agricultural Workgroup at the September 2015 meeting. The cover crop observation percentages that are made during the survey are categorized by species, planting time, and planting method based on the NEIEN appendix for approved cover crop BMPs. The observation percentages for each of the cover crop categories were then applied the 2012 NASS county-wide harvested cropland acreages, yielding estimated acreages for traditional cover crops based on the survey. The acreages for each cover crop category reported by the county conservation districts are then subtracted from the matching cover crop category calculated from the CTIC-based survey. The acres left are submitted through NEIEN at the county-wide level, where model simulation calculates acreages within the watershed. NRCS acres of cover crops are not reported in order to prevent double counting of cover crops.

The calculations will be submitted to CTIC as part of their national survey. The data collected will be submitted to the Chesapeake Bay Program to receive nutrient reduction credits towards meeting Delaware’s Watershed Implementation Plan Goals. The data will be submitted in the form of implementation percentages under each residue category. In addition, the survey data will also be submitted to CTIC as part of their National Crop Residue Management (CRM) Survey. The latest Crop Residue Management Survey results previously reported for every county in the U.S. are posted on the CTIC Web site http://www.ctic.purdue.edu/CRM/.

APPENDIX A

Tillage Definitions

Tillage Systems Definitions as featured in the National Crop Residue Management Survey:

The following set of definitions was established by CTIC and is recognized as a standard. They are used nationwide by many government agencies and private industry.

Conservation Tillage systems include high residue minimum soil disturbance, no-till, ridge-till and mulch-till.

Any tillage and planting system that covers 30 percent or more of the soil surface with crop residue, after planting, to reduce soil erosion by water. Regional studies have showed that the highest level of soil conservation and water quality benefits are achieved when crop residue cover is greater than 60 percent. This methodology serves as a revision to the current CTIC methodology to specifically include the >60% residue cover category into the field transect survey.

High Residue, Minimum Soil Disturbance – The Continuous High-Residue Minimum Soil-Disturbance (HR) BMP is a crop planting and residue management practice in which soil disturbance by plows and implements intended to invert residue is eliminated. Any disturbance must leave a minimum of 60% crop residue cover on the soil surface as measured after planting. HR involves all crops in a multi-crop, multi-year rotation and the crop residue cover requirement (including living or dead material) is to be met immediately after planting of each crop. The purpose of implementing the HR BMP is to improve soil organic matter content and soil quality, and to reduce runoff and sediment and nutrient losses coupled with a continuous high-residue management system. Multi-crop, multi-year rotations on cropland are eligible. The system must be maintained for a minimum of one full crop rotation.

High Residue, Minimum Soil Disturbance

- Minimum of 60% crop residue cover after planting

- Must be maintained for a minimum of one full crop rotation

No-till/strip-till - The soil is left undisturbed from harvest to planting except for strips up to 1/3 of the row width (strips may involve only residue disturbance or may include soil disturbance). Planting or drilling is accomplished using disc openers, coulter(s), row cleaners, in-row chisels or rototillers. Weed control is accomplished primarily with crop protection products. Cultivation may be used for emergency weed control. Other common terms used to describe No-till include direct seeding, slot planting, zero-till, row-till, and slot-till.

No-till/strip-till

- Less than 1/3 of row disturbed

- Greater than 30% residue after planting

- Crop protection products used for weed control

Ridge-till - The soil is left undisturbed from harvest to planting except for strips up to 1/3 of the row width. Planting is completed on the ridge and usually involves the removal of the top of the ridge. Planting is completed with sweeps, disk openers, coulters, or row cleaners. Residue is left on the surface between ridges. Weed control is accomplished with crop protection products (frequently banded) and/or cultivation. Ridges are rebuilt during row cultivation.

Ridge-till

 Less than 1/3 of row disturbed

 Greater than 30% residue after planting

 Top 1-2” of ridge removed at planting

 Crop protection products are usually banded

 Row cultivation is used for weed control and to rebuild ridges

Mulch-till – Full-width tillage that involves one or more tillage trips, disturbs the entire soil surface and is done prior to and/or during planting. Tillage tools such as chisels, field cultivators, disks, sweeps or blades are used. Weed control is accomplished with crop protection products and/or cultivation.

Mulch-till

 Entire field is tilled

 Greater than 30% residue after planting

 Usually one to 3 tillage trips

 Chisel plow, disk, field cultivator, and combination tools are used

Other Tillage Types:

Reduced-till (15-30% residue)  Full-width tillage that involves one or more tillage trips, disturbs the entire soil surface and is performed prior to and/or during planting. There is 15-30 percent residue cover after planting or 500 to 1,000 pounds per acre of small grain residue equivalent throughout the critical wind erosion period. Weed control is accomplished with crop protection products and/or row cultivation.

Reduced-till

 Entire field is tilled

 15 to 30% residue after planting

 Usually one to 3 tillage trips (maybe more)

 Chisel plow, disk, field cultivator, and combination tools are used

Conventional-till or intensive-till  Full-width tillage that involves one or more tillage trips and disturbs the entire soil surface and is performed prior to and/or during planting. There is less than 15 percent residue cover after planting, or less than 500 pounds per acre of small grain residue equivalent throughout the critical wind erosion period. Generally involves plowing or intensive (numerous) tillage trips. Weed control is accomplished with crop protection products and/or row cultivation.

Conventional-till

 Entire field is tilled

 Less than 15% residue after planting

 Usually two to as many as four or more tillage trips are used involving the moldboard plow, chisel plow, disk, field cultivator, or combination tools.

APPENDIX B

Background on Surveys

Transects have been used by a number of states to quantify the amount of various tillage systems being used by crop. Although the exact method of data collection and procedure varies, all sought to improve the accuracy of the amount of conservation tillage by county.

Cropland surveys designed to estimate the amount of conservation tillage being used on the land are a relatively new concept. The Conservation Technology Information Center (CTIC) initiated the annual National Crop Residue Management Survey in 1982. The data gathered for this national survey usually involved a meeting of minds and data. NRCS field office personnel (usually district conservationists) in each county were annually urged to utilize area agricultural statistical data and meet with others who may have information to arrive at “best estimates” for the national survey. NRCS district conservationists are often assisted by soil and water conservation district personnel, county extension agents, agribusiness, local farm organizations, and other interested parties to complete a survey form that denotes these best estimates, which are generally based on personal knowledge.

Another survey conducted on a national basis is the 5-year NRCS National Resources Inventory (NRI). These data are collected on some 22 parameters, including physical characteristics of the land and the effects of agronomic practices on soil erosion. The NRI is a “point” survey method, where points correspond to random locations within a field. The first NRI in 1977 contained limited data on conservation tillage systems, as did subsequent surveys in 1982, 1987, 1992, and 1997.

Use of the NRI to estimate accurate acreage of conservation tillage or to document annual cropland trends in a state or county is greatly limited. The NRI has proven valuable in development of national resource policies.

References Cited

Chesapeake Bay Program. 2013. High Residue, Minimum Soil Disturbance: definition and recommended

sediment and nutrient reduction effectiveness estimates Report (DRAFT). Chesapeake Bay

Program, 410 Severn Avenue / Suite 112, Annapolis, Maryland 21403.

CTIC. 1994. 1994 National Crop Residue Management Survey. Conservation Technology

Information Center, West Lafayette, IN.

Eck, K.J., P.R. Hill, and J.R. Wilcox. 1994. Estimating corn and soybean residue cover. Purdue

University Cooperative Extension Service. Agronomy Guide AY-269. West Lafayette, IN.

Hess, P.J., D.T. Cooper, and P.R. Hill. 1990. TRANSECT: A microcomputer program for managing

conservation tillage survey data. Agron. Abst. Am. Soc. Agron., Madison, Wisc. p. 71.

Hill, Peter R. 1996. Cropland Roadside Survey Method: Procedures for Cropland Transect Surveys for

Reliable County-and Watershed-Level Tillage, Crop Residue, and Soil Loss Data. Conservation

Technology Information Center, West Lafayette, IN.

Iowa NRCS. 1989. Conservation tillage report. Iowa Natural Resources Conservation Service (formerly

Soil Conservation Service). Iowa Bulletin No. IA190-9-10. Des Moines, IA.

Krieger, K.A. 1986. Simulated surveys of crops and tillage practices in the Honey Creek Watershed using

fixed-interval and random points methods. Report for Division of Soil and Water Conservation,

Ohio Department of Natural Resource, Columbus, OH.

Kush, D.M., and E.B. Crawford. 1987. Tillage and residue cover transect survey in eleven

northwest Ohio counties, 1986. Report for US Environmental Protection Agency, Region V.,

Chicago, IL. Ohio Department of Natural Resources, Columbus, OH.

Steel, R.G.D., and J.H. Torrie. 1980. Principles and procedures of statistics. Second edition.

McGraw-Hill, Inc., New York.

# Appendix K Standard Operating Procedures for Delaware Nutrient Management Plan Verification for Land and/or Animal Operations

Overview

The DE Nutrient Management Program (Program) was designed to protect Delaware waters from agricultural and commercial organic and inorganic pollutants while maintaining farm profitability. Part of the Program’s goal is to verify these practices are being followed by the regulated community thereby measuring compliance and providing technical assistance for farmers out of compliance. Estimates of verified compliance is a tool for measuring Program success, fiscal responsibility and taking credit for pollution reduction activities for water quality restoration efforts around the state and region.

Definitions:

Best Management Practice (BMP): Methods or techniques found to be the most effective and practical means in achieving water quality while making the optimum use of farms’ resources.

Commercial Nutrient Handler: Individuals who apply nutrients to 10 or more acres of land as part of a commercial business.

Compliance: operation is in good standing according to nutrient management law based upon verification evaluation.

Disabled: Certification holder has a lapsed certification due to lack of continuing education credits.

Enabled: Certification holder is in good standing and up to date on continuing education credits.

Program staff scientist: Person performing verification evaluations on behalf of the Program and the Delaware Department of Agriculture.

Farm Gate: the management unit of one farm, linked by operator and consisting of multiple fields managed as one.  Fields may have different soil tests, but agronomic outcomes should be consistent.

Field-Scale: a management unit with the same crop, application rate and representative soil sample under the direction of an operator.

Management Unit: an area of crop production that is continuous in its extent and recognized individually in a nutrient management plan with specific application rates and timing from other farm areas.

Nutrient Generator: Individuals who need to be certified at the Nutrient Generator level have operations that include 8 or more animal units and less than 10 acres of land receiving nutrients.

Nutrient Management Consultant: Individual who is certified to write nutrient management plans.

Non-Compliance: operation has areas of improvement to gain compliance and failure to improve could result in public hearing and/or civil penalty.

Operation: Any agricultural operation that has a need for nutrient management practices per the nutrient management law

Operator: Any person that is responsible for nutrient management activities associated with agricultural production.

Private Nutrient Handler: Individuals who apply nutrients to 10 or more acres that is owned, managed, or rented by the individual.

Records: receipts, logs, journals or electronic notes that indicate the amount, location and temporal specifics of farm activities.

State Technical Standards: The State Technical Standards can be found at https://dda.delaware.gov/nutrients/NM_TechStandards.shtml Specifications for component practices required or advised by Nutrient Management Plan or CAFO regulation can be found here and provide additional metrics by which component practices may be verified for compliance.

Verification Evaluation: Scheduled meeting with operator to evaluate operation nutrient management plans, activities and records to protect water quality.

Criteria

Operator Eligibility

The certified person should have attended the initial certification sessions with the University of Delaware and have a valid certification number. This person should also be “enabled” and current on continuing education credits. Continuing Education Credits can be issued based on time spent for the verification evaluation at discretion of the administrator.

Verification Evaluation Types

Operation verification evaluation type is determined by operation practices as listed below.

- Animal Only (and CAFO GP1): any operation that has 8 animal units or more. An animal unit is equal to 1000 lbs. These operations have no land under production.

- Land & Animal (and CAFO with land GP2 & 3): any operation that has 8 animal units or more AND applies nutrients to 10 acres or more.

- Land Only: any operation that applies nutrients to 10 acres or more. These operations have no animal units.

Eligibility & Selection

Based on recent previous compliance rates between 75 and 85 percent the Program estimates 17.7% of farms require verification evaluations annually (See Appendix A, Table 1).  The Program staff can select an operator and/or operation for audit via any of the following manners:

- Random selection- operators may be picked at random from a database of contacts by Program staff scientist at any given time- not due to non-compliance or complaint- to reach target verification evaluation numbers

  - Selection interval follows the recommendation in Appendix A below:  Sampling Recommendations for Delaware Nutrient Management Verification

- Consultant selection- if willing, consultants can help arrange verification evaluations for their clients to mitigate the travel and schedule burdens of random audits.

- Targeted by Non-Compliance- an operator may be selected for a verification evaluation due to annual report inaccuracies or incompleteness or non-compliance from previous audit.

- Targeted by Program staff- an operator may be selected for a verification evaluation due to complaint investigation initiated by a public citizen or program official. If the Program staff investigates a complaint and has suspicion of non-compliance, officer may ask Program staff scientist to conduct verification evaluation.

- Participation in other state and federal programs – higher inspection rates for select groups of farmers participating in state land leasing, federal cost-share programs or other programs for which nutrient management activities are involved may be preselected by a fellow agency for Nutrient Management Program verification evaluation based on available funding and program verification goals.  These evaluations may be performed with Program partner staff in an advisory role and may result in regulatory enforcement beyond the scope of the Program itself.

Notification

Once the Program staff has selected an operator and/or operation for a verification evaluation they must schedule a time to meet with the certified operator via the following guidelines.

- Initiate contact by phone to the farmer, schedule an appointment date & time.

- Schedule appointment 1-2 weeks in advance of the verification evaluation date and the farmer should feel free to notify their plan writer to prepare materials or generally invite them to join.

- Mail a letter to the farmer providing a list of the documents & records that will be needed to conduct the verification evaluation.

- If the farmer cannot be reached by phone after 3 calls during a 2 week period, a certified Departmental letter will be sent containing a date and time for a verification evaluation. The letter should state that contact has been initiated by the DDA via telephone on the dates provided and that this letter serves as an official notification. The letter will require that a response confirming or rescheduling the evaluation sent back to the department must be received within 14 days of the letter date. Failure to respond in a timely manner may result in a public hearing before the Commission or a fine from the Program.

- The Program strives to exercise flexibility in scheduling evaluations during agricultural operations, so in extenuating circumstances, a verification evaluation may be delayed to a more appropriate time at the discretion of the Program staff.  Considerations such as planting and harvest seasons for grain and vegetable farmers or heat waves during which poultry operations are tenuously managed.

- The farmer will need to have available the most current NMP and implementation records as well as the previous NMP and implementation records from a minimum of previous 3 crops or 24 months of history, whichever is less. Retrieving crop yield history of up to 7 years is advised to verify accuracy of yield goals in accordance with the State Technical Standards.

Areas of Verification Evaluation

- Certification: Each operation is required to have a minimum of one (1) certified operator. The certified operator or a certified designee should be present for the verification evaluation. An hour-long verification evaluation should receive one credit. No more than one credit should be issued per verification evaluation within each renewal cycle.

  - Animal only (and CAFO no-land – GP1): Operator(s) should be certified as Nutrient Generator or higher.

  - Land plans (and CAFO with land – GP2 & 3): Operator(s) should be certified as Private Nutrient Handler or higher.

- Nutrient Management Plan or Animal Waste Management Plan: The Nutrient Management Plan or Animal Waste Management Plan is to be written by a certified nutrient management consultant without conflict of interest in the operation. An Animal Waste Management Plan may be written by the certified (Nutrient Generator or higher) operator as outlined below. During the verification evaluation the Program staff scientist will check for completeness of each plan according to the plan requirements.

- Records of Implementation: Each operator is required to keep certain records of implementation outlining their operation practices for five years. We advise a minimum of previous 3 crops or 24 months of history, whichever is less, be available upon inspection. Plans that are cost-shared or updated annually would need to present two consecutive plans.  Two and three year plans cover the requested time period for routine inspection.  The Program Staff scientist may inspect as little as 12 months of record keeping for the following areas.

  - Actual Yield: Specific field or management unit yield information for the last 7 years or less if the yield goal is based on less historical information.

  - Nutrient Type(s): Type of nutrients applied such as inorganic fertilizer, organic fertilizer (manure), or other (ex. DAF). i.e. fertilizer receipts or manure weigh tickets; note: if all inorganic fertilizer was purchased and applied by a custom application fertilizer company, then the invoice detailing the total blend weight would be sufficient.

  - Analysis/Nutrient Content of fertilizers or manures: N-P-K analysis of fertilizers applied i.e. fertilizer labels, blend work orders, manure analyses.

  - Application Rates & Quantity:  Pounds, gallons, or tons applied per acre and total amount applied per total crop acres per application period. Applications such as pre-plant, side-dress or fertigation should be itemized by quantity and traceable to analysis as indicated above. i.e. application records sent by a co-op, log book of applications kept by operator or bulk order receipts with supplemental records of annualized rates.

It is expected that all records are kept on a field scale. Two to three fields will be evaluated per operation. Program staff, at their discretion, may accept farm gate scale records in the few cases where the operation has homogenous management across multiple fields and records to match.

  - Application Timing & Placement: Date(s) applied and indicated method such as banded or starter.  Date of last calibration of spreading equipment should be verified as appropriate. i.e. log book of maintenance and application records kept by operator or plan notations in the margins

  - Manure Management Information:  Manure type, date of removal from production area or manure shed, receiver information, approximate tonnage removed (if applicable). i.e. manure export records kept by operator, receipts from manure export company, weigh tickets from certified truck scales.

- Inspection of Component BMPs-

  - Phosphorus Site Index (PSI) (if applicable): spot check records of actual nutrient application on fields where a PSI has been calculated to make sure the actual application did not result in over fertilization of phosphorus (P).  If no PSI has been performed on fields with a FIV >150, then the P application can be no more than a 3 year crop removal rate. (Staff would need a printout detailing the University of Delaware's crop removal rates by bushel and crop)  See https://cdn.extension.udel.edu/wp-content/uploads/2016/03/23084930/Part-A-phosphorus-loss-potential-due-to-site-and-transport-characteristics.pdf and https://dda.delaware.gov/nutrients/downloads/PSI_DE_All_Counties_110910.pdf

  - Carry over nitrogen credits: Legumes and cover crops provide plant available nitrogen in the next season with sufficient yield of biomass. Residual Nitrogen values can be estimated using the Mid-Atlantic Nutrient Management Handbook (https://s3.amazonaws.com/udextension/ag/files/2013/06/The-Mid-Atlantic-Nutrient-Management-Handbook-2006.pdf). Additionally, available nitrogen from legume cover crops, non-legume cover crop, or any other green manure source, can be assessed by determining plant available nitrogen (Staff would need to see available nitrogen test results and the provider of the results).

  - Pre-side-dress Nitrate Tests (PSNT): Provide all relevant test results on all fields that received manure applications to check that side-dress N application rates were within the allowable range recommended by the a certified consultant or lab.  (Example: UD calculation table & knowledge of how to perform the calculations to determine the side-dress N recommendation - See Attached) http://extension.udel.edu/factsheets/nitrogen-removal-by-delaware-crops/ http://extension.udel.edu/factsheets/phosphorus-removal-by-delaware-crops/

- Management Changes & Plan Modifications during Implementation - Any revisions to the NMP must be justified, documented, and included in the records. Any significant alterations in operations or upon a 25% or greater increase in operation caused by unforeseen circumstances (ex. weather) that occur prior to a NMP's expiration date will require an addendum to the NMP from the certified nutrient consultant.

- Supplemental record inspection:  In the event that 1-2 years of records does not exhibit compliance with the plan specifications, a Program staff scientist, in their sole discretion can evaluate antecedent conditions and records to better evaluate overall farmer performance.

- Additional BMPs may be inspected for crediting practices that exist as separately tracked items.  Some of these may be required for CAFO compliance and others may be collected in an effort to better capture BMP extent across the state for non-cost shared practices.

  - Animal Operation BMPs:

    - Composter

    - Mortality Freezers

    - Manure Shed

    - Heavy use area pads (HUAPs) or concrete end pads

    - Stockpiling

    - Storm Water Retention Pond

    - Pasture Rotation

    - Pasture Stream Fencing

    - Grassed Waterway

    - Windbreaks

    - Regular Manure Sample

  - Land Operation BMPs:

    - Temporary Field Staging

    - Application Setbacks

      - Can be verified with inquiry and application rate and quantity information

    - Application Rates less than Recommendations

    - Cover Crops

    - Grid Soil Sampling

    - Annual or semi-annual Soil Test

    - PSNT; following a recommendation from a certified consultant or approved lab.

    - CSNT; following a recommendation from a certified consultant or approved lab.

    - Precision Application

    - Yield Mapping

    - Strip Trials

    - Split N or P Applications

    - Variable Rate N or P Applications

    - Manure Incorporation

    - Subsurface Injections

Results

Levels of Compliance

During a verification evaluation two different parties are subject to evaluation- the operator and the certified Nutrient Management Consultant (Consultant). The consultant is held accountable for elements of the plan that are missing or incomplete. The operator is held accountable for the certification, implementation (e.g. records) and farm management aspects of the audit. Compliance and non-compliance will be aggregated on a semi-annual basis for reporting purposes under various agreements, but protecting anonymity where legally required.

- Compliance: an operation is in compliance if all aspects of the verification evaluation pass with no reason to follow up. Certification status is enabled and up to date, the operation plan is complete and valid, records of implementation are kept and complete, and farm management is up to specification.

- Substantive Compliance: an operation is in substantive compliance if the findings of the verification evaluation have a good explanation and are within nutrient management recommendations. (i.e. the operator applied less N or P than the plan recommended and this rate maintained productivity.)

- Procedural Non-Compliance: an operation could be in procedural non-compliance if the findings of the verification evaluation have a good explanation and within nutrient management recommendations. (i.e. According to plan a field should have been planted in corn, instead soybeans were planted due to weather the nutrients however were applied at soybean rate.)

- Non-Compliance: an operation will be in non-compliance if there is an environmental issue that needs to be addressed. (i.e. not following nutrient management plan recommendations, records of implementation not being kept,  waste being handled improperly.)

Follow-Up Guidelines

Follow up practices will be communicated to the operator and/or the consultant at the conclusion of a verification evaluation. If compliant, no verification evaluation should be repeated for up to 5 years, unless otherwise determined by selection process. Follow-ups can be made as soon as two weeks or as long as 1 year determined by the cause of follow up. These guidelines are outlined below.

- Compliant: normal selection criteria employed

- Substantive Compliance: normal selection criteria employed, next random selection will seek to cull further deviations.

- Procedural Non-compliant: discretion of program staff

- Non- Compliant: follow up verification evaluation as soon as 2 weeks and as late as 1 year depending on cause, examples provided:

  - Waste storage facility being improperly used or not tidy- 2 week follow up

  - Insufficient records- 1 year follow-up implementation verification

  - No plan or out of date plan- 3 month follow-up to validate current plan and subsequent 1 year implementation verification

Appendix A: Sampling Recommendations for Delaware Nutrient Management Verification-DRAFT

February 5, 2018

Tetra Tech, Inc., Fairfax, VA

EPA Contract EP-C-12-055, Task Order 003, Task 04, Technical Direction 53

Background

Sampling recommendations were derived using information contained in records for 893 unique reporting IDs found for 2016 in data provided by the Delaware Department of Agriculture (DDA). This data set is described in detail in Delaware Nutrient Management Data (Tetra Tech, January 5, 2018). These 893 records constitute the population from which samples are assumed to be taken. These recommendations are made in accordance with Activity 3 of Technical Direction 53 (Statistical Support to the State of Delaware for Determination of Statistically Significant Sub-sampling of Farms Implementing Nutrient Management Practices), which calls for “recommendations for how to design a statistically valid, sub-sampling-based approach to verification of nutrient management practices and compliance with State regulations.”

Method

The sampling plan is based on assigning each of the 893 records to one of three strata and designing a stratified random sampling plan. The three strata are (see Delaware Nutrient Management Data for details on the development of these categories):

- Priority 1: 	Highest priority, potential CAFOs

- Priority 2: 	Next highest priority (includes non-pastured animals)

- Priority 3: 	Lowest priority (no non-pastured animals)

Of the 893 records in the data set, 423 records were assigned to Priority 1, 286 were assigned to Priority 2, and 184 were assigned to Priority 3.

As used in this analysis, the overall (i.e., statewide) proportion of operations meeting requirements, p, can be estimated with Equation 1.

| p=i=1Lwipi | Eq 1 |
| --- | --- |

where wi=NiN,  pi is proportion of operations meeting requirements in strata i, Ni is the number of operations in strata i, and N is the total number of operations across all strata. The standard deviation of the strata spi and overall proportion sp are given Equations 2 and 3, respectively.

| spi= pi(1-pi)ni1-niNi | Eq 2 |
| --- | --- |
| sp=1N2i=1LNi2 Ni-niNipi(1-pi)ni12 | Eq 3 |

L is the number of strata and ni is the number of samples collected for strata i. The 90th and 95th percent confidence interval (CI) half widths (or error margins) are calculated with Equation 4.

| CI=Z1-α2s | Eq 4 |
| --- | --- |

where s is either spi or sp and Z1-α2 is 1.645 and 1.96 for the 90th and 95th percent CI, respectively.

The above equations were applied to evaluate alternative stratified random sampling plans assuming a binomial distribution. The binomial distribution assumes that each evaluated facility can be uniquely classified as meeting or not meeting requirements.

The following characteristics were considered.

- Restrict sampling in Priority 1 (potential CAFOs) to 20 percent.

- The minimum proportion of facilities in compliance:

  - All strata exceed 0.70 compliance

  - All strata exceed 0.80 compliance

- Error margins (CI half width)

  - Not to exceed 0.05

  - Not to exceed 0.10

- Confidence Level

  - 95 percent

  - 90 percent

The binomial distribution is used in cases where there are two choices: yes or no, pass or fail, present or not present. While an assessment of whether an operation is in compliance with nutrient management requirements may involve a checklist or observation of multiple factors, application of the binomial distribution assumes that the final result of an audit will be expressed simply as either “in compliance” or “not in compliance.”

The 20 percent sampling for Priority 1 is designed for consistency with Chesapeake Bay Program verification requirement for CAFOs (concentrated animal feeding operations) (Strengthening Verification of Best Management Practices Implemented in the Chesapeake Bay Watershed: A Basinwide Framework, Chesapeake Bay Program 2014). These requirements state that random, follow-up inspections are recommended to be conducted on 20% of [permit-issued] BMPs. All records designated as Priority 1 represent “potential” CAFOs.

The compliance assumption of 0.80 (i.e., at least 80 percent of operations are in compliance with nutrient management plan implementation requirements) is based on actual data from Delaware. The value of 0.70 is used to provide a conservative estimate as a reference; this lower compliance rate will result in a greater sampling requirement. When using the binomial distribution in an application such as this, a value of 50 percent (i.e., 0.50) results in the largest sample size. As the value approaches 0 or 100 percent, sample size is reduced.

Error margins, or confidence interval half-widths, determine the upper and lower bounds of the estimate. For example, an error margin of 0.05 (or 5 percent) means that if the measured compliance were 0.82, then the actual compliance would fall between 0.77 and 0.87 at the specified confidence level. In the applications presented here, the compliance rate estimates used for sample size calculations are 0.70 or 0.80.

The confidence level indicates the certainty associated with the calculated error margins for the estimate. For example, a confidence level of 90 percent (0.90) means that the true value of compliance has a 90 percent chance of falling within the range determined by the measured compliance ± the calculated error margin (e.g., 0.05). Continuing with the above example, there would be a 90 percent chance that the compliance rate falls between 77 and 87 percent, with a 10 percent chance that the true compliance rate falls outside of that range. In other words, there is a 90 percent chance that the estimated range is correct.

Results

Multiple scenarios were run using the characteristics described above. Findings from calculations led to a narrowing of scenarios to those represented in Table 1.

Central to the calculations is an assumption regarding compliance rate. This assumption is represented in Column 1 for Priority 1 (Prty. 1 Comp. Est.) and Column 2 for the other two categories (Prty. 2-3 Comp. Est.). For example, the assumption for all categories is 80 percent compliance (0.8) in Row 1. It is assumed for these calculations that the statewide compliance rate is the same as the compliance rate for each category.

The calculated sample sizes are shown in Column 3 for Priority 1 (Prty. 1 Sample Size), Column 4 for Priority 2 (Prty. 2 Sample Size), Column 5 for Priority 3 (Prty. 3 Sample Size), and Column 6 for statewide (Total Samples). For example, the statewide sample size is 158 in Row 1.

Columns 7-10 show the confidence interval half width values (CI Half Width, or error margins) for the 90% confidence level. These CI Half Width values are to be applied to the compliance rate assumption values. For example, the 90% CI Half Width value for statewide compliance (Statewide) in Row 1 is 0.049, so the actual compliance rate based on 158 statewide samples should be expressed as 0.80 ±0.049, or 75 to 85 percent at the 90% confidence level.

Assumptions represented in Table 1 include the following:

- A 90 percent confidence level is assumed for all cases.

- The statewide error margin is fixed at no greater than 0.05 (rounded) in all cases: Column 7.

- The error margin for each priority level is fixed at no greater than 0.10 (rounded) in all cases: Columns 8-10.

- The sampling percentage for Priority 1 is fixed at 20 percent (n=85): Rows 1 and 2

- The compliance rate is assumed to be 0.80 for all priority levels and statewide: Rows 1 and 3.

- The compliance rate is assumed to be 0.70 for all priority levels and statewide: Rows 2 and 4.

- The total sample sizes in Rows 3 and 4 are set to equal those in Rows 1 and 2, respectively, while sampling of Priority 1 records (Column 3) is set well below the 20 percent requirement of 85. This was done to test the impact of the Priority 1 sample size on meeting overall requirements of a statewide error margin of no greater than 0.05 (Column 7) and priority level error margins of no greater than 0.10 (Columns 8-10).

The table shading identifies assumptions, input, and resulting calculations. The yellow shading indicates the two levels of assumed compliance. The grey shaded values in Column 3 represent the sampling assumption for CAFOs (Priority 1). The green and peach shaded cells represent the “not-to-exceed” margins of error of 0.050 (actually 0.049-0.050) for statewide and 0.10 (actually 0.064-0.099) for each priority level, respectively.

Table 3. Comparison of alternative sampling options

|  |  |  |  |  |  |  | 90% CI Half Width (error margins) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Row | Prty. 1 Comp. Est. | Prty. 2-3 Comp. Est. | Prty. 1 Sample Size | Prty. 2 Sample Size | Prty. 3 Sample Size | Total Samples | State-wide | Prty. 1 | Prty. 2 | Prty. 3 |
| 1 | 0.8 | 0.8 | 85 | 38 | 35 | 158 | 0.049 | 0.064 | 0.099 | 0.100 |
| 2 | 0.7 | 0.7 | 85 | 54 | 44 | 183 | 0.050 | 0.073 | 0.092 | 0.099 |
| 3 | 0.8 | 0.8 | 58 | 50 | 50 | 158 | 0.050 | 0.080 | 0.085 | 0.079 |
| 4 | 0.7 | 0.7 | 77 | 62 | 44 | 183 | 0.050 | 0.078 | 0.085 | 0.099 |
| Column | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

Discussion

Sample size requirements generally increase as error margin is reduced, confidence level increases, and compliance rate decreases from 80 to 70 percent.

All error margin constraints (0.05 for statewide and 0.10 for each priority level) were met with statewide sample sizes of 158 and 183 for compliance rates of 80 and 70 percent, respectively, at the 90 percent confidence level under the assumption that 20 percent (n=85) of Priority 1 operations are sampled. Assumptions of lower sampling rates for Priority 1 resulted in greater sampling rates for Priority 2 and Priority 3 to achieve the same statewide sample sizes, but did not alter attainment of error margin goals. See Figure 1 for a graphical summary of data in Table 1.

Recommendation

Based on the findings presented here, it is recommended that DDA sample 20 percent of CAFOs (thereby meeting EPA verification requirements), set the statewide error margin goal to not exceed 0.05, set the error margin goal for each priority level at 0.10, and apply a confidence level of 90% to sample size calculations. An initial assumption of 80 percent compliance is appropriate based on existing DDA compliance data, but the assumption should be reconsidered annually and altered to reflect the most recent compliance rates. To ensure that calculated sample size requirements are met, DDA should develop a randomly-selected list of operations that exceeds by 5 or 10 percent the calculated needs. This will facilitate meeting sampling goals in the event that operations on the list cannot be inspected (e.g., operation is no longer in business).

Figure 1. Summary of error margin calculations by sampling scenario

# Appendix L Delaware Agriculture BMP Guidebook (2022)

| Delaware Agriculture Best Management Practices Guidebook This document contains all eligible best management practices (BMPs) that can be submitted to the Chesapeake Bay Program (CBP) for annual progress and verification. The CBP BMP names are cross-refernced with all accepted and other commonly used BMP names as well as their official CBP definitions and standards and specifications to receive nutrient reduction credit in the Chesapeake Bay model (if applicable). An explanation of the information contained in each of the columns on the Agriculture BMPs worksheet is provided below. |  |
| --- | --- |
| Chesapeake Bay Program BMP Name: | BMP name used by the CBP in its documentation and guidance. |
| NEIEN BMP Name: | Official BMP named required for submission to CBP through NEIEN. |
| Other BMP Names Included (If Applicable): | Other official BMP names that can be submitted to CBP through NEIEN and fall under the umbrella of the NEIEN BMP name. |
| Common Partner BMP Names: | Other common BMP names used by partners that are not accepted for submission to CBP but share the same definitions as the NEIEN BMPs. |
| Type: | Annual (receives credit for one year) or Cumulative (receives credit for multiple years) |
| Unit: | Measurement units to be used when submitting BMPs to the CBP. |
| Credited in CB Model (Y/N): | Indicates whether the BMP receives nutrient reduction credit in the model (Y) or if it is in draft form and does not currently receive credit but may at a later date (N). |
| Chesapeake Bay Program BMP Definition: | Official CBP BMP definition. |
| Chesapeake Bay Standards/Specs for Credit: | Official CBP standards and specifications required to be met to receive nutrient reduction credit. |
| Partners: | List of partners who implement this BMP in Delaware. |
| Soil Conservation and Water Quality Plans | Note that all BMP names highlighted in orange are BMPs that are part of Soil Conservation and Water Quality Plans. All of these highlighted BMPs are in DRAFT form; however, some of these BMPs have been approved to be submitted as Soil Conservation and Water Quality Plans. See column C (Other BMP Names Included) for BMPs that can be submitted as Soil Conservation and Water Quality Plans. The list of BMPs accepted as Soil Conservation and Water Quality Plans is expected to expand in the future. |

| Partner ID | Partner Name |
| --- | --- |
| DDA | Delaware Department of Agriculture |
| DFS | Delaware Forest Service |
| DNREC | Department of Natural Resources and Environmental Control |
| DNREC - Drainage | Department of Natural Resources and Environmental Control – Drainage |
| DNREC - FW | DNREC Division of Fish & Wildlife |
| DNREC - NPS | Department of Natural Resources and Environmental Control - Nonpoint Source |
| FSA | Farm Service Agency |
| KCD | Kent County Conservation District |
| NCCD | New Castle County Conservation District |
| NRCS | Natural Resources Conservation Service |
| SCD | Sussex County Conservation District |
| ShoreRivers | ShoreRivers |
| USFWS | U.S. Fish and Wildlife Service |

| Chesapeake Bay Program BMP Name | NEIEN BMP Name | Other BMP Names Included (If Applicable) | Common Partner BMP Names | Type (C= Cumulative; A = Annual) | Unit (A=Acres, C=Count; DT = Dry Tons; AU – Animal Units; F=Feet; F2 = Square Feet; lbs = Pounds: | Credited in CB Model (Y/N) | Chesapeake Bay Program BMP Definition | Chesapeake Bay Standards/Specs for Credit | Partners |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ag Stormwater Management | Agricultural Stormwater Management | Vegetated Treatment Area | Ag E&S | C | A | Y | Agricultural stormwater associated with confined agricultural livestock production land area through practices that reduce nutrient and sediment pollutant loads through engineered mechanisms such as settling or filtering. | In most cases, Agricultural Stormwater Ponds are designed and constructed according to engineering criteria and specifications outlined in state urban stormwater design manuals, although some states allow employment of standardized plans to address agricultural stormwater for poultry houses and similar facilities. For CBP purposes, agricultural stormwater ponds do not include any practices that fall under existing barnyard BMPs nor any practices applied to cropland or pasture sources. |  |
| Agricultural Drainage Management | Drainage Water Management | Drainage Water Management Structure for Water Control Water Control Structure Water Control Structure RI | Drainage Water Management (NRCS 554) Water Control Structure Structure for Water Control (NRCS 587) | C | A | N | The process of managing water discharges from surface and/or subsurface agricultural drainage systems, to raise and lower the water level within the soil profile throughout the year following an operation and maintenance (O&M) plan. | This practice refers to NRCS practice 554, Drainage Water Management (DWM), and provides BMP credit when DWM is used for Controlled Drainage (CD) of tile drained agricultural fields – i.e., to seasonally alter the water table elevation over the field, raising the water table to retain drainwater during the dormant season, and lowering the water table for trafficability and field operations. This practice does not consider or provide BMP credit for other uses of DWM with Water Control Structures, such as controlling the water table elevation to maintain constructed wetlands or flow regulation for bioreactors. Also see definition and specs for Water Control Structures. | DNREC - FW DNREC - NPS KCD SCD ShoreRivers |
| Agricultural Energy Management Plan | Agricultural Energy Management Plan, Headquarters - Written |  |  | A | C | N | N/A | N/A | NRCS |
| Alternative Crops | Alternative Crops | Alternative Crop/Switchgrass RI Alternative Crops | Carbon sequester alternative crop | C | A | Y | Accounts for those crops that are planted and managed as permanent, such as warm season grasses, to sequester carbon in the soil. Carbon sequestration refers to the conversion of crop to hay land. | N/A | DNREC USFWS |
| Ammonia Emission Reductions (Litter Amendments) | Amendments for the Treatment of Agricultural Waste | Amendments for the Treatment of Agricultural Waste | Alum Treatment of Poultry Litter Amendments for Treatment of Agricultural Waste (NRCS 591) Ammonia Emissions Reduction Poultry Litter Amendments Poultry Litter Treatment | A | AU | Y | Surface application of alum, an acidifier, to poultry litter to acidify poultry litter and maintain ammonia in the non-volatile ionized form (ammonium). | Practice components meet criteria standards for NRCS practice 591 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://agriculture.delaware.gov/wp-content/uploads/sites/108/2017/12/AmendTreatmentAgWaste-591.pdf | FSA NRCS |
| Animal Mortality Facility | Animal Mortality Facility | Animal Compost Structure RI Animal Mortality Facility Composter Facilities Composting Facility Dead Bird Composting Facility | Animal Mortality Facility (NRCS 316) Eco Drum Large Animal Mortality Livestock Mortality Composting Poultry Composter Poultry Mortality Composting | C | AU | Y | A physical structure and process for disposing of any type of dead animals. Composted material is land applied using nutrient management plan recommendations. | Enter units of the percent of dead animals composted, animal count, animal units, or number of systems. Practice components meet criteria standards for NRCS practice 316 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://agriculture.delaware.gov/wp-content/uploads/sites/108/2017/12/AnimalMortalityFacility.pdf | FSA KCD NRCS SCD |
| Animal Waste Management System | Animal Waste Management Systems (All Types) | Dry Waste Storage Structure RI Solid/Liquid Waste Separation Facility Waste Control Facilities Waste Control Facility Waste Storage Facility Waste Storage Pond Waste Storage Structure Waste Treatment - Poultry Waste Treatment Lagoon | Agricultural Waste Control Systems Agriculture Waste Control Systems Equine Manure Storage Structure Livestock Waste Management Systems Livestock Waste Structures Manure Storage Structures Poultry Manure Storage Structure Poultry Waste Management Systems Poultry Waste Structures Roofed Animal Manure Structures Waste Field Storage Area Waste Management System Waste Storage Facility (NRCS 313) Waste Treatment Lagoon (NRCS 359) Waste Storage Structure Waste Storage Pond Waste Treatment - Poultry Dry Waste Storage Structure | C | #Sys | Y | Any structure designed for collection, transfer and storage of manures and associated wastes generated from the confined portion of animal operations and complies with NRCS 313 (Waste Storage Facility) or NRCS 359 (Waste Treatment Lagoon) practice standards. Enter units of percent, number of animals or number of animal units. | There are no additional specifications or qualifying conditions beyond those described in the definition. | DDA FSA KCD NCCD NRCS SCD |
| Barnyard Runoff Control | Barnyard Runoff Controls | Animal Trails and Walkways Barnyard Clean Water Diversion RI Concrete Pads Roof runoff management Roof Runoff Structure Wastewater Treatment Strip | Barnyard Runoff Management Diversion (NRCS 362) Roof Runoff Structure (NRCS 558) Roof Water Management Stormwater Runoff Control (NRCS 570) Trails and Walkways (NRCS 575) Wastewater and Feedlot Runoff Control | C | C | Y | Includes the installation of practices to control runoff from barnyard areas. This includes practices such as roof runoff control, diversion of clean water from entering the barnyard and control of runoff from barnyard areas. | Cost-shared runoff control or stabilization must meet the standards of the federal or state program in which they are enrolled. Non-cost-shared gutters or runoff control structures must be documented and meet the CBP’s criteria as defined for the relevant Resource Improvement (RI) practice (CBP RI-16, barnyard clean water diversion). https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf Roof gutters should have a minimum top width of 5 inches and supports no greater than 24 inch spacing. All downspouts, gutters and outlets should be protected from damage by livestock and equipment. The water from roof runoff structures may empty into surface drains or underground outlets, or onto the ground surface and should be directed away from foundations, structures or contaminated areas. Stone filled trenches with an underground outlet, under the roof drip line, may be used in lieu of roof gutter. Locate the trench so the trench centerline follows the roof drip line. | FSA NRCS SCD |
| Biofilters | Biofilters | Biofilters | Poultry biofilters | C | AU | Y | Ammonia emission reduction includes housing ventilation systems that pass air through a biofilter media with a layer of organic material, typically a mixture of compost and wood chips or shreds, that supports a microbial population. The ammonia emissions are reduced by oxidizing volatile organic compounds into carbon dioxide, water and inorganic salts. | N/A | FSA NRCS |
| Blind inlets | Blind inlets | Subsurface Drain Underground Outlets |  | C | A | N | Drain structure backfilled with pervious materials (gravel or sand) that filter drainage water prior to entering subsurface tile drain. Eligible when installed to replace existing tile riser. | N/A | ShoreRivers |
| Blind inlets with P sorbing materials | Blind inlets with P sorbing materials |  |  | C | A | N | Drain structure backfilled with phosphorus sorption material (PSM) solid media that filter drainage water prior to entering subsurface tile drain. Eligible when installed to replace existing tile riser. | N/A |  |
| Broiler Mortality Freezers | Broiler Mortality Freezers |  |  | A | DT | N | This BMP represents the temporary storage of routine broiler mortalities in large on-farm freezer units for collection by a contractor or service-provider. Enter county where the bird carcass originated, county where the bird carcass was transported and the tons of bird carcasses. | N/A |  |
| Brush Management | Brush Management |  | Brush Management (NRCS 314) | C | A | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 314 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2251/314_DE_CPS_Brush_Management_2018 | NRCS |
| Combustion System Improvement | Combustion System Improvement |  |  | A | C | N | N/A | Practice components meet criteria standards for NRCS practice 327 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2278/372_DE_CPS_Combustion_System_Improvement_2013 | NRCS |
| Commodity Cover Crop | Commodity Cover Crop (multiple variations) | Commodity Cover Crop (multiple variations) | Cover Crop - Harvestable Commodity Cover Crop | A | A | Y | A winter cereal crop planted for harvest in the spring that does not receive any fall nutrient applications. | Commodity cover crops may be harvested, but if it received nutrient applications then it is not eligible as a BMP. The planting date (early, standard, or late) is based on the average frost date for the area. Early means the cover crop is planted more than two weeks before the average frost date. Standard or normal is when the cover crop is planted between the average frost date and two weeks before that date. Late is when the cover crop is planted within three weeks after the average frost date. Cover crop BMPs can also be distinguished by the planting or seeding method (aerial, drilled, other). Aerial includes seeding by airplane and other broadcast methods where the seed is not incorporated into the soil (including broadcast only and broadcast/stalk-chopped). Drilled involves planting with a seed drill, whether no-till or conventional till conditions apply. Other includes any non-drilled seeding method where the seed is incorporated into the soil, e.g., broadcast and disked. | FSA KCD NCCD NRCS SCD |
| Comprehensive Nutrient Management Plan | Comprehensive Nutrient Management Plan |  |  | C | C | N | Part of Soil Conservation and Water Quality Plans | N/A | NRCS |
| Concrete pads | Concrete pads |  |  | C | A | N | N/A | N/A |  |
| Conservation Crop Rotation | Conservation Crop Rotation |  | Conservation Crop Rotation (NRCS 328) | C | A | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 328 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2367/328_DE_CPS_(Con)servation_Crop_Rotation_2015 | NRCS |
| Conservation Tillage | Conservation Tillage | Mulch Tillage No Tillage Ridge Tillage | Ago Tillage Conservation Tillage Residue Management Residue and Tillage Management, NoTill (NRCS 329) Residue and Tillage Management, Reduced Till (NRCS 345) Tillage Management | A | A or Percent Available Land | Y | Conservation tillage requires: (a) a minimum 30% residue coverage at the time of planting, and (b) a non-inversion tillage method. | The tillage routine must maintain 15 percent or greater crop residue coverage immediately after planting to be considered a BMP. There are no additional specifications or qualifying conditions beyond those described in the definitions. | DNREC - NPS |
| Contour Buffer Strips | Contour Buffer Strips |  |  | C | A | N | Part of Soil Conservation and Water Quality Plans | N/A |  |
| Contour Farming | Contour Farming |  |  | C | A | N | Part of Soil Conservation and Water Quality Plans | N/A |  |
| Contour Orchard and Other Fruit Area | Contour Orchard and Other Fruit Area |  |  | C | A | N | Part of Soil Conservation and Water Quality Plans | N/A |  |
| Cover Crop | Cover Crop (multiple variations) | Cover Crop (multiple variations) | Cover Crop (NRCS 340) | A | A | Y | Traditional Cover Crop: A short-term crop grown after the main cropping season to reduce nutrient losses to ground and surface water by sequestering nutrients. This type of cover crop may not receive nutrients in the fall, and may not be harvested in the spring. Traditional Cover Crop with Fall Nutrient Applications: A short-term crop grown after the main cropping season to reduce nutrient losses to ground and surface water by sequestering nutrients. This type of cover crop is planted upon cropland where manure is applied following the harvest of a summer crop and prior to cover crop planting. The crop may not be harvested in the spring. | The application of nutrients in the fall determines which Traditional Cover Crop practice is applicable. Traditional Cover Crops are not harvested in the spring. If a cover crop is harvested (e.g., a winter cereal) then it would count as a Commodity Cover Crop. The planting date (early, standard, or late) is based on the average frost date for the area. Early means the cover crop is planted more than two weeks before the average frost date. Standard or normal is when the cover crops is planted between the average frost date and two weeks before that date. Late is when the cover crop is planted within three weeks after the average frost date. Cover crop BMPs can also be distinguished by the planting or seeding method (aerial, drilled, other). Aerial includes seeding by airplane and other broadcast methods where the seed is not incorporated into the soil (including broadcast only and broadcast/stalk-chopped). Drilled involves planting with a seed drill, whether no-till or conventional till conditions apply. Other includes any non-drilled seeding method where the seed is incorporated into the soil, e.g., broadcast and disked. | FSA KCD NCCD NRCS SCD |
| Crop Irrigation Management | Irrigation System, Microirrigation Irrigation System, Sprinkler Irrigation Water Conveyance, Pipeline, High-Pressure, Underground, Plastic Irrigation Water Management | Cropland Irrigation Management Irrigation System, Microirrigation Irrigation System, Sprinkler Irrigation Water Conveyance, Pipeline, High-Pressure, Underground, Plastic Irrigation Water Management | Irrigation Pipeline | C | A | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 449 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://agriculture.delaware.gov/wp-content/uploads/sites/108/2017/12/Irrigation.pdf | FSA NRCS |
| Dairy Precision Feeding Forage Management | Feed Management | Feed Management | Feed Management (NRCS 592) for dairy | A | AU | Y | Dairy Precision Feeding reduces the quantity of phosphorus and nitrogen fed to livestock by formulating diets within 110% of Nutritional Research Council recommended level in order to minimize the excretion of nutrients without negatively affecting milk production. Enter units of percent, number of animals or number of animal units. | This BMP is only applicable to dairy operations. | NRCS |
| Denitrifying Bioreactor | Denitrifying Bioreactor | Denitrifying Ditch Bioreactors | Denitrifying Bioreactor (NRCS 605) Ditch Controls | C | A | N | Structure that diverts agricultural tile-drainage water to pass through a media chamber filled with a carbon source for denitrification of dissolved nitrate to occur. | Practice components meet criteria standards for NRCS practice 605 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2261/605_DE_CPS_Denitifying_Bioreactor_2013 | ShoreRivers |
| Diversion | Diversion |  | Diversion (NRCS 362) | C | A | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 362 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2258/362_DE_CPS_Diversion_2018 |  |
| Early Successional Habitat Development/Management | Early Successional Habitat Development/Management |  |  | C | A | N | N/A | Practice components meet criteria standards for NRCS practice 647 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/36389/647_DE_CPS_Early_Successional_Habitat_Development-Mgt_2022 |  |
| Farmstead Enery Improvement | FARMSTEAD ENERGY IMPROVEMENT |  |  | A | C | N | N/A | N/A | NRCS |
| Fencing | Fencing |  | Fence (NRCS 382) | A | F | N | See Exclusion Fence BMPs | Practice components meet criteria standards for NRCS practice 382 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://agriculture.delaware.gov/wp-content/uploads/sites/108/2017/12/Fence-382.pdf |  |
| Firebreak | Firebreak |  |  | C | A | N | N/A | Practice components meet criteria standards for NRCS practice 394 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/36390/394_DE_CPS_Firebreak_2022 |  |
| Floodplain Restoration | Floodplain Restoration |  |  | C | A | N | N/A | N/A |  |
| Forage and Biomass Planting | Forage and Biomass Planting |  |  | A | A | N | N/A | N/A | NRCS |
| Forest Buffer - Narrow | Narrow Forest Buffers | Narrow Forest Buffers | 5, 10, or 35-ft Riparian Buffer Setback | C | A | Y | Forest buffers are linear wooded areas that help filter nutrients, sediments and other pollutants from runoff as well as remove nutrients from groundwater. Narrow buffer width is between 10 and 35 feet. | These practices are only applicable on converted cropland. Any buffer less than 35 feet in (average) width is only eligible for the narrow buffer practices. Cost-shared buffers must meet the standards of the federal or state program in which they are enrolled. Non-cost-shared buffers must be documented and meet the CBP’s criteria as defined for the relevant Resource Improvement (RI) practices (CBP RI-9,10 for forest buffers). https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf To create a forested nutrient exclusion area or buffer, reduce excess amounts of sediment, organic material, nutrients, pesticides and other pollutants in surface runoff. The minimum width shall be at least 10 feet measured horizontally on a line perpendicular to the water body, beginning at the top of bank or wetland edge. In order to adequately address water quality, the buffer width may need to be expanded to include important resource features such as wetlands, steep slopes, areas that are occasionally or seasonally flooded, or critical habitats. Dominant vegetation (>50% canopy cover) consists of existing, naturally regenerated, or planted trees and/or shrubs. | DDA DNREC USFWS |
| Forest Buffer with stream exclusion fencing | Exclusion Fence with Forest Buffer | Exclusion Fence with Forest Buffer Exclusion Fence with Forest Buffer RI | Forest Nutrient Exclusion Area on Watercourse Riparian Forest Buffer (NRCS 391) Riparian Buffer (FSA CP22) Watercourse Access Control - Trees aka Exclusion Fence with Forest Buffer RI | C | A | Y | Converts streamside pasture to forest and prevents livestock from entering the stream. The recommended buffer width is 100 feet, with a 35 feet minimum width required. Enter units of feet of fence or acres excluded by the fence. There are 17.6 animal units excluded for every 1,000 feet of fencing. Acres are converted to length if you enter a width. If width is not entered, 35 feet is used. Widths less than 35 should use the Forest Buffer-Narrow with Exclusion Fencing BMP. | These buffer practices with exclusion fencing are only applicable on converted pasture. Any buffer less than 35 feet in (average) width is only eligible for the narrow buffer practices. Cost-shared buffers must meet the standards of the federal or state program in which they are enrolled. Non-cost-shared buffers must be documented and meet the CBP’s criteria as defined for the relevant Resource Improvement (RI) practices (CBP RI-6 for forest buffers). https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf The barrier shall be appropriately installed and maintained sufficient to control or restrict the access of livestock. The minimum buffered width between barrier and surface water and or environmentally sensitive area shall be no less than 10 feet measured horizontally on a line perpendicular to the water body, beginning at the top of bank. In order to adequately address water quality, the buffer width may need to be expanded to include important resource features such as wetlands, steep slopes, areas that are occasionally or seasonally flooded, or critical habitats. Vegetation in the buffer between the barrier and surface water should be of a density to help reduce sediment, organic material, nutrients, pesticides and other pollutants in surface runoff. | DDA DNREC USFWS |
| Forest Buffer-Narrow with stream exclusion fencing | Exclusion Fence with Narrow Forest Buffer | Exclusion Fence with Narrow Forest Buffer Exclusion Fence with Narrow Forest Buffer RI | Watercourse Access Control- Narrow Trees, aka Exclusion Fence with Narrow Forest Buffer RI | C | A | Y | Converts streamside pasture to forest and prevents livestock from entering the stream. Narrow buffer width is between 10 and 35 feet. Enter units of feet of fence or acres excluded by the fence. There are 17.6 animal units excluded for every 1,000 feet of fencing. Acres are converted to length if you enter a width. If width is not entered, 10 feet is used. Widths greater than 34 should use the Forest Buffer-Streamside with Exclusion Fencing BMP. | These buffer practices with exclusion fencing are only applicable on converted pasture. Any buffer less than 35 feet in (average) width is only eligible for the narrow buffer practices. Cost-shared buffers must meet the standards of the federal or state program in which they are enrolled. Non-cost-shared buffers must be documented and meet the CBP’s criteria as defined for the relevant Resource Improvement (RI) practices (CBP RI-4a,4b for narrow forest buffers). https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf The barrier shall be appropriately installed and maintained sufficient to control or restrict the access of livestock. The minimum buffered width between barrier and surface water and or environmentally sensitive area shall be no less than 10 feet measured horizontally on a line perpendicular to the water body, beginning at the top of bank. In order to adequately address water quality, the buffer width may need to be expanded to include important resource features such as wetlands, steep slopes, areas that are occasionally or seasonally flooded, or critical habitats. Vegetation in the buffer between the barrier and surface water should be of a density to help reduce sediment, organic material, nutrients, pesticides and other pollutants in surface runoff. | DDA DNREC USFWS |
| Forest Buffers | Forest Buffers | CREP Riparian Forest Buffer Forest Buffer on Watercourse RI Forest Buffers Riparian Forest Buffer Riparian forest buffer, terrestrial and aquatic wildlife habitat Woodland Buffer Filter Area | CREP Hardwood Tree Planting (CP3A) Riparian Forest Buffer (NRCS 391) Riparian Buffer (FSA CP22) | C | A | Y | Forest buffers are linear wooded areas that help filter nutrients, sediments and other pollutants from runoff as well as remove nutrients from groundwater. The recommended buffer width is 100 feet, with a 35 feet minimum width required. | These practices are only applicable on converted cropland. Any buffer less than 35 feet in (average) width is only eligible for the narrow buffer practices. Cost-shared buffers must meet the standards of the federal or state program in which they are enrolled. Non-cost-shared buffers must be documented and meet the CBP’s criteria as defined for the relevant Resource Improvement (RI) practices (CBP RI-9,10 for forest buffers). https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf To create a forested nutrient exclusion area or buffer, reduce excess amounts of sediment, organic material, nutrients, pesticides and other pollutants in surface runoff. The minimum width shall be at least 10 feet measured horizontally on a line perpendicular to the water body, beginning at the top of bank or wetland edge. In order to adequately address water quality, the buffer width may need to be expanded to include important resource features such as wetlands, steep slopes, areas that are occasionally or seasonally flooded, or critical habitats. Dominant vegetation (>50% canopy cover) consists of existing, naturally regenerated, or planted trees and/or shrubs. | DDA DNREC FSA KCD NRCS SCD USFWS |
| Forest Harvesting Practices | Forest Harvesting Practices | Forest Harvesting Practices Forest Stand Improvement | Forest - Direct Seeding Forest - Erosion Control Forest Site Preparation Forest Slash Treatment Forest Stand Improvement (NRCS 666) Forest Trails and Landings Prescribed Forestry (Forest - Land Management) | A | A | Y | Forest harvesting practices are a suite of BMPs that minimize the environmental impacts of road building, log removal, site preparation and forest management. These practices help reduce suspended sediments and associated nutrients that can result from forest operations. | Practice components meet criteria standards under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/14574/666_DE_CPS_Forest_Stand_Improvement_2018 Components consisting of conservation measures included in the Forest Harvesting Practices definition include, but may not be limited to the following USDA-NRCS conservation practices: • Forest Trails and Landings (655) • Forest Slash Treatment (384) | DFS FSA NRCS |
| Grade Stabilization Structure | Grade Stabilization Structure |  | Grade Stabilization Structure (NRCS 410) | C | A | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 410 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2269/410_DE_CPS_Grade_Stabilization_Structure_2018 | NRCS SCD |
| Grass Buffer | Grass Buffers | CREP Grass Buffer Field Border Filter Strip Grass Buffer on Watercourse RI Grass Buffer Strip Grass Buffers Grass Filter Strips Grassed Waterway Grassed waterways – Non Easement Riparian [Grass] buffers Riparian Herbaceous Cover | Riparian Herbaceous Cover (NRCS 390) Filter Strip (NRCS 393) Filter Strip (FSA CP21) Field Border (NRCS 386) Grass Waterway (NRCS 412) Grass Waterway, Noneasement (FSA CP8A) Vegetated Filter Vegetative Buffer Strip | C | A | Y | Grass buffers are linear strips of grass or other non-woody vegetation maintained to help filter nutrients, sediment and other pollutants from runoff. The recommended buffer width for buffers is 100 feet, with a 35 feet minimum width required. Vegetated open channels are modeled identically to grass buffers. | These practices are only applicable on converted cropland. Any buffer less than 35 feet in (average) width is only eligible for the narrow buffer practices. Cost-shared buffers must meet the standards of the federal or state program in which they are enrolled. Non-cost-shared buffers must be documented and meet the CBP’s criteria as defined for the relevant Resource Improvement (RI) practices (CBP RI-7,8 for grass buffers). https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf To create a grass nutrient exclusion area or buffer, reduce excess amounts of sediment, organic material, nutrients, pesticides and other pollutants in surface runoff and reduce excess nutrients and other chemicals in shallow ground water flow. For areas adjacent to surface water, the minimum width shall be at least 10 feet measured horizontally on a line perpendicular to the water body, beginning at the top of bank or wetland edge. There should be at least 75% perennial grass cover. In order to adequately address water quality, the buffer width may need to be expanded to include important resource features such as wetlands, steep slopes, areas that are occasionally or seasonally flooded, or critical habitats. Plant and animal pest species shall be controlled to the extent feasible to achieve and maintain the intended purpose of the vegetative cover. Noxious weeds shall be controlled as required by state law. | DDA DNREC FSA KCD NRCS SCD USFWS |
| Grass Buffer - Narrow | Narrow Grass Buffers | Narrow Grass Buffers |  | C | A | Y | Grass buffers are linear strips of grass or other non-woody vegetation maintained to help filter nutrients, sediment and other pollutants from runoff. Narrow buffer width is between 10 and 35 feet. | These practices are only applicable on converted cropland. Any buffer less than 35 feet in (average) width is only eligible for the narrow buffer practices. Cost-shared buffers must meet the standards of the federal or state program in which they are enrolled. Non-cost-shared buffers must be documented and meet the CBP’s criteria as defined for the relevant Resource Improvement (RI) practices (CBP RI-7,8 for grass buffers). https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf To create a grass nutrient exclusion area or buffer, reduce excess amounts of sediment, organic material, nutrients, pesticides and other pollutants in surface runoff and reduce excess nutrients and other chemicals in shallow ground water flow. For areas adjacent to surface water, the minimum width shall be at least 10 feet measured horizontally on a line perpendicular to the water body, beginning at the top of bank or wetland edge. There should be at least 75% perennial grass cover. In order to adequately address water quality, the buffer width may need to be expanded to include important resource features such as wetlands, steep slopes, areas that are occasionally or seasonally flooded, or critical habitats. Plant and animal pest species shall be controlled to the extent feasible to achieve and maintain the intended purpose of the vegetative cover. Noxious weeds shall be controlled as required by state law. | DDA DNREC FSA KCD NRCS SCD USFWS |
| Grass Buffer with stream exclusion fencing | Exclusion Fence with Grass Buffer | Exclusion Fence with Grass Buffer Exclusion Fence with Grass Buffer RI |  | C | A | Y | Converts streamside pasture to open space and prevents livestock from entering the stream. The recommended buffer width is 100 feet, with a 35 feet minimum width required. Enter units of feet of fence or acres excluded by the fence. There are 17.6 animal units excluded for every 1,000 feet of fencing. Acres are converted to length if you enter a width. If width is not entered, 35 feet is used. Widths less than 35 should use the Grass Buffer-Narrow with Exclusion Fencing BMP. | These buffer practices with exclusion fencing are only applicable on converted pasture. Any buffer less than 35 feet in (average) width is only eligible for the narrow buffer practices. Cost-shared buffers must meet the standards of the federal or state program in which they are enrolled. Non-cost-shared buffers must be documented and meet the CBP’s criteria as defined for the relevant Resource Improvement (RI) practices (CBP RI-5 for grass buffers). https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf The barrier shall be appropriately installed and maintained sufficient to control or restrict the access of livestock. The minimum buffered width between barrier and surface water and or environmentally sensitive area shall be no less than 10 feet measured horizontally on a line perpendicular to the water body, beginning at the top of bank. In order to adequately address water quality, the buffer width may need to be expanded to include important resource features such as wetlands, steep slopes, areas that are occasionally or seasonally flooded, or critical habitats. Vegetation in the buffer between the barrier and surface water should be of a density to help reduce sediment, organic material, nutrients, pesticides and other pollutants in surface runoff. | DDA DNREC FSA NRCS SCD USFWS |
| Grass Buffer-Narrow with stream exclusion fencing | Exclusion Fence with Narrow Grass Buffer | Exclusion Fence with Narrow Grass Buffer Exclusion Fence with Narrow Grass Buffer RI | Watercourse Access Control Narrow Grass, aka Exclusion Fence with Narrow Grass Buffer RI | C | A | Y | Converts streamside pasture to open space and prevents livestock from entering the stream. Narrow buffer width is between 10 and 35 feet. Enter units of feet of fence or acres excluded by the fence. There are 17.6 animal units excluded for every 1,000 feet of fencing. Acres are converted to length if you enter a width. If width is not entered, 10 feet is used. Widths greater than 34 should use the Grass Buffer-Streamside with Exclusion Fencing BMP. | These buffer practices with exclusion fencing are only applicable on converted pasture. Any buffer less than 35 feet in (average) width is only eligible for the narrow buffer practices. Cost-shared buffers must meet the standards of the federal or state program in which they are enrolled. Non-cost-shared buffers must be documented and meet the CBP’s criteria as defined for the relevant Resource Improvement (RI) practices (CBP RI-4a,4b for narrow grass buffers). https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf The barrier shall be appropriately installed and maintained sufficient to control or restrict the access of livestock. The minimum buffered width between barrier and surface water and or environmentally sensitive area shall be no less than 10 feet measured horizontally on a line perpendicular to the water body, beginning at the top of bank. In order to adequately address water quality, the buffer width may need to be expanded to include important resource features such as wetlands, steep slopes, areas that are occasionally or seasonally flooded, or critical habitats. Vegetation in the buffer between the barrier and surface water should be of a density to help reduce sediment, organic material, nutrients, pesticides and other pollutants in surface runoff. | DDA DNREC FSA NRCS SCD USFWS |
| Heavy Use Area Protection | Heavy Use Area Protection |  | Heavy Use Area Protection (NRCS 561) Heavy Use Poultry Area Pads Livestock Use Area Protection | A | A | N | N/A | Practice components meet criteria standards for NRCS practice 561 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://agriculture.delaware.gov/wp-content/uploads/sites/108/2017/12/Heavy-Use-Area-Protection-561.pdf | NRCS |
| Hedgerow Planting | Hedgerow Planting |  | Hedgerow Planting (NRCS 422) | C | F | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 422 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://agriculture.delaware.gov/wp-content/uploads/sites/108/2017/12/Hedgerow-Planting-422.pdf | NRCS |
| Herbaceous Weed Control | Herbaceous Weed Control |  | Herbaceous Weed Treatment (NRCS 315) Invasive Species/Noxious Weed Control | A | A | N | N/A | Practice components meet criteria standards for NRCS practice 315 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2268/315_DE_CPS_Herbaceous_Weed_Treatment_2018 | NRCS |
| High Residue Tillage | High Residue Tillage Management | High Residue Tillage Management | Conservation Tillage Residue Management Residue and Tillage Management, No-Till (NRCS 329) Tillage Management-Continuous High Residue Tillage Management | A | A Percent | Y | Continuous, High Residue, Minimum Soil Disturbance Tillage (HRTill) Management eliminates soil disturbance by plows and implements intended to invert residue. A minimum of 60% crop residue cover must remain on the soil surface as measured after planting. The practice involves all crops in a multi-crop, multi-year rotation and the crop residue cover requirement (including living and dead material) is to be met immediately after planting of each crop. | The tillage routine must maintain 15 percent or greater crop residue coverage immediately after planting to be considered a BMP. There are no additional specifications or qualifying conditions beyond those described in the definition. | DNREC - NPS |
| Horse Pasture Management | Horse Pasture Management | Horse Pasture Management | Pasture Management Prescribed grazing (NRCS 528 or 528A) | C | A | Y | Horse Pasture Management is defined as maintaining a 50% pasture cover with managed species (desirable, inherent) and managing high traffic areas. | Jurisdictions may have additional requirements for management of grazing and pasture areas, such as stocking rates (animals per acre). For CBP purposes the only requirement is the minimum vegetative cover. These BMPs can be applied with or without related BMPs such as stream exclusion fencing or off-stream watering systems. |  |
| Integrated Pest Management (IPM) | Integrated Pest Management (IPM) |  | Pest management | A | A | N | N/A | N/A | NRCS |
| Lagoon Covers | Lagoon Covers | Lagoon Covers | Waste facility cover | C | C | Y | Permeable and impermeable covers of lagoons to prevent volatilization of ammonia. A cover can be, and is applied, to various species including swine and dairy. | N/A | DDA FSA NRCS |
| Land Retirement to Open Space | Land Retirement | Conservation Cover Conversion to Hayland RI CREP Wildlife Habitat Critical Area Planting Establishment of permanent native grasses Establishment of permanent vegetative cover (contour grass strips) – Non Easement Grass Nutrient Exclusion Area on Watercourse Narrow RI Habitat buffer for upland birds Marginal pastureland wetland buffer Marginal pastureland wildlife habitat buffer Permanent wildlife habitat, corridors – Non Easement Permanent wildlife habitat, non - easement Retirement of Highly Erodible Land | Critical area planting (NRCS 342) Conservation cover (NRCS 327) Conservation Landscaping Permanent Vegetative Cover Permanent Vegetative Cover Establishment Pollinator Habitat | C | A | Y | Converts land area to hay without nutrients. Agricultural land retirement takes marginal and highly erosive cropland out of production by planting permanent vegetative cover such as shrubs, grasses, and/or trees. | There are no specific conditions for CBP purposes beyond the BMP definitions, with the expectation that reported cost-share practices conform to state or federal practice standards, and any non-cost-shared practices conform to the criteria described in the Resource Improvement (RI) Practice Definitions and Verification Visual Indicators Report https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf | DNREC - FW DNREC - NPS FSA NRCS SCD |
| Land Retirement to Pasture | Land Retirement | Conversion of cropped land to grass-based agriculture Conversion to Pasture RI Establishment of permanent introduced grasses and legumes Pasture and Hay Planting |  | C | A | Y | Converts land area to pasture. Agricultural land retirement takes marginal and highly erosive cropland out of production by planting permanent vegetative cover such as shrubs, grasses, and/or trees. Agricultural agencies have a program to assist farmers in land retirement procedures. | There are no specific conditions for CBP purposes beyond the BMP definitions, with the expectation that reported cost-share practices conform to state or federal practice standards, and any non-cost-shared practices conform to the criteria described in the Resource Improvement (RI) Practice Definitions and Verification Visual Indicators Report https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf | DNREC - FW DNREC - NPS FSA NRCS SCD |
| Lined Waterway or Outlet | Lined Waterway or Outlet |  | Lined Waterway or Outlet (NRCS 468) | C | F | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 468 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2431/468_DE_CPS_Lined_Waterway_or_Outlet_2018 |  |
| Livestock Exclusion with Riparian Buffer | Livestock Exclusion with Riparian Buffer |  |  | A | A | N | See Exclusion Fence BMPs | N/A |  |
| Livestock Pipeline | Livestock Pipeline |  | Livestock Pipeline (NRCS 516) | A | F | N | N/A | Practice components meet criteria standards for NRCS practice 516 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/28038/516_DE_CPS_Livestock_Pipeline_2020 | NRCS |
| Loafing Lot Management | Loafing Lot Management System | Loafing Lot Management System |  | C | Area or C | Y | The stabilization of areas frequently and intensively used by people, animals or vehicles by establishing vegetative cover, surfacing with suitable materials, and/or installing needed structures. This does not include poultry pad installation. | Cost-shared runoff control or stabilization must meet the standards of the federal or state program in which they are enrolled. Non-cost-shared gutters or runoff control structures must be documented and meet the CBP’s criteria as defined for the relevant Resource Improvement (RI) practice (CBP RI-16, barnyard clean water diversion; https://www.chesapeakebay.net/documents/Appendix_H--CBP_Resource_Improvement_Practice_definitions_and_visual_indicators_document_8-8-14.pdf: Roof gutters should have a minimum top width of 5 inches and supports no greater than 24 inch spacing. All downspouts, gutters and outlets should be protected from damage by livestock and equipment. The water from roof runoff structures may empty into surface drains or underground outlets, or onto the ground surface and should be directed away from foundations, structures or contaminated areas. Stone filled trenches with an underground outlet, under the roof drip line, may be used in lieu of roof gutter. Locate the trench so the trench centerline follows the roof drip line. | DDA KCD NCCD NRCS SCD |
| Low Residue Tillage | Reduced Tillage | Reduced Tillage | Conservation Tillage Residue Management Tillage Management-Low Residue Residue and Tillage Management, No-Till (NRCS 329) Residue and Tillage Management, Reduced Till (NRCS 345) Tillage Management | A | A | Y | Low residue tillage management requires 15 – 29% cover, strip till or no-till, and less than 40% soil disturbance. | The tillage routine must maintain 15 percent or greater crop residue coverage immediately after planting to be considered a BMP. There are no additional specifications or qualifying conditions beyond those described in the definition. | DNREC - NPS |
| Manure Incorporation | Manure Incorporation High Disturbance Manure Incorporation Low Disturbance | Manure Incorporation High Disturbance Manure Incorporation High Disturbance Immediate Manure Incorporation High Disturbance Late Manure Incorporation Low Disturbance Manure Incorporation Low Disturbance Immediate Manure Incorporation Low Disturbance Late |  | A | A | Y | Manure Incorporation High Early - Manure is incorporated into the soil within 24 hours of application. The level of soil disturbance is assumed to be high. Manure Incorporation High Late - Manure is incorporated into the soil within 1 and 3 days of application. The level of soil disturbance is assumed to be high. Manure Incorpoartion Low Early - Manure is incorporated into the soil within 24 hours of application with less than 40% soil disturbance. Manure Incorporation Low Late - Manure is incorporated into the soil between 1 and 3 days of application with less than 40% soil disturbance. | Manure must be incorporated into the soil within 1-3 days to be eligible for the manure incorporation (late) BMPs and must be incorporated within 24 hours to be eligible for the incorporation (early) BMPs. The expert panel report (https://www.chesapeakebay.net/documents/Phase_6_FINAL_MII_Final_Report_FINAL.pdf) provides other qualifying conditions, such as appropriate application technologies for injection and incorporation (low-disturbance). Any tillage system is appropriate for high-disturbance incorporation, but not all tillage systems may be consistent with disturbance or crop residue requirements for separate BMPs such as conservation tillage. |  |
| Manure Injection | Manure Injection | Manure Injection |  | A | A | Y | Manure is incorporated into the soil immediately. | Manure injection is a specialized category of placement in which organic nutrient sources (including manures, biosolids, and composted materials) are mechanically applied into the root zone with surface soil closure at the time of application. Injection is expected to provide the greatest level of nutrient loss reduction to both atmospheric and surface runoff pathways (including both dissolved and sediment bound nutrients), as well as odor reduction, due to limited quantities of material left on the soil surface, limited soil disruption, and immediate soil closure. Total soil surface disturbance for injection plus planting and any other field operations should be less than 40% so that the practice is compatible with the Low Residue, Strip Till/No-Till (329) NRCS practice. The expert panel report (https://www.chesapeakebay.net/documents/Phase_6_FINAL_MII_Final_Report_FINAL.pdf) provides other qualifying conditions, such as appropriate application technologies for injection. |  |
| Manure Transport (Out of Area) | Manure Transport | Manure Transport | Manure relocation Manure/litter hauling Manure/litter transport | A | DT | Y | Transport of excess manure in or out of a county. Manure may be of any type—poultry, dairy, or any of the animal categories. Transport should only be reported for county to county transport. Movement within the same county should not be included. Enter either the dry or wet tons. Calculations are performed on dry tons, so if you enter wet tons, they are converted for you. | None for CBP purposes beyond what is included in the definition. States may have requirements for haulers or producers that are not summarized here. | DDA |
| Manure Treatment (Composting) | Manure Compost Forced Aeration High CN Manure Compost Forced Aeration Low CN Manure Compost Static Pile Windrow Manure Compost Static Pile Windrow High CN Manure Compost Static Pile Windrow Low CN Manure Compost Turned Pile Windrow Manure Compost Turned Pile Windrow High CN Manure Compost Turned Pile Windrow Low CN Manure Treatment Forced Aeration Manure Treatment Rotating Bin Manure Treatment Rotating Bin High CN Manure Treatment Rotating Bin Low CN | Manure Compost Forced Aeration High CN Manure Compost Forced Aeration Low CN Manure Compost Static Pile Windrow Manure Compost Static Pile Windrow High CN Manure Compost Static Pile Windrow Low CN Manure Compost Turned Pile Windrow Manure Compost Turned Pile Windrow High CN Manure Compost Turned Pile Windrow Low CN Manure Treatment Forced Aeration Manure Treatment Rotating Bin Manure Treatment Rotating Bin High CN Manure Treatment Rotating Bin Low CN | In-vessel Rotating Bin Rotating Drum Aerated Static Pile Turned Pile Turned Windrow Static Pile Static Windrow | A | DT | Y | For all Manure Treatment BMPs, report units of dry tons and location where the manure is generated as "county from" and the location where the MTT product is applied as "county to". Manure Compost Turned Pile Windrow High CN: Manure is composted using frequent turning. The C:N is > 100. Manure Treatment Rotating Bin High CN: Manure composted in an insulated silo, channel, or bin using a controlled aeration system. The C:N is > 100. Manure Compost Static Pile Windrow: Manure is composted using natural aeration. Use this BMP when the C:N is unknown. Manure Treatment Forced Aeration: Manure is composted using mechanical ventilation. Use this BMP where C:N is unknown. Manure Compost Static Pile Windrow Low CN: Manure is composted using natural aeration. The C:N is < 100. Manure Compost Turned Pile Windrow: Manure is composted using frequent turning. Use this BMP when the C:N is unknown. Manure Treatment Rotating Bin Low CN: Manure composted in an insulated silo, channel, or bin using a controlled aeration system. The C:N is < 100. Manure Compost Forced Aeration High CN: Manure is composted using mechanical ventilation. The C:N is > 100. Manure Compost Turned Pile Windrow LowCN: Manure is composted using frequent turning. The C:N is < 100. Manure Compost Static Pile Windrow High CN: Manure is composted using natural aeration. The C:N is > 100. Manure Compost Forced Aeration Low CN: Manure is composted using mechanical ventilation. The C:N is < 100. Manure Treatment Rotating Bin: Manure composted in n insulated silo, channel, or bin using a controlled aeration system. Use this BMP where C:N is unknown. | By definition, finished compost has a C:N at or below 25. Manure composting BMPs are only applicable to agricultural operations and excludes composting systems used for animal mortality management. Inhouse windrowing of poultry litter between flocks is not considered a composting BMP, but is considered a storage process. | FSA NRCS |
| Manure Treatment (Thermochemical) | Manure Treatment Combustion Manure Treatment Direct Monitor Manure Treatment Fast Pyrolysis Manure Treatment High Heat Combustion Manure Treatment High Heat Gasification Manure Treatment Low Heat Gasification Manure Treatment Slow Pyrolysis | Manure Treatment Combustion Manure Treatment Direct Monitor Manure Treatment Fast Pyrolysis Manure Treatment High Heat Combustion Manure Treatment High Heat Gasification Manure Treatment Low Heat Gasification Manure Treatment Slow Pyrolysis |  | A | DT | Y | For all Manure Treatment BMPs, report units of dry tons and location where the manure is generated as "county from" and the location where the MTT product is applied as "county to". Manure Treatment Direct Monitor: Any manure treatment system that has monitoring data to determine the nitrogen load eliminated from the primary manure stream. Report units of both dry tons and pounds of TN. Manure Treatment High Heat Combustion: Manure combusted to produce heat without generating gas or liquid. Temperature between 1,500 and 3,000 degrees Fahrenheit. Manure Treatment Combustion: Manure combusted to produce heat without generating gas or liquid. Use when temperature is unknown. Manure Treatment Fast Pyrolysis: Manure organic matter converted in the absence of oxygen to produce char with a residence time of seconds. Manure Treatment High Heat Gasification: Manure thermochemically reformulated at temperatures between 1,500 and 2,730 degrees Fahrenheit in a low oxygen environment. Manure Treatment Slow Pyrolysis: Manure organic matter converted in the absence of oxygen to produce char with a residence time of hours to days. Manure Treatment Low Heat Gasification: Manure thermochemically reformulated at temperatures less than 1,500 degrees Fahrenheit in a low oxygen environment. | These BMPs are applicable to systems designed for treatment of animal manure and do not apply to systems used for animal mortality management. | FSA NRCS |
| Monitored Non-Tidal Algal Flow-way | Monitored Non-Tidal Algal Flow-way | Monitored Non-Tidal Algal Flow-way |  | A | A lbs. | Y | Algal flow-way technologies are inclined race-ways in non-tidal waters that receive nutrient-laden water so natural algal assemblages can accumulate and then be harvested for an end use. Inflow/outflow biomass monitoring is required. | N/A |  |
| Monitored P removal system | Monitored P removal system |  |  | A | A | N | A landscape-scale filter that traps dissolved phosphorus from animal production areas using phosphorus sorption material (PSM). | N/A |  |
| Monitored Spring Bioreactor | Monitored Spring Bioreactor |  |  | A | A | N | Structure that diverts emerging groundwater to pass through a media chamber filled with a carbon source for denitrification of dissolved nitrate to occur. The treated flow volume and nitrate concentrations are directly measured to calculate the annual removal of N. | N/A |  |
| Mulching for moisture conservation | Mulching for moisture conservation |  | Mulching | A | A | N | N/A | Practice components meet criteria standards for NRCS practice 484 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2440/484_DE_CPS_Mulching_2018 | NRCS |
| Nitrogen Core Nutrient Management | Nutrient Management Core N | Nutrient Management Core N | Nutrient Management (NRCS 590) | A | A | Y | The nutrient management core nitrogen BMP includes 5 elements: 1) application rate modification; 2) manure analysis used in plan; 3) spreader must be calibrated within one year; 4) yield estimates used in plan; 5) legume residual N credits and manure mineralization are credited as part of plan. | See Section 2 of Nutrient Management Practices For Use in Phase 6.0 of the Chesapeake Bay Program Watershed Model: https://www.chesapeakebay.net/documents/Phase_6_NM_Panel_Report_11-28-2016_New_Template_FINAL.pdf | DDA |
| Nitrogen Placement Supplemental NM | Nutrient Management N Placement | Nutrient Management N Placement |  | A | A | Y | Nitrogen rate placement practice requires that the core nitrogen nutrient management BMP be implemented. Includes any of the following: injection of inorganic N, incorporation, or setbacks. | All elements of the Core Nutrient Management BMP must be met to be eligible for one or more of the Supplemental BMPs for nitrogen and/or phosphorus. https://www.chesapeakebay.net/documents/Phase_6_NM_Panel_Report_11-28-2016_New_Template_FINAL.pdf | DDA |
| Nitrogen Rate Supplemental NM | Nutrient Management N Rate | Nutrient Management N Rate |  | A | A | Y | Nitrogen rate adjustment practice requires that the core nitrogen nutrient management BMP be implemented. Includes any of the following: split applications, variable rate N application, or reduced rate from core NM BMP. | All elements of the Core Nutrient Management BMP must be met to be eligible for one or more of the Supplemental BMPs for nitrogen and/or phosphorus. https://www.chesapeakebay.net/documents/Phase_6_NM_Panel_Report_11-28-2016_New_Template_FINAL.pdf | DDA |
| Nitrogen Timing Supplemental NM | Nutrient Management N Timing | Nutrient Management N Timing |  | A | A | Y | Nitrogen rate timing practice requires that the core nitrogen nutrient management BMP be implemented. Includes split application. | All elements of the Core Nutrient Management BMP must be met to be eligible for one or more of the Supplemental BMPs for nitrogen and/or phosphorus. https://www.chesapeakebay.net/documents/Phase_6_NM_Panel_Report_11-28-2016_New_Template_FINAL.pdf | DDA |
| Non Urban Stream Restoration | Stream Restoration Stream Restoration Ag | Channel Bed Stabilization Channel Stabilization Stream Channel Stabilization Stream Improvement for Fish Habitat Streambank and Shoreline Protection Streambank Stabilization Stream Restoration Ag | Channel Bank Vegetation Cut Bank Stabilization Ditch Stabilization Natural channel design Natural channel restoration Legacy sediment removal Regenerative stream channel Regenerative stormwater conveyance (wet channel only) Stream Channel Restoration (Dam Removal) Stream Channel Restoration (Stream Bed/Habitat) Streamside restoration Stream Restoration Protocol Tax ditch restoration | C | A F Lbs | Y | Non Urban Stream Restoration Protocol: Stream restoration is a change to the land stream corridor that improves the stream ecosystem by restoring the natural hydrology and landscape of a stream, and helps improve habitat and water quality conditions in degraded streams. Multiple protocols are defined to characterize different pollutant load reductions associated with individual projects. Feet must be specified. To receive credit for a specific protocol, also specify the pounds reduced for TN, TP, and/or TSS. Non Urban Stream Restoration: Stream restoration is a change to the stream corridor that improves the stream ecosystem by restoring the natural hydrology and landscape of a stream, and helps improve habitat and water quality conditions in degraded streams. Use this BMP if the specific project design is not known. Feet must be specified. | There are further protocol-specific qualifying criteria detailed in other resources (https://www.chesapeakebay.net/documents/Stream_Panel_Report_Final_08282014_Appendices_A_G.pdf). All projects must meet the following criteria to be eligible for credit: • Reach restored must be greater than 100ft in length. • Reach restored must be actively enlarging or degrading. • Reach restored MAY NOT be tidally influenced. • The project MAY NOT be primarily designed to protect public infrastructure. Bank armoring and rip rap are not eligible for stream restoration credit. • Restoration plan must utilize a comprehensive approach to stream restoration design, addressing long-term stability of the channel, banks, and floodplain. • Must comply with all state and federal permitting requirements, including 404 and 401 permits. Stream restoration is a carefully designed intervention to improve the hydrologic, hydraulic, geomorphic, water quality, and biological condition of degraded urban streams, and must not be implemented for the sole purpose of nutrient or sediment reduction. Restoration projects should be developed through a functional assessment process, such as the stream functions pyramid (Harman et al. 2012) or functional equivalent. | DNREC - Drainage SCD |
| Non-Tidal Algal Flow-way | Non-Tidal Algal Flow-way | Non-Tidal Algal Flow-way |  | A | A | Y | Algal flow-way technologies are inclined race-ways in non-tidal waters that receive nutrient-laden water so natural algal assemblages can accumulate and then be harvested for an end use. Where tidal conditions are not known, use this BMP. | N/A |  |
| Nursery Runoff Capture & Reuse | Nursery Runoff Capture & Reuse |  |  | A | A | N | N/A | N/A |  |
| Obstruction Removal | Obstruction Removal |  |  | A | A | N | N/A | Practice components meet criteria standards for NRCS practice 500 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2245/500_DE_CPS_Obstruction_Removal_2003 | NRCS |
| Off stream watering without fencing Alternative water source | Alternative Water System | Alternative Water System Extension of CREP Watering System Watering Facility Watering Trough RI | Alternative Water Sources Pasture Alternative Watering Stream Protection without Fencing Water Well Watering facility (NRCS 614) | C | A C | Y | This BMP requires the use of alternative drinking water sources such as permanent or portable livestock water troughs placed away from the stream corridor. Implementing off-stream shade for livestock is encouraged where applicable. The source of water supplied to the facilities can be from any source including pipelines, spring developments, water wells, and ponds. In-stream watering facilities such as stream crossings or access points are not considered in this definition. The modeled benefits of alternative watering facilities can be applied to pasture acres in association with or without improved pasture management systems such as rotational grazing. | This BMP is only applicable for livestock pastures that do not have stream exclusion practices, as pastures that exclude livestock from streams already provide alternative water sources as part of those practices. See buffers with exclusion fencing as an example. Otherwise, there are no specific conditions for CBP purposes. It is expected that reported cost-share practices conform to state or federal practice standards, and that any non cost-shared practices conform to the criteria described in the Resource Improvement (RI) Practice Definitions and Verification Visual Indicators Report https://www.chesapeakebay.net/documents/RI_Report_5_8-8-14.pdf | FSA NRCS |
| P removal system | P removal system | Sorbing Materials in Ag Ditches | Ditch Controls | C | A | N | A landscape-scale filter that traps dissolved phosphorus from agricultural drainage water using phosphorus sorption material (PSM). | N/A |  |
| Phosphorus Core Nutrient Management | Nutrient Management Core P | Nutrient Management Core P | Nutrient Management (NRCS 590) | A | A | Y | The nutrient management core phosphorus BMP includes 6 elements: 1) application rate modification; 2) P soil test used in plan; 3) manure analysis used in plan; 4) spreader must be calibrated within one year; 5) yield estimates used in plan; 6) legume residual N credits and manure mineralization are credited as part of plan. | See Section 2 of Nutrient Management Practices For Use in Phase 6.0 of the Chesapeake Bay Program Watershed Model: https://www.chesapeakebay.net/documents/Phase_6_NM_Panel_Report_11-28-2016_New_Template_FINAL.pdf | DDA |
| Phosphorus Placement Supplemental NM | Nutrient Management P Placement | Nutrient Management P Placement |  | A | A | Y | Phosphorus rate placement practice requires that the core phosphorus nutrient management BMP be implemented. Includes any of the following: incorporation, setbacks, or use of P Index for application rate. | All elements of the Core Nutrient Management BMP must be met to be eligible for one or more of the Supplemental BMPs for nitrogen and/or phosphorus. https://www.chesapeakebay.net/documents/Phase_6_NM_Panel_Report_11-28-2016_New_Template_FINAL.pdf | DDA |
| Phosphorus Rate Supplemental NM | Nutrient Management P Rate | Nutrient Management P Rate |  | A | A | Y | Phosphorus rate adjustment practice requires that the core phosphorus nutrient management BMP be implemented. Includes any of the following: split applications, variable rate P application, or reduced rate from core NM BMP. P-based manure application must be equivalent to crop P removal. | All elements of the Core Nutrient Management BMP must be met to be eligible for one or more of the Supplemental BMPs for nitrogen and/or phosphorus. https://www.chesapeakebay.net/documents/Phase_6_NM_Panel_Report_11-28-2016_New_Template_FINAL.pdf | DDA |
| Phosphorus Timing Supplemental NM | Nutrient Management P Timing | Nutrient Management P Timing |  | A | A | Y | Phosphorus rate timing practice requires that the core phosphorus nutrient management BMP be implemented. Includes either split application or application in lower P-loss risk season. | All elements of the Core Nutrient Management BMP must be met to be eligible for one or more of the Supplemental BMPs for nitrogen and/or phosphorus. https://www.chesapeakebay.net/documents/Phase_6_NM_Panel_Report_11-28-2016_New_Template_FINAL.pdf | DDA |
| Pipeline | Pipeline |  |  | C | F | N | N/A | N/A |  |
| Poultry Manure Incorporation | Poultry Manure Incorporation |  |  | A | A | N | N/A | N/A |  |
| Prescribed Grazing | Prescribed Grazing | Grazing Land Protection Intensive Management of Rotational Grazing Planned Grazing System Prescribed Grazing Rotational Grazing RI | Managed intensive grazing Pasture Management Prescribed grazing (NRCS 528 or 528A) Precision Intensive Rotational/Prescribed Grazing | C | A | Y | This practice utilizes a range of pasture management and grazing techniques to improve the quality and quantity of the forages grown on pastures and reduce the impact of animal travel lanes, animal concentration areas or other degraded areas. PG can be applied to pastures intersected by streams or upland pastures outside of the degraded stream corridor (35 feet width from top of bank). The modeled benefits of prescribed grazing practices can be applied to pasture acres in association with or without alternative watering facilities. They can also be applied in conjunction with or without stream access control. Pastures under the PG systems are defined as having a vegetative cover of 60% or greater. | Jurisdictions may have additional requirements for management of grazing and pasture areas, such as stocking rates (animals per acre). For CBP purposes the only requirement is the minimum vegetative cover. These BMPs can be applied with or without related BMPs such as stream exclusion fencing or off-stream watering systems. | FSA NRCS |
| P-Sorbing Materials | P-Sorbing Materials |  |  | C | A | N | N/A | N/A |  |
| Pumping Plant | Pumping Plant |  |  | A | C | N | N/A | N/A | NRCS |
| Restoration and Management of Rare and Declining Habitats | Restoration and Management of Rare and Declining Habitats |  | Restoration of Rare or Declining Natural Communities (NRCS 643) | C | A | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 643 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/30346/643_DE_CPS_Restoration_of_Rare_or_Declining_Natural_Communities_2021 |  |
| Saturated buffer | Saturated buffer |  | Saturated buffer (NRCS 604) | C | A | N | Diversion of tile-line flow to a subsurface, perforated distribution pipe used to divert and spread drainage system discharge to a vegetated area to increase soil saturation. | N/A |  |
| Seasonal High Tunnel System for Crops | Seasonal High Tunnel System for Crops |  |  | A | F2 | N | N/A | N/A | NRCS |
| Shallow Water Development and Management | Shallow Water Development and Management |  | Shallow Water Development and Management (NRCS 646) | C | A | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 646 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2454/646_DE_CPS_Shallow_Water_Development_and_Management_2015 | NRCS |
| Soil Conservation and Water Quality Plans | Conservation Plans Conservation Plans/SCWQP | Conservation Plans Conservation Plans/SCWQP Contour Orchard and Other Perennial Crops Cross Slope Farming Herbaceous Wind Barriers Strip-Cropping, Field Strip-Cropping, Wind ***More BMPs coming soon*** | Conservation Crop Rotation Wind Barrier - Herbaceous | C | A | Y | Farm conservation plans are a combination of agronomic, management and engineered practices that protect and improve soil productivity and water quality, and to prevent deterioration of natural resources on all or part of a farm. Plans must meet technical standards. | As noted above, for CBP purposes plans and any associated conservation practices implemented must meet applicable NRCS technical standards. Plans are subject to other state-specific programmatic requirements, where they exist; the term used for a “Soil Conservation and Water Quality Plan” may vary based on state programs but the purpose of the qualifying conservation plans remains the same. | FSA NRCS |
| Spring Development | Spring Development | Water Well | Spring Development (NRCS 574) | C | C | N | N/A | N/A |  |
| Stream Habitat Improvement and Management | Stream Habitat Improvement and Management |  | Stream Habitat Improvement and Management (NRCS 395) | C | A | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 395 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2353/395_DE_CPS_Stream_Habitat_Improvement_and_Management_2019 |  |
| Stripcropping | Stripcropping |  |  | C | A | N | Part of Soil Conservation and Water Quality Plans | N/A |  |
| Subsurface Drain | Subsurface Drain |  | Subsurface Drain (NRCS 606) | C | F | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 606 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2297/606_DE_CPS_Subsurface_Drain_2004 |  |
| Surface Drainage, Main or Lateral | Surface Drainage, Main or Lateral |  | Surface Drainage, Field Ditch Surface Drain, Main or Lateral (NRCS 608) | C | F | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 608 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2336/608_DE_CPS_Surface_Drain_Main_or_Lateral_2003 |  |
| Terrace | Terrace |  | Terrace (NRCS 600) | C | A | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 600 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2248/600_DE_CPS_Terrace_2003 |  |
| Tree Planting | Tree Planting | Field wind break establishment - Non Easement Field windbreak Forest Nutrient Exclusion Area on Watercourse Narrow RI Hardwood Tree Planting Reforestation of Erodible Crop and Pastureland Shelterbelt establishment – Non Easement Tree Planting Tree/Shrub Establishment Windbreak/Shelterbelt Establishment | Reforestation Forest planting Windbreak/shelter establishment (NRCS 380) Field Windbreak Afforestation Tree/Shrub Establishment (NRCS 612) Tree Planting (FSA CP3) Hardwood Tree Planting (FSA CP3A) Windbreaks | C | A | Y | Tree planting includes any tree planting, except those used to establish riparian forest buffers, targeting lands that are highly erodible or identified as critical resource areas. | This BMP does not apply to trees planted as riparian buffers or for trees planted in developed settings, which are separate BMPs. | DDA DFS DNREC KCD NRCS SCD USFWS |
| Tree/Shrub Pruning | Tree/Shrub Pruning |  |  | C | A | N | Part of Soil Conservation and Water Quality Plans | N/A |  |
| Underground Outlet | Underground Outlet |  | Underground Outlet (NRCS 620) | C | F | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 620 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2377/620_DE_CPS_Underground_Outlet_2018 | NRCS |
| Upland Wildlife Habitat Management | Upland Wildlife Habitat Management |  | Upland Wildlife Habitat Management (NRCS 645) | C | A | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 645 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2447/645_DE_CPS_Upland_Wildlife_Habitat_Management_2015 |  |
| Vegetative Environmental Buffer Grass RI | Vegetative Environmental Buffer Grass RI |  |  | A | F | N | N/A | N/A |  |
| Vegetative Environmental Buffer Trees RI | Vegetative Environmental Buffer Trees RI |  |  | C | F | N | N/A | N/A |  |
| Waste Facility Closure | Waste Facility Closure |  |  | A | C | N | N/A | N/A | SCD |
| Waste Transfer | Waste Transfer |  |  | A | C | N | N/A | N/A | NRCS |
| Waste Treatment | Waste Treatment |  |  | A | C | N | N/A | N/A | NRCS |
| Water and Sediment Control Basin | Water and Sediment Control Basin |  | Water and Sediment Control Basin (NRCS 638) | C | C | N | Part of Soil Conservation and Water Quality Plans | Practice components meet criteria standards for NRCS practice 638 under the USDA-NRCS National Handbook of Conservation Practices (NHCP) and associated Field Office Technical Guides for each state. https://efotg.sc.egov.usda.gov/api/CPSFile/2461/638_DE_CPS_Water_and_Sediment_(Con)trol_Basin_2003 | NRCS |
| Water Control Structure | Water Control Structure | Water Control Structure | Structure for Water Control | C | A | Y | Installing and managing boarded gate systems in agricultural land that contains surface drainage ditches. | Water control structures are a component in a water management system that conveys water, controls the direction or rate of flow, maintains a desired water surface elevation, or measures water. These structures may be installed for a wide variety of conservation purposes. The structure may be part of a wildlife project that requires modification of the water flow with chutes or cold-water releases. Examples of other uses of this practice include: sluices to provide silt management, screens to keep trash, debris, or weed seeds out of pipelines, tide gates to prevent backflow into a channel, and flow regulation components of bioreactors (flashboard risers). Not all of these uses result in nutrient load reductions. To receive credit for nutrient reduction, WCSs must be a component of a Drainage Water Management system designed and operated for the primary purpose of reducing nutrient loading from drainage systems into downstream receiving waters by restricting subsurface drainage from leaving the field. Water control structures that are components of other CBP BMPs, such as wetland restoration or denitrifying bioreactors, are not eligible for standalone credit. The operation and management of the water control structure must meet the criteria and follow the operation and maintenance guidelines described in NRCS Conservation Practice Standard (Code 554), Drainage Water Management. Drainage water management is defined here as the process of managing water discharges from surface and/or subsurface agricultural drainage systems and does not apply to the management of irrigation water supplied through a subsurface drainage system. | DNREC DNREC - FW KCD SCD |
| Wetland Creation | Wetland Creation | Headwater Wetland Creation Headwater Wetland Gains - Established Wetland Creation Wetland Gains - Established Wetland Restoration |  | C | A | Y | Wetland Creation - Floodplain: Establish or create wetlands in a floodplain by manipulation of the physical, chemical, or biological characteristics to develop a wetland where one did not previously exist. Changes acres from existing land use to the wetland land use. Wetland Creation - Headwater: Establish or create wetlands in a headwater area by manipulation of the physical, chemical, or biological characteristics to develop a wetland where one did not previously exist. Changes acres from existing land use to the wetland land use. | Wetland restoration practices are critical to meeting the Chesapeake Bay’s water quality 2025 goals under both the Chesapeake Bay TMDL and the 2014 Watershed Agreement. However, the conversion or alteration of high quality wetlands strictly for the purposes of nitrogen, phosphorus or sediment load reductions should be avoided. Changing the functions and/or values of existing high quality wetland systems and high quality nonwetland ecosystems that already provide enitrification and phosphorous or sediment trapping should not be pursued. Also, important ecosystems such as rare and endangered species habitat, older growth forests, unique ecotones (i.e. Delmarva Bays, Magnolia bogs, critical fish spawning areas, among others) should not be priorities for wetland practices solely for the nutrient and sediment reductions under the Bay TMDL. Each project should be assessed based on federal, state, and local regulatory requirements, according to best professional judgments in the field, and supported by benchmarks presented in state and federal guidance documents. | DNREC - Drainage DNREC - FW DNREC - NPS FSA KCD NCCD NRCS SCD |
| Wetland Rehabilitation | Wetland Rehabilitation | Wetland Functional Gains - Enhanced Wetland Rehabilitation | Wetland Enhancement | C | A | Y | Rehabilitate wetlands by manipulation of the physical, chemical, or biological characteristics of a site with the goal of returning natural/historic functions to a degraded wetland. Provides a load reduction to the acres draining to the wetland. | Wetland restoration practices are critical to meeting the Chesapeake Bay’s water quality 2025 goals under both the Chesapeake Bay TMDL and the 2014 Watershed Agreement. However, the conversion or alteration of high quality wetlands strictly for the purposes of nitrogen, phosphorus or sediment load reductions should be avoided. Changing the functions and/or values of existing high quality wetland systems and high quality nonwetland ecosystems that already provide enitrification and phosphorous or sediment trapping should not be pursued. Also, important ecosystems such as rare and endangered species habitat, older growth forests, unique ecotones (i.e. Delmarva Bays, Magnolia bogs, critical fish spawning areas, among others) should not be priorities for wetland practices solely for the nutrient and sediment reductions under the Bay TMDL. Each project should be assessed based on federal, state, and local regulatory requirements, according to best professional judgments in the field, and supported by benchmarks presented in state and federal guidance documents. | DNREC - Drainage DNREC - FW DNREC - NPS FSA KCD NCCD NRCS SCD |
| Wetland Restoration | Wetland Restoration Wetland restoration, non-flood plain | CREP Wetland Restoration Headwater CREP Wetland Restoration Headwater Wetland Gains - Reestablished Headwater Wetland Restoration Wetland Gains - Reestablished Wetland Restoration Wetland restoration, non-flood plain | Wetland restoration (NRCS 657) CRP or CREP wetland restoration (CP23) Wetland restoration, nonfloodplain (CP23A) Restore hydrology to prior-converted agricultural land (cropland or pasture) Elevate subsided marsh and re-vegetate Ditch plugging on cropland Legacy sediment removal | C | A | Y | Wetland Restoration - Headwater: Re-establish wetlands in a headwater area by manipulation of the physical, chemical, or biological characteristics of a site with the goal of returning natural/historic functions to a former wetland. Changes acres from existing land use to the wetland land use. Wetland Restoration - Floodplain: Re-establish wetlands in a floodplain by manipulation of the physical, chemical, or biological characteristics of a site with the goal of returning natural/historic functions to a former wetland. Changes acres from existing land use to the wetland land use. | Wetland restoration practices are critical to meeting the Chesapeake Bay’s water quality 2025 goals under both the Chesapeake Bay TMDL and the 2014 Watershed Agreement. However, the conversion or alteration of high quality wetlands strictly for the purposes of nitrogen, phosphorus or sediment load reductions should be avoided. Changing the functions and/or values of existing high quality wetland systems and high quality nonwetland ecosystems that already provide enitrification and phosphorous or sediment trapping should not be pursued. Also, important ecosystems such as rare and endangered species habitat, older growth forests, unique ecotones (i.e. Delmarva Bays, Magnolia bogs, critical fish spawning areas, among others) should not be priorities for wetland practices solely for the nutrient and sediment reductions under the Bay TMDL. Each project should be assessed based on federal, state, and local regulatory requirements, according to best professional judgments in the field, and supported by benchmarks presented in state and federal guidance documents. | DNREC - Drainage DNREC - FW DNREC - NPS FSA KCD NCCD NRCS SCD |
| Wetland Wildlife Habitat Management | Wetland Wildlife Habitat Management |  | Wetland Wildlife Habitat Management (NRCS 644) | A | A | N | N/A | N/A |  |